3GPP TSG-RAN WG4 Meeting # 105		R4-2220553

Toulouse, France, 14th –18th November, 2022

Title:	LS on PSFCH configured power with multiple resource pools

Response to:

Release:	Rel-17

Work Item:	5G_V2X_NRSL-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name: 	Sang-Wook Lee

E-mail Address: 	sangwook1.lee@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1	Overall description

In RAN4#104-e meeting, an LS [1] was sent to RAN1 and RAN2, relating to the parameter of PEMAX,c in the configured transmitted power for S-SSB. RAN4 would like to thank RAN1 for the reply LS [2] on the PEMAX,c issue for S-SSB.

In RAN4#104-e and RAN4#105, RAN4 also discussed the PEMAX,c parameter for PSFCH transmission. The agreement is captured highlighted yellow as below:

6.2E.4 Configured transmitted power for V2X 6.2E.4.1 General The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f, c = MIN {PEMAX,c, PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + TIB,c , P-MPRc), PRegulatory,c } PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X, PRegulatory,c } where <Unnecessary parts omitted> - PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively; - - For the total transmitted power PCMAX,PSFCH, PEMAX,c is the value given by IE sl-maxTransPower when single resource pool configured is transmitted at a given time and sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted at a given time, defined by TS 38.331.

Question 1: RAN4 agreed to define PEMAX,c for PSFCH configured Tx Power in TS38.101-1 as sum of IEs sl-maxTransPower when multiple resource pools configured are transmitted simultaneously and would like to know if this can have any impact to RAN1.

Question 2: RAN4’s understanding is that the total transmit power is shared equally between simultaneously transmitted PSFCHs regardless of whether they are from a single or multiple resource pools. RAN4 would like to confirm with RAN1 that this understanding is correct.

2	Actions

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to provide feedback for the above questions.

3	Dates of next TSG-RAN WG4 meetings

TSG RAN WG4 Meeting #106		February 27th – March 03rd, 2023		Athens, GR

TSG RAN WG4 Meeting #106BIS-e		April 17th – April 26th, 2023		Electronic meeting

References

[1] R4-2214421 (R1-2208348), LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier, VIVO, RAN4#104-e.

[2] R4-2218017 (R1-2210549), Reply LS on Pemax,c of S-SSB transmission, VIVO, RAN4#105-e.