3GPP TSG-RAN WG4 Meeting # 106                                                                       R4-2300139

Electronic Meeting, 27 February - 3 March, 2023

Title: 	Proposals for isotropy and CBW test procedures in RC

Source: 	Huawei, HiSilicon

Agenda item:	9.15.2.2

Document for:	Agreement

# Introduction

The following agreements are captured in RAN4-105 [1].

Issue 2-1-1: Isotropy definition for RC system

Agreements:

  - Use isotropy definition (d) given in equation B.1 of [IEC 61000-4-21: EMC, Part 4; Section 21] with a pass/fail limit of 3dB for frequencies between 0.4 GHz and 6 GHz.

  - Test zone definition must maintain ½ wavelength from conducted surfaces of the chamber.

  - RAN4 shall specify test procedure for isotropy characterization over the defined test zone.

  - RAN4 to study the measurement uncertainty related to the isotropy verification procedure

Issue 2-1-3: Verification procedure for Coherence bandwidth of RC

Agreements:

  - RAN4 should define detailed procedure for measurement and calculation of chamber Coherence Bandwidth.

  - Encourage interested companies to provide Text Proposals for the definition and verification of coherence bandwidth.

This contribution further discusses test issues associated with RC.

# Discussion

Test procedure for isotropy in appendix 1 is based on [2] and is for discussion and/or correction before agreeing to formal text proposal

Proposal 1: review and correct the isotropy test procedure proposal in appendix 1.

Test procedure for CBW (Coherence Bandwidth) in appendix 2 is based on [3] and is for discussion and/or correction before agreeing to formal text proposal. Values for discussion are in square brackets.

Proposal 2: review and correct the CBW test procedure proposal in appendix 2.

# Conclusions

This contribution makes the following proposals.

Proposal 1: review and correct the isotropy test procedure proposal in appendix 1.

Proposal 2: review and correct the CBW test procedure proposal in appendix 2.

# References

- R4-2220610 WF for Rel-18 TRP TRS WI

- “Reverberation Chambers through the Magnifying Glass: An Overview and Classification of Performance Indicators”, Ramiro Serra, IEEE Electromagnetic Compatibility Magazine, Volume 6, Quarter 2, 2017, pp.76-88

- “Reverberation Chamber Metrology for Wireless Internet of Things Devices”, Robert D. Horansky, Kate A. Remley and Sara Catteau, IEEE Microwave Magazine, February 2022, pp.75-85

# Appendix 1 Test procedure for isotropy

The test steps are as follows.

- Select a cubic working volume. The 8 corner points of the cubic working volume will be used as locations for measurement.

- Measure the electrical fields polarized in x, y and z direction at all 8 corner points for a full rotation of the stirrer, Eα, where α = x, y and z, is the electrical field values after averaging over a full stirrer sequence.

- Normalize the measurement in step (b) by the square root of input power.

- Calculate the average of Eα, where α = x, y and z using equation (1).

Eα, avg = 18i=18Eα, i                                               (1)

- Calculate the average of Etotal using the equation (2)

Etotal, avg = 124α={x,y,z} i=18Eα, i                        (2)

- Calculate the per-axis standard deviation using equation (3)

σα=i=18(Eα,i - Eα,avg)8-1                                          (3)

- Calculate the combined standard deviation using equation (4)

σtotal=α={x,y,z} i=18(Eα,i - Etotal, avg)24-1                   (4)

- Express the standard deviation relative to the average. σ and E in equation (5) can be for either α={x,y,z} or total.

σdB=20log10(σ+EavgEavg)                                          (5)

- Repeat steps (a) to (g) for [20] frequency points evenly across the NR bands. (discussion point: [2] recommends 20 points per decade. Currently defined NR frequency range for FR1 is 700 MHz to 7000 MHz. Hence the recommendation of 20 frequency points.)

- Note: step (i) can be swapped with step (a) in order to save the effort on positioning probes.

# Appendix 2 Test procedure for coherence bandwidth

CBW or Coherence Bandwidth is a metric to determine the correlation in frequency within a given working volume of a reverberation chamber (RC) and defined in [3] as

R(i,n)=j=1P-iS21(fj,n)S21*(fj+i,n)j=1P-iS21(fj,n)S21*(fj,n)         (1)

Where

S21 (fj, n) corresponds to the measured complex S21 at the frequency step fj with [P] frequency points measured within a given bandwidth (BW) such that f1 = fc – (BW/2) and fp = fc + (BW/2).

n is the mode stirring sample index with a total equal to [N].

i is the frequency offset index with a frequency resolution of (f1 - fp)/(P - 1). (discussion point: because typical CBW is a few MHz, frequency resolution can be selected as [200 kHz])

Test steps are as follows.

- Select the center frequency fc and BW. A typical measurement frequency range of [100MHz] would be used as a default.

- Determine the number of measurement points P.

- Determine the total number of stirring sequence N.

- Perform measurement and process the data using equation (1)

- Average the auto-correlation R(i, n) over a complete stirring sequence (i.e. index n) for each frequency point index i.

- Identify the coherence bandwidth corresponding to a value of [0.5] in auto-correlation R(i, n)