3GPP TSG-RAN WG4 Meeting # 106												R4-2300143

Athens, Greece, 27 February –03 March, 2023

Agenda item:			9.19.2.1

Source:	CableLabs, Charter Communications

Title:	Simulation assumptions for SBFD adjacent channel coexistence study on the UMa-to-UMi scenario

Document for:	Approval

# Introduction

The sub-band full duplex (SBFD) is introduced in Rel-18. The adjacent channel coexistence study is underway in RAN4 to evaluate the uncoordinated SBFD and legacy TDD networks [1]. However, the urban macro-to-micro (UMa-to-UMi) scenario is not currently considered in [1].

Operators are deploying and considering deploying a dual-layer network topology: macrocells provide coverage and mobility, and microcells (pole-mount or strand-mount) offload the capacity. Band n48 has a tight EIRP limit of 47 dBm/10MHz, which will mainly be used as microcells or indoors. SAS coordinates TDD configurations among operators in the shared band n48, but SBFD will break the TDD configuration coordination. The coexistence between uncoordinated SBFD and TDD networks in bands n77 and n48 is probably even worse with the super high EIRP of 75 dBm/10MHz in n77. We would ask RAN4 to conduct such a macro-to-micro coexistence study between SBFD and legacy TDD networks [2].

The preliminary urban macro-to-macro (UMa-to-UMa) results showed the SBFD adjacent channel interference (ACI) significantly degrades victim legacy TDD network performance [3].

# Proposal simulation assumptions for the macro-to-micro scenario

Proposal 1: The simulation assumptions for the UMa-to-UMi scenario are not included in the RAN4 SBFD adjacent channel coexistence study. We would ask RAN4 to define the corresponding assumptions and conduct the macro-to-micro coexistence study.

A set of simulation assumptions are proposed following the [1] template. All new assumptions for urban microcells are highlighted in the yellow background color. The parameters in bracket [] are open to discussion. The detailed proposed simulation assumptions on urban macro and urban micro network layout models are listed in Table 1.

Table 1: Network layout for urban macro to urban micro in FR1 (4GHz)

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | Macro: 500 m, Micro: [289 m] |
| Grid offset | [10m, 50m] |
| Carrier frequency | 4 GHz [or 3.7 GHz] |
| Path-loss model | - Macro(Aggressor) → Micro(Victim): - MacroBS-to-UE: UMa see TR 38.803 - MicroBS-to-UE: UMi see TR 38.803 - Macro-to-Micro: UMa (h_UE = [10 m]) see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: Reuse the same model as in TR 38.828 with h_UT equals to [10 m]; - Option 2: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.828. - X = [0.75] - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.828. - Use Option 2 for initial calibration purpose. - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 - UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. |
| BS Tx power | Macro: - For legacy TDD conducted power: 49 dBm - For SBFD antenna configuration 1: 46 dBm - For SBFD antenna configuration 2: 49 dBm - Note 1,2,5 Micro: - Max EIRP density: 47 dBm/10MHz (conducted power depends on the max antenna array gain) |
| UE Tx power | 23 dBm (30 dBm for macrocell UEs is not precluded) |
| BS antenna configurations | Macro BS - Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Note 1,2,3,4,5 - Optional: Extended AAS model Section 5.2.3.2.4 of TR 38.803 For legacy TDD: TBA For SBFD: TBA Micro BS - Option 1: omnidirectional coverage One antenna with 5 dBi gain - Option 2: 3-sector For legacy TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna: (Mg,Ng,M,N,P)= (1,1,2,2,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m for macro BSs, [10 m] for micro BSs |
| BS antenna element gain + connector loss | Macro: 5 dBi (assuming antenna 1.8dB loss) Micro: does not apply for option 1, 5 dBi for option 2 |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | - Baseline: Reuse TR 38.828 UE dropping assumption hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 - The cluster based parameters are in 2.2.3.9.1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Note 1: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 2: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. Note 3: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 4: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 5: Using SBFD antenna configuration 1 for calibration purpose; Both two configurations are recommended for simulation. |  |

RAN4 needs to discuss the network layout of both macrocell and microcell networks. We would suggest using a macro-to-micro BS ratio of 3, which means to cover the same area the number of needed microcell BSs is three times of the number of needed macrocell BS. The ISD of UMa is 500 m as agreed by RAN4 [1]. The corresponding ISD of UMi is 50023≈289 m. The network layout of both UMa and UMi networks is illustrated in Figure 1. Because macrocell and microcell networks have different inter-site distances (ISDs), the traditional grid offset defined as 100% does not apply. We suggest using absolute numbers of 10 and 50 m between the center BSs in both networks, and we are open to discuss on other methods. The 19 BSs layout with wrap-around will still work for the UMa-to-UMi scenario.

(a)														(b)

Figure 1. UMa and UMi network layout. Black triangles are 19 victim network BS locations and red triangles are 19 aggressor BS locations. (a) grid offset of 10 m; (b) grid offset of 50 m.

Proposal 2: RAN4 needs to discuss the network layout of the UMi network. We suggest using 289 m as the ISD of the UMi network, which is derived from a macro-to-micro BS ratio of 3. The grid offset is defined as absolute numbers of 10 and 50 m. The 19 BSs layout with wrap-around method will still work for both UMa and UMi.

Microcell BS antenna array will need to be defined. Microcell BSs typically employ smaller antenna array sizes than macro cells. Microcells may provide omnidirectional, 2-sector, or 3-sector coverage. We would propose two options for microcell BS antenna: a) one omnidirectional antenna (without active antenna system) which is the most cost-effective deployment; b) a 3-sector case that aligns with the UMa network layout.

Proposal 3a: An omnidirectional microcell BS antenna with 5 dBi max gain.

Proposal 3b: 3-sector antennas with (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ for both legacy TDD and SBFD networks. SBFD will need two 2×2 antenna arrays to split in uplink and downlink.

Proposal 4: We propose to adopt the FCC BS EIRP limit of 47 dBm/10MHz in band 48/n48 for microcell BSs, and 72 dBm/10MHz in band n77 for urban macrocell BSs. Other BS power limits are not precluded.

Proposal 5: The FCC UE EIRP limit is 23 dBm in band 48/n48 and 30 dBm in band n77. We propose to adopt 23 dBm as the max UE power for microcells and study both 23 and 30 dBm EIRP for macrocell UEs.

The adjacent channel leakage ratio and adjacent channel sensitivity are listed in Table 2.

Table 2: ACLR and ACS for FR1

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 45 dBc |
| BS ACS | 46 dBc |
| UE ACLR | [15] dBc for BW ≤ 50MHz 30 dBc for BW > 50MHz (TS 38.101-1 Table 6.5.2.4.1-1) |
| UE ACS | 33 dBc (TS 38.101-1 Table 7.5-2) |

The BS ACLR in FR1 is assumed as frequency flat with some detailed explanation below:

- when aggressor BW is narrower than victim, e.g., SBFD gNB -> legacy TDD gNB

    - equivalent ACLR is equal to normal ACLR

- when aggressor BW is wider than victim, e.g., legacy gNB -> SBFD gNB

    - total received interference = Ptx – (ACLR + the ratio of aggressor BW to victim BW)

    - for example, when aggressor is 100MHz and victim is 20MHz, the equivalent ACLR is 45+10*log10(100/20)=51.9dB

Proposal 6: RAN4 SBFD coexistence study does not consider the out-of-band emission (OOBE). We propose to combine OOBE and ACLR requirements.

UE distribution assumptions for both microcell and microcell networks are listed in Table 3.

Table 3: UE distribution for FR1

| Scenarios | UE distribution |
| --- | --- |
| Urban Micro (Micro-to-Micro) | Uniformly distributed in the cell. 20% indoor and 80% outdoor |
| Urban Macro (Macro-to-Macro) | - Baseline: 20% indoor and 80% outdoor - Optional: 80% indoor and 20% outdoor |

Other simulation assumptions are listed in Table 4.

Proposal 7: We propose to adopt a 10 MHz channel bandwidth for microcells. The channel allocation in band n48 is coordinated by SAS per 10 MHz chunks. It is unrealistic to assign a 100MHz BW for a band n48 BS.

Proposal 8: If the coexistence study concluded the macro SBFD network causes significant BS-to-BS interference to the legacy TDD network, 3GPP should avoid applying SBFD in the uplink TDD timeslots.

Table 4: Other simulation parameters for FR1

| Parameters | Urban micro | Urban macro |
| --- | --- | --- |
| Channel bandwidth | 10 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 10 MHz For SBFD {DUD}: 4MHz + 4MHz For SBFD {DU}: 8MHz Note 1, 2, 3 | For legacy TDD: 100 MHz For SBFD {DUD}: 40MHz + 40MHz For SBFD {DU}: 80MHz Note 1, 2, 3 |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 10 MHz For SBFD {DUD} and {DU}: 2MHz Note 1, 2, 3 | For legacy TDD: 100 MHz For SBFD {DUD} and {DU}: 20MHz Note 1, 2, 3 |
| SBFD BS PSD | Option 1: the PSD of SBFD is the same as legacy TDD at gNB side Option 2: total Tx power per SBFD DL sub-band is the same as legacy TDD total power. i.e. the PSD of SBFD is higher than legacy TDD PSD Note: Option 1 for calibration purpose. companies are encouraged to report SBFD PSD when submitting simulation results. | Option 1: the PSD of SBFD is the same as legacy TDD at gNB side i.e. for SBFD antenna configuration 1, 26dBm/MHz PSD. for SBFD antenna configuration 2, 29dBm/MHz PSD. Option 2: total Tx power per SBFD DL sub-band is the same as legacy TDD total power. i.e. the PSD of SBFD is higher than legacy TDD PSD i.e. for SBFD antenna configuration 1, 46dBm total output power. for SBFD antenna configuration 2, 49dBm total output power Note: Option 1 for calibration purpose. companies are encouraged to report SBFD PSD when submitting simulation results. |
| Traffic model | Full buffer, Note 4, 5 | Full buffer, Note 4, 5 |
| Inter-BS distance | [289m] | 500m |
| Minimum BS-UE (2D) distance | [10m] | 35m |
| Minimum UE-UE (2D) distance | [3m] | 3m for UMa 1m when UEs are in cluster as in Urban Hotspot scenario |
| DL power control | NO | NO |
| UL power control | YES | YES |
| UE max TX power in dBm | 23 dBm | 23 dBm (30 dBm not precluded) |
| UE min TX power in dBm | -40 dBm (10 MHz CBW) see TS 38.101-1 Table 6.3.1-1 | -33 dBm (100 MHz CBW) see TS 38.101-1 |
| BS Noise figure in dB | 5 dB | 5 dB |
| UE Noise figure in dB | 9 dB | 9 dB |
| Handover margin in dB | 3 dB (Same as FR2) | 3 dB (Same as FR2) |
| BS mechanical downtilt angle in degrees | [0] degrees | 6 degrees |
| Note 1: Above sub-band BW assumption used for simulation not aligned existing RAN4 agreed CHBW sets. Note 2: Above parameters used for simulation purpose only. Note 3: Companies are encouraged to provide the assumption they used for simulation (whether guard-band assumed and the values of guard-band if any) Note 4: Start with full buffer while other RU is not precluded. Companies are encouraged to provide simulation results while indicating their RU assumption used. If the lower RU other than full buffer is suggested or implemented, the explanation of how this RU or traffic model is implemented in simulation should be provided. Note 5: Using Full Buffer case for calibration. Further study whether to and how to simulate low RU case. |  |  |

# Conclusion

Proposal 1: The simulation assumptions for the UMa-to-UMi scenario are not included in the RAN4 SBFD adjacent channel coexistence study. We would ask RAN4 to define the corresponding assumptions and conduct the UMa-to-UMi coexistence study.

Proposal 2: RAN4 needs to discuss the network layout of the UMi network. We suggest using 289 m as the ISD of the UMi network, which is derived from a macro-to-micro BS ratio of 3. The grid offset is defined as absolute numbers of 10 and 50 m. The 19 BSs layout with wrap-around method will still work for both UMa and UMi.

Proposal 3a: An omnidirectional microcell BS antenna with 5 dBi max gain.

Proposal 3b: 3-sector antennas with (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ for both legacy TDD and SBFD networks. SBFD will need two 2×2 antenna arrays to split in uplink and downlink.

Proposal 4: We propose to adopt the FCC BS EIRP limit of 47 dBm/10MHz in band 48/n48 for microcell BSs, and 72 dBm/10MHz in band n77 for urban macrocell BSs. Other BS power limits are not precluded.

Proposal 5: The FCC UE EIRP limit is 23 dBm in band 48/n48 and 30 dBm in band n77. We propose to adopt 23 dBm as the max UE power for microcells and study both 23 and 30 dBm EIRP for macrocell UEs.

Proposal 6: RAN4 SBFD coexistence study does not consider the out-of-band emission (OOBE). We propose to combine OOBE and ACLR requirements.

Proposal 7: We propose to adopt a 10 MHz channel bandwidth for microcells. The channel allocation in band n48 is coordinated by SAS per 10 MHz chunks. It is unrealistic to assign a 100MHz BW for a band n48 BS.

Proposal 8: If the coexistence study concluded that the macro SBFD network causes significant BS-to-BS interference to the legacy TDD network, 3GPP should avoid applying SBFD in the uplink TDD timeslots.

# Reference

[1] R4-2220247, “Simulation assumptions for SBFD adjacent channel co-existence study,” Samsung

[2] R4-2300145, “Adjacent channel co-existence use-case of non-overlapping sub band full duplex (SBFD) between n48 and n77,” Charter Communications, CableLabs

[3] R4-2300145, “ Preliminary simulation results for SBFD adjacent channel coexistence study on the UMa-to-UMa scenario in FR1: a comparative analysis on grid offset that impacts the BS-to-BS ACI,” CableLabs, Charter Communications