3GPP TSG-RAN WG4 Meeting # 106	R4-2300253

# Athens, Greece, February 27 – March 3, 2022

Agenda item:	6.2.6.1.1

Source:	Apple

Title:	On PDSCH demod requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#105 demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our views on some of the open issues and simulation results for PDSCH demodulation requirements.

# 2. Discussion

## PDSCH Demod Requirements for Single Carrier

For PDSCH requirements, several agreements were reached in RAN4#105 as captured in [1]. For the agreed simulation parameters, we provide simulation results for PDSCH demodulation requirements definition. The alignment and ideal results are based on the agreed simulation assumptions:

Table 1: Simulation results for PDSCH demod in FR2-2

| Duplex CBW/SCS | Antenna configuration | RB allocation | MCS and rank | Channel model | % of TP | Alignment Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD 100MHz/120kHz | 2x2 Low | 66 | MCS4, Rank 1 | TDLA30-650 | 70 | -1.8 | 0.6 |
|  |  | 66 | MCS13, Rank 1 | TDLA30-200 | 70 | 6.1 | 8.7 |
|  |  | 66 | MCS13, Rank 1 | TDLA30-650 | 30 | 0.1 | 2.7 |
|  |  | 66 | MCS17, Rank 1 | TDLD30-200 | 70 | 8.3 | 11.4 |
|  |  | 32 | MCS17, Rank 1 | TDLD30-200 | 70 | 8.7 | 11.8 |
| TDD 400MHz/480kHz | 2x2 Low | 66 | MCS4, Rank 1 | TDLA10-200 | 70 | -1.5 | 0.8 |
|  |  | 20 | MCS13, Rank 1 | TDLD10-200 | 70 | 4.9 | 7.8 |

In RAN4#105 there was a discussion on span of results > 2.5 for cases with TDL-D channel model, where some companies’ results showed much lower SNR in TDL-D channels.

We evaluated the performance in TDL-D and compared it with TDLA channel. Our simulation results are captured in the table below:

Table 2: Simulation results comparing TDL-D and TDL-A performance

| Duplex CBW/SCS | Antenna configuration | RB allocation | MCS and rank | Channel model | % of TP | Alignment Results |
| --- | --- | --- | --- | --- | --- | --- |
| TDD 100MHz/120kHz | 2x2 Low | 66 | MCS17, Rank 1 | TDLD30-200 | 70 | 8.3 |
|  |  |  |  | TDLA30-200 | 70 | 9.4 |
|  |  | 32 |  | TDLD30-200 | 70 | 8.7 |
|  |  |  |  | TDLA30-200 | 70 | 9.9 |
| TDD 400MHz/480kHz | 2x2 Low | 20 | MCS13, Rank 1 | TDLD10-200 | 70 | 4.9 |
|  |  |  |  | TDLA10-200 | 70 | 6.0 |

We observe a degradation of 1-1.2 dB with TDL-A channel compared to TDL-D channel.

Observation #1: For test cases in TDL-D channel model with large span, in comparison with TDL-A channel we see performance improvement of 1-1.2 dB with TDL-D channel.

# 3. Conclusion

In this paper we present simulation results for PDSCH demodulation requirements definition for 52.6-71GHz. Our observations and proposals are captured below:

Observation #1: For test cases in TDL-D channel model with large span, in comparison with TDL-A channel we see performance improvement of 1-1.2 dB with TDL-D channel.

# Reference

- R4-2220187, “WF for FR2-2 UE demodulation part”, Qualcomm