3GPP TSG-RAN WG4 Meeting # 106	R4-2300277

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2822 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR RedCap measurement requirement in IDLE mode with eDRX R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a mismatch between the values of PTW length in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 for DRX=1.28s/2.56s and RAN2 agreements on maximum PTW length. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For IDLE intra-frequency and inter-frequency relaxed measurement requirement with eDRX in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 of TS38.133, following parameters and requirement shall be applied: - Regarding DRX=1.28s, K3=4 and N1=4, and the PTW length shall be revised to 40.96s - Regarding DRX=2.56s, the PTW length shall be revised to 40.96s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is a mismatch between the values of PTW length in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 for DRX=1.28s/2.56s and RAN2 agreements on maximum PTW length. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 4.2B.2.9.2 and 4.2B.2.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

##### 4.2B.2.9.2	Measurements for UE fulfilling stationary criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion

The requirements defined in clause 4.2B.2.3 apply for this clause except that:

-	Tdetect,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

-	Tmeasure,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

-	Tevaluate,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.9.2-3 and Table 4.2B.2.9.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table Table 4.2B.2.9.2-5 and Table 4.2B.2.9.2-6 respectively apply provided eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 4.2B.2.9.2-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 1 Rx RedCap UE

| DRX cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x M2 x K3 (36 x M2 x K3) | 1.28 x M2 x K3 (4 x M2 x K3) | 5.12 x M2 x K3 (16 x M2 x K3) |
| 0.64 | 17.92 x K3 (28 x K3) | 1.28 x K3 (2 x K3) | 5.12 x K3 (8 x K3) |
| 1.28 | 32 x K3 (25 x K3) | 1.28x K3 (1 x K3) | 6.4 x K3 (5 x K3) |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 7.68 x K3 (3 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-2: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K3 (36 x N1 x M2 x K3) | 1.28 x N1 x M2 x K3 (4 x N1 x M2 x K3) | 5.12 x N1 x M2 x K3 (16 x N1 x M2 x K3) |
| 0.64 |  | 5 | 17.92 x N1 x K3 (28 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) | 5.12 x N1 x K3 (8 x N1 x K3) |
| 1.28 |  | 4 | 32 x N1 x K3 (25 x N1 x K3) | 1.28 x N1 x K3 (1 x N1 x K3) | 6.4 x N1 x K3 (5 x N1 x K3) |
| 2.56 |  | 3 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 3: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.9.2-3: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle upto 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 7.68 x K3 (3 x K3) |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| 10.24 | 235.52 x K3 (23 x K3) | 10.24 x K3 (1 x K3) | 20.48 x K3 (2 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-4: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle upto 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-5: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥[6.4] ([5]) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K3 (23 x K3) | 0.32 x M2 x K3 (1 x M2 x K3) | 0.64 x M2 x K3 (2 x M2 x K3) |
|  | 0.64 | ≥[12.8] ([10]) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥[15.36] ([12]) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥[30.72] ([24]) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 4: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra_RedCap is expected. Note 5: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.9.2-6: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | 40.96 (32) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 5: The measurement shall not be performed across PTW’s. In this case the measurement is performed in the next available PTW. Note 6: The evaluation shall not be performed across PTW’s. In this case the evaluation is performed in the next available PTW. Note 7: For DRX cycle length is 0.32s, 0.64s, and 2.56s, K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. For DRX cycle length is 1.28s, K3 = 4 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |

End of Change 1

Start of Change 2

##### 4.2B.2.10.2	Measurements for UE fulfilling stationary criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

The requirements defined in clause 4.2B.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 240.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.10.2-3 and Table 4.2B.2.10.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table Table 4.2B.2.10.2-5 and Table 4.2B.2.10.2-6 respectively apply provided that eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 4.2B.2.10.2-1: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 1 Rx RedCap UE

| DRX cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x 1.5 x K4(36 x 1.5 x K4) | 1.28 x 1.5 x K4 (4 x 1.5 x K4) | 5.12 x 1.5 x K4 (16 x 1.5 x K4) |
| 0.64 | 17.92 x K4 (28 x K4) | 1.28x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
| 1.28 | 32 x K4 (25 x K4) | 1.28x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
| 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
| Note 1: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-2: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K4 (36 x N1 x 1.5 x K4) | 1.28 x N1 x 1.5 x K4 (4 x N1 x 1.5 x K4) | 5.12 x N1 x 1.5 x K4 (16 x N1 x 1.5 x K4) |
| 0.64 |  | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
| 1.28 |  | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
| 2.56 |  | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
| Note 1: Applies for RedCap UE of all supporting power class. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-3: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | Tdetect,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-4: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle upto 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-5: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_ Inter _RedCap_Relax and Tevaluate,NR_ Inter _RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥[6.4] ([5]) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K3 (23 x K3) | 0.32 x M2 x K3 (1 x M2 x K3) | 0.64 x M2 x K3 (2 x M2 x K3) |
|  | 0.64 | ≥[12.8] ([10]) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥[15.36] ([12]) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥[30.72] ([4]) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 4: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-6: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | 40.96 (32) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: For DRX cycle length is 0.32s, 0.64s and 2.56s, K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. For DRX cycle length is 1.28s, K3 = 4 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |

End of Change 2