3GPP TSG RAN WG4 Meeting #106                         R4- 2300295

Athens, Greece, February 27 – March 3, 2022

Title:	Discussion on LTM general aspects and scenarios

Source:	China Telecom

Agenda item:			9.23.3.1

Document for:	Discussion

- Introduction

In the RAN4 #105 meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues on general aspects and scenarios are further discussed.

- Discussion

Issue 1-2-1: Whether to consider simultaneous data Rx/Tx with both source cell and target cell during cell switch delay

In last meeting, companies had different views on whether to consider simultaneous data Rx/Tx. Almost companies agree not to consider simultaneous data Rx/Tx in non-CA case. In detail, the way forward during the last meeting is duplicated as below [1]:

<Way forward>: FFS the following options - Option 1 (MTK, Ericsson): UE does not receive or transmit data on source cell after ACK transmission on cell switch command during cell switch delay. In other words, RAN4 to agree that DAPS plus LTM is not supported in Rel-18. - Option 2 (Apple, ZTE): UE is not required to perform simultaneous data Rx or Tx with both source cell and target cell during LTM for both intra-frequency and inter-frequency scenario. - Option 3 (xiaomi): For inter-frequency L1/L2 mobility, - RAN4 not to consider simultaneous reception with both source cell and target cell during L1/L2 inter-cell mobility delay in non-CA case, - RAN4 to consider simultaneous reception with both source cell and target cell during L1/L2 inter-cell mobility delay in CA case. - Option 4 (Huawei): For inter-frequency L1/L2 mobility, not to consider simultaneous RX/TX on serving cell and target cell, except: - For the case that inter-frequency target cell is a current serving SCell (i.e., role change), there is almost zero interruption during cell switch procedure. - Option 5 (vivo): - For R18 LTM, RAN4 assumes that UE needs not to set up 2 RLC entities with different DUs in the inter-DU cell switch, and the corresponding discussion should be done in RAN2. - RAN4 should discuss the ‘simultaneous data Rx/Tx’ in a case-by-case manner. RAN4 not to discuss the simultaneous data Rx/Tx unless for some cases where the impact to RRM/RF/demod requirements is clear. - UE is able to simultaneous Rx with both source cell and target cell for the scenarios at least when - the RTD between source cell and intra-band target cell is within CP in FR1, or - the RTD between source cell and inter-band target cell is within MRTD for inter-band CA in FR1, or - considering single UE panel per FR2 band, the RTD between source cell and inter-frequency target cell is within MRTD for inter-band CA in FR2, and IBM is assumed - UE is able to simultaneous Tx with both source cell and target cell for the scenarios at least when - the Tx timing difference (TTD) between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR1, or - considering single UE panel per FR2 band, the TTD between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR2, and IBM is assumed

In our understanding, simultaneous data Rx/Tx with both source cell and target cell during cell switch delay means that UE simultaneously receives/transmit data from/to both source cell and target cell after UE receives cell switch command.

After receiving cell switch command, the UE usually disconnects from the source cell and initiates the connection with the target cell, which lead to interruption. So DAPS were been proposed to avoid the effects of interruptions, which is beneficial to mobility enhancement. But according to workload  and the main objective in WID[2]. We suggest to deprioritize it.

Observation 1: DAPS is beneficial to mobility enhancement. But it brings more workload and is not closely related to the main objective in WID.

Proposal 1:

Deprioritize simultaneous data Rx/Tx with both source cell and target cell during cell switch delay.

Issue 1-3-2: Definition of inter-frequency cell switch

In last meeting, companies had different views on definition of inter-frequency cell switch. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (CATT, vivo): where the SSBs of active serving cell(s) and the corresponding candidate target cell(s) are on different frequency layers - Option 2 (Apple): where the SSBs of SpCell and the target cell are on different frequency layers. - Option 3 (OPPO): From the point of cell switch, inter-frequency L1/L2-based mobility is considered assuming a current Scell is the target cell with different frequency layers from the SSBs of SpCell. - Other options not precluded.

As moderator clarified, the definitions of intra-frequency and inter-frequency may be different from the point of measurement and the point of cell switch. For the point of cell switch, we think inter-frequency cell switch is that the SSB of Pcell/PScell and the candidate target cell are on different frequency layers.

Proposal 2: Inter-frequency cell switch is that the SSB of Pcell and/or PScell and the candidate target cell are on different frequency layers.

Issue 1-3-3: Whether to cover inter-frequency L1-RSRP measurement

In last meeting, companies had different views on whether to cover inter-frequency L1-RSRP measurement. Some companies think inter-frequency measurement could lead to longer latency. In detail, the Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (MTK): deprioritize the discussion on L1 inter-frequency measurement - Option 2 (Intel): Don’t define inter-frequency L1-RSRP measurement with MG requirement. - Option 3 (CATT, OPPO): Further study whether to cover inter-frequency L1-RSRP measurement from the perspective of reducing measurement delay - give priority to the inter-frequency without gap case, if inter-frequency L1-RSRP measurement needed. - A measurement is regarded as a inter frequency L1-RSRP measurement without gap provided the center frequency and SCS of the SSB of the neighbor cell are different from the SSB of the serving cell, but the SSB of the neighbor cell is in the active BWP of serving cell. - FFS: inter-frequency L1-RSRP measurement with gap - Option 4 (CTC, Xiaomi, ZTE, Huawei, Nokia, Ericsson, CMCC): cover inter-frequency L1-RSRP measurement - Option 4a (Apple): using MG for inter-frequency L1-RSRP can be considered as a baseline. - Option 5 (vivo): - For inter-frequency measurement, further discuss how to avoid making the L1 measurement delay too long for fast cell switch in LTM if it is supposed to be performed within measurement gaps. - For inter-frequency measurement, further discuss whether in R18 to support using intermediate results from L3 measurements in L1-RSRP reporting for both serving cells and candidate cells.

In LS[3] from RAN1, inter-frequency measurement is supported from RAN1 point of view. In revised WID[2], both intra-frequency and inter-frequency are in the scope. And for deployment of network, inter-frequency is important. So we think cover inter-frequency L1-RSRP measurement scenario.

Take the main objective of WID[2] that specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction into according, inter-frequency will extend the measurement delay. So we need find a procedure to reduce the delay caused by inter-frequency L1-RSRP measurement.

For example, in FR1, Tx beams and Rx beams is same for L1-RSRP measurement and L3 measurement. So we can reuse the intermedia result of L3 measurement as L1-RSRP measurement result, in which case, the measurement delay is not almost effected. In other case, i.e.FR2, we should specify whether the Rx and Tx beams are same and further study the procedures to get L1-RSRP measurement.

Observation 2: Both intra-frequency and inter-frequency are in the scope and it is important for the deployment of network.

Observation 3: Inter-frequency measurement will extend the measurement delay.

Proposal 3: Cover inter-frequency L1-RSRP measurement scenario and study the procedure to reduce the delay caused by it.

- For FR1, reuse the intermedia result of L3 measurement as L1-RSRP measurement result.

- For FR2, FFS.

Issue 1-4-1: Whether to consider RTD of serving cell and neighbour cell larger than one CP for intra-frequency L1-RSRP measurement

In last meeting, companies had different views on RTD. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (MTK, Xiaomi, OPPO, vivo): Start the discussion from RTD of serving cell and neighbour cell within one CP for SSB based L1-RSRP measurement. FFS impact to UE complexity, measurement delay and interruptions for RTD>CP. - Option 2 (Intel, Ericsson, QC): No need to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement - Option 3 (Apple): FFS after L1 measurement procedure become clearer and more stable. - Option 4 (CATT, vivo): depends on UE implementation - FFS: Whether to relax the RTD< CP restriction can be an optional capability of UE. - Option 5(Huawei): For SSB based L1-RSRP, discuss whether Rx time difference between serving cell and non serving cell is larger than [x]us. Whether UE supports out of [x]us depends on UE capability.

As shown in table 1, we could find the cyclic prefix duration in FR2 is very short. For 5G, the inter-site distance is about 350~500m, corresponds to propagation timing difference 1.2~1.7us. If restrict the RTD between serving cell and neighbour cell to be within CP, some cases with higher SCS are also restricted. So we think need not to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement.

| Parameter/Numerlogy(u) | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Cyclic Prefix Duration(us) | 4.69 | 2.34 | 1.17 | 0.57 | 0.29 |

Table 1 The cyclic Prefix Duration with different u

Proposal 4: Need not to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement.

Issue 1-5-2: Whether to support non-synchronous

In last meeting, companies had different views on whether to support non-synchronous. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (Apple, Xiaomi): whether to support sync and async can be discussed after L1 measurement procedure become clearer and more stable. - Option 3 (Huawei): depend on UE capability - Option 4: Depends on issue 1-5-1 conclusion

We think before confirming whether to support non-synchronous, we should confirm the definition of synchronous and non-synchronous. So wait for the conclusion of issue 1-5-1.

Proposal 5: Confirm whether to support non-synchronous after confirming the conclusion of issue 1-5-1.

- ummary

In this paper, we provide our views on general aspects and scenarios. From this discussion we have derived the following observations and proposals:

Observation 1: DAPS is beneficial to mobility enhancement. But it brings more workload and is not closely related to the main objective in WID.

Proposal 1: Deprioritize simultaneous data Rx/Tx with both source cell and target cell during cell switch delay.

Proposal 2: Inter-frequency cell switch is that the SSB of Pcell and/or PScell and the candidate target cell are on different frequency layers.

Observation 2: Both intra-frequency and inter-frequency are in the scope and it is important for the deployment of network.

Observation 3: Inter-frequency measurement will extend the measurement delay.

Proposal 3: Cover inter-frequency L1-RSRP measurement scenario and study the procedure to reduce the delay caused by it.

- For FR1, reuse the intermedia result of L3 measurement as L1-RSRP measurement result.

- For FR2, FFS.

Proposal 4: Need not to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement.

Proposal 5: Confirm whether to support non-synchronous after confirming the conclusion of issue 1-5-1.

- References

[1] R4-2220403, WF on L1/L2 inter-cell mobility, MediaTek Inc.

[2] RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc, Apple.

[3] R4-2300009, LS on RAN1 agreements for L1/L2-based inter-cell mobility, Fujitsu, CATT.