3GPP TSG-RAN4 Meeting #106	R4-2300347

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

Agenda item:	8.27.2

Source:	Apple

Title:	On simultaneous Rx-Tx for NR inter-band combinations

Release:	Rel-18

Document for:	Approval

# 1  Introduction

The WI [1] objective is to introduce requirements for simultaneous Rx-Tx for certain band combinations. Based on the WI goals this contribution provides analysis and measurement results for CA_n34A-n41A and CA_n40A-n41A. Additionally, the implementation challenges of CA_n40A-n41A with simultaneous Rx-Tx and the n41 4Rx requirements are discussed.

# 2  Discussion

The WI [1] features a list of band combinations which are under investigation for non-synchronized scenario and specifying applicable requirements for simultaneous Rx-Tx operation. The current baseline is that networks run in synchronized operation between the two bands which allows deployment of low-loss filter with less out-of-band suppression as no de-sense of receiver is expected. Low-loss filter enable increased coverage for UEs and is therefore preferred pick for implementation. In the following we want to provide our considerations and proposals on those two band combinations.

##### 2.1 On MSD for CA_n34-n41

Analysis based on measurements was also done for CA_n34A-n41A with simultaneous Rx-Tx. The proposed MSD values and the UL and DL configuration can be found in Table 1.

Table 1: MSD results for CA_34A-n41A with simultaneous Rx-Tx

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n41 | 2017.5 | 15 | 15 | 78 (RBstart=0) | 2501 | 10 | 2.3 | >ACLR2 |
| n34 | n41 | 2017.5 | 15 | 15 | 78 (RBstart=0) | 2546 | 100 | 2.3 | >ACLR2 |
| n41 | n34 | 2546 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 9.2 | >ACLR2 |
| n41 | n34 | 2456 | 100 | 30 | 270 (RBstart=0) | 2017.5 | 15 | 9.4 | >ACLR2 |

Proposal 1: Consider the MSD proposal from table 1 when defining requirements for CA_34A-n41A with simultaneous Rx-Tx.

##### 2.2 On MSD for CA_n7-n40

The requirements for CA_n7A-n40A were discussed during the last meetings. The original proposed MSD values are currently in brackets and provided below. Those values would be acceptable, and it is proposed to remove the brackets.

Table 5.4.1.4-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1 UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n40; n7; 2350; 100; 30; 270 (RBstart=3); 2622.5; 5; [21.9]; >ACLR2. n40; n7; 2350; 100; 30; 270 (RBstart=3); 2645; 50; [13.5]; >ACLR2.

Proposal 2: Finalise requirements for CA_n7A-n40A by agreeing on proposed MSD values and remove brackets.

##### 2.3 On MSD for CA_n40-n41

The bands n40 and n41 are in near proximity with a minimum frequency separation of 96MHz. The details of our MSD proposal were discussed in [2] and table 2 captures the MSD results.

Table 2: MSD results for simultaneous Rx-Tx with CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 32.0 | ACLR2 |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 25.3 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=1) | 2395 | 10 | 23.8 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=0) | 2350 | 100 | 21.4 | ACLR2 |

Proposal 3: Consider the MSD values provided in Table 2 when defining requirements for CA_n40A-n41A with simultaneous Rx-Tx.

During the initial discussion the ΔTIB,c values for simultaneous Rx-Tx have been discussed and the proposals ranged from 0dB to 0.5dB. The parameter ΔTIB,c covers the degradation in Tx path such as insertion loss from band filter. When a UE is designed for non-simultaneous Rx-Tx it is possible to implement very low loss pass-band filter. Those filters have good in-band performance but less out-of-band suppression. When introducing simultaneous Rx-Tx then different filter with better out-of-band suppression need to be deployed which typically feature increased insertion loss. Due to this change in properties it would be expected that the ΔTIB,c would need to be increased and therefore be larger than 0.5dB. However, considering the initial discussion we would like to propose a compromise which uses the same values of 0.5dB for both cases as captured in Table 3. Similar consideration holds for ΔRIB,c as increased insertion loss affects Rx as well. The proposal is captured in Table 4.

Table 3: ΔTIB,c proposal for CA_n40A-n41A

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration |  |
| CA_n40-n41 | 0.5 | 0.5 |

Proposal 4: As proposed in table 3 use ΔTIB,c = 0.5dB for both bands for CA_n40A-n41A.

Table 4: ΔRIB,c proposal for CA_n40A-n41A

| Inter-band CA combination | ΔRIB,c for NR bands (dB) |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration |  |
| CA_n40-n41 | 0.3 | 0.3 |

Proposal 5: As proposed in table 4 use ΔRIB,c = 0.3dB for both bands for CA_n40A-n41A.

##### 2.4 On simultaneous Rx-Tx and MIMO for CA_n40-n41

According to TS38.101-1 clause 7.2 a UE is required to feature a minimum of four Rx antenna ports for band n41. This requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antenna switch to reduce implementation complexity and number of required antennas.

In case of non-simultaneous Rx-Tx it is possible to share antennas between n40 and n41 as uplink and downlink transmission never occurs at the same time for both bands. However, in case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. If an implementation is done for simultaneous Rx-Tx and MIMO either strong out-of-band filters are required typically featuring high insertion loss or separate antennas are needed. The challenge increases if support for MIMO related features e.g. antenna switching are considered in addition. In case 2x2 UL-MIMO for n40 and a filter with increased insertion loss is not an option then sufficient isolation can be achieved by deploying two dedicated antennas for band n40 Tx. In this scenario four additional antennas would be required for n41 receiver due to the 4Rx requirement. In case of 4-layer DL-MIMO on n41 and single port uplink on n40 five separate antennas would be required. This means with simultaneous Rx-Tx and MIMO operation around 5 to 6 separate antennas would be required. Integrating 6 antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

A solution to the described issue is to relax the 4Rx requirement for n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. This relaxation would only be applicable to handheld devices and allow improved performance of simultaneous Rx-Tx together with MIMO operation on band n40 and n41.

The relaxation for carrier aggregation with simultaneous Rx-Tx between n40 and n41 can be implemented by introducing a new note to Table 7.3.2-1b from TS38.101-1. The note specifies the relaxation for handheld devices only. The proposal can be found below:

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: Four Rx antenna ports are not mandatory for this band when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. This relaxation does not apply to FWA form factor devices..

Observation 1: Due to close frequency distance of n40 and n41 those bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. If an implementation is done for simultaneous Rx-Tx and MIMO either strong out-of-band filters are required typically featuring high insertion loss or separate antennas are needed. The challenge increases if support for MIMO related features e.g. antenna switching are considered in addition. In case 2x2 UL-MIMO for n40 and a filter with increased insertion loss is not an option then sufficient isolation can be achieved by deploying two dedicated antennas for band n40 Tx. In this scenario four additional antennas would be required for n41 receiver due to the 4Rx requirement. In case of 4-layer DL-MIMO on n41 and single port uplink on n40 five separate antennas would be required. This means with simultaneous Rx-Tx and MIMO operation around 5 to 6 separate antennas would be required. Integrating 6 antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Observation 2: Carrier aggregation with simultaneous Rx-Tx including band n41 and n40 might not be feasible to implement for handheld devices if the 4Rx requirement is not relaxed.

Proposal 6: It is proposed to relax the 4Rx requirement for n41 if carrier aggregation is configured with band n40. This relaxation would only be applicable to handheld devices and allow improved performance of simultaneous Rx-Tx together with MIMO operation on band n40 and n41. FWA/CPE type devices would still have mandatory 4Rx requirement as those might be able to handle the implementation due to larger form factor.

# 3  Conclusions

This contribution provides MSD analysis and considerations on simultaneous Rx-Tx with respect to UL-MIMO capable UEs. The following observations and proposal are made:

Proposal 1: Consider the MSD proposal from table 1 when defining requirements for CA_34A-n41A with simultaneous Rx-Tx.

Proposal 2: Finalise requirements for CA_n7A-n40A by agreeing on proposed MSD values and remove brackets.

Proposal 3: Consider the MSD values provided in Table 2 when defining requirements for CA_n40A-n41A with simultaneous Rx-Tx.

Proposal 4: As proposed in table 3 use ΔTIB,c = 0.5dB for both bands for CA_n40A-n41A.

Proposal 5: As proposed in table 4 use ΔRIB,c = 0.3dB for both bands for CA_n40A-n41A.

Observation 1: Due to close frequency distance of n40 and n41 those bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. If an implementation is done for simultaneous Rx-Tx and MIMO either strong out-of-band filters are required typically featuring high insertion loss or separate antennas are needed. The challenge increases if support for MIMO related features e.g. antenna switching are considered in addition. In case 2x2 UL-MIMO for n40 and a filter with increased insertion loss is not an option then sufficient isolation can be achieved by deploying two dedicated antennas for band n40 Tx. In this scenario four additional antennas would be required for n41 receiver due to the 4Rx requirement. In case of 4-layer DL-MIMO on n41 and single port uplink on n40 five separate antennas would be required. This means with simultaneous Rx-Tx and MIMO operation around 5 to 6 separate antennas would be required. Integrating 6 antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Observation 2: Carrier aggregation with simultaneous Rx-Tx including band n41 and n40 might not be feasible to implement for handheld devices if the 4Rx requirement is not relaxed.

Proposal 6: It is proposed to relax the 4Rx requirement for n41 if carrier aggregation is configured with band n40. This relaxation would only be applicable to handheld devices and allow improved performance of simultaneous Rx-Tx together with MIMO operation on band n40 and n41. FWA/CPE type devices would still have mandatory 4Rx requirement as those might be able to handle the implementation due to larger form factor.

# 4  References

- R4-2217123, “Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18”, 3GPP TSG-RAN WG4 Meeting #104-bis-e

- R4-2218106, “On simultaneous Rx/Tx for NR inter-band combinations”, Apple, 3GPP RAN WG4 Meeting #105