3GPP RAN WG4 Meeting #106		R4-2300362

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	8.23.2

Source:	Apple

Title:	PC2 FDD bands MSD with 1Tx

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The basket WID for NR FR1 PC2 FDD bands has been approved in RAN #97-e meeting [1]. The initially proposed bands include n2, n5, n8, n13, n25, n26, n28, n66, n71, and n85 where most of them are expectedly subject to further sensitivity degradation due to increased UL interference caused by 3dB higher maximum output power from PC3 to PC2. In this contribution, we provide our MSD simulation results for n2, n5, n8, n25, n26, n28, n71, and n85 based on 1Tx PC2 configuration. We also resubmit our 2Tx MSD analysis results which have already been presented in RAN4 #104bis-e meeting [2].

# Discussion

## 2.1	PC2 MSD with 1Tx configuration

The MSD analysis is based on the simulation assumptions as summarized in Table 2.1-1 where only Tx noise into Rx band was considered.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Tx power | 26 dBm | At antenna port |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |
| n2 channel BW | UL/DL: 10/10, 15/15, 20/20, 25/25, 30/30, 35/35 | MHz |
| n5 channel BW | UL/DL: 10,10, 15/15, 20/20, 20/25 | MHz |
| n8 channel BW | UL/DL: 10/10, 15/15, 20/20, 20/35 | MHz |
| n25 channel BW | UL/DL: 10/10, 15/15, 20/20, 25/25, 30/30, 35/35, 40/40, 40/45 | MHz |
| n26 channel BW | UL/DL: 10/10, 15/5, 20/20, 20/25, 20/30 | MHz |
| n28 channel BW | UL/DL: 10/10, 15/15, 20/20, 25/25, 30/30 | MHz |
| n71 channel BW | UL/DL: 10/10, 15/15, 20/20, 20/25, 20/30, 20/35 | MHz |
| n85 channel BW | UL/DL: 10/10, 15/15 |  |

Table 2.1-1 Simulation assumptions for MSD analysis

Table 2.1-2 summarizes MSD results from our simulations.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0.8 | 1.6 | 1.8 | 2.5 | 2.8 | 3.5 | 5.1 | N/A | N/A |
| n5 | 0 | 1.2 | 0.9 | 2.1 | 2.6 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 1.2 | 0.9 | 1.9 | N/A | N/A | 4.4 | N/A | N/A | N/A |
| n25 | 0 | 0.6 | 1.0 | 1.4 | 2.1 | 2.4 | 3.0 | 4.7 | 5.1 | N/A |
| n26 | 0 | 0.9 | 0.7 | 1.3 | 1.7 | 2.2 | N/A | N/A | N/A | N/A |
| n28 | 0 | 1.2 | 1.5 | 1.4 | 1.9 | 2.3 | N/A | N/A | N/A | N/A |
| n71 | 0 | 1.6 | 0.6 | 1.6 | 1.8 | 2.0 | 2.8 | N/A | N/A | N/A |
| n85 | 0 | 1.1 | 2.8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2.1-2 1Tx PC2 MSD relative to PC3 REFSENS

Proposal 1: RAN4 to take the MSD values in Table 2.1-2 into consideration for 1Tx PC2 MSD relative to PC3 REFSENS.

## 2.2	PC2 MSD with 2Tx configuration

The 2Tx MSD analysis was based on the PC3 REFSENS requirements to first extract the Tx noise, assuming antenna isolation is 10 dB. The Tx noise was then added to thermal noise for both main and diversity Rx paths, followed by MRC to calculate the equivalent total noise for 2Tx configuration. The MSD was then derived by taking the difference between the 2Tx PC2 equivalent total noise and the PC3 equivalent total noise. To account for the PA reverse IMD when 2PA are transmitting simultaneously, additional 1dB Tx noise contribution was assumed in our analysis.

Table 2.2-1 summarizes MSD results based on our analyses.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0 | 0 | 0 | 0.2 | 6.7 | 6.6 | 7.3 | N/A | N/A |
| n5 | 0 | 0 | 0 | 6.2 | 6.8 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 0 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
| n25 | 0 | 0 | 0 | 0 | 0.1 | 6.9 | 6.8 | 7.5 | 7.9 | N/A |
| n26 | 0 | 0 | 0 | 5.0 | 6.7 | 7.5 | N/A | N/A | N/A | N/A |
| n28 | 0 | 0 | 0 | 2.1 | 7.3 | 8.4 | N/A | N/A | N/A | N/A |
| n71 | 0 | 0 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 | N/A | N/A | N/A |
| n85 | 0 | 0 | 7.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2.2-1 2Tx PC2 MSD relative to PC3 REFSENS

Proposal 2: RAN4 to take the MSD values in Table 2.2-1 into consideration for 2Tx PC2 MSD relative to PC3 REFSENS.

# 3	Conclusion

In this contribution, we provide our MSD analysis results for n2, n5, n8, n25, n26, n28, n71, and n85 based on both 1Tx and 2Tx PC2 configurations.

Proposal 1: RAN4 to take the MSD values in the table below into consideration for 1Tx PC2 MSD relative to PC3 REFSENS.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0.8 | 1.6 | 1.8 | 2.5 | 2.8 | 3.5 | 5.1 | N/A | N/A |
| n5 | 0 | 1.2 | 0.9 | 2.1 | 2.6 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 1.2 | 0.9 | 1.9 | N/A | N/A | 4.4 | N/A | N/A | N/A |
| n25 | 0 | 0.6 | 1.0 | 1.4 | 2.1 | 2.4 | 3.0 | 4.7 | 5.1 | N/A |
| n26 | 0 | 0.9 | 0.7 | 1.3 | 1.7 | 2.2 | N/A | N/A | N/A | N/A |
| n28 | 0 | 1.2 | 1.5 | 1.4 | 1.9 | 2.3 | N/A | N/A | N/A | N/A |
| n71 | 0 | 1.6 | 0.6 | 1.6 | 1.8 | 2.0 | 2.8 | N/A | N/A | N/A |
| n85 | 0 | 1.1 | 2.8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Proposal 2: RAN4 to take the MSD values in the table below into consideration for 2Tx PC2 MSD relative to PC3 REFSENS.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0 | 0 | 0 | 0.2 | 6.7 | 6.6 | 7.3 | N/A | N/A |
| n5 | 0 | 0 | 0 | 6.2 | 6.8 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 0 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
| n25 | 0 | 0 | 0 | 0 | 0.1 | 6.9 | 6.8 | 7.5 | 7.9 | N/A |
| n26 | 0 | 0 | 0 | 5.0 | 6.7 | 7.5 | N/A | N/A | N/A | N/A |
| n28 | 0 | 0 | 0 | 2.1 | 7.3 | 8.4 | N/A | N/A | N/A | N/A |
| n71 | 0 | 0 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 | N/A | N/A | N/A |
| n85 | 0 | 0 | 7.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

# 4	References

RP-222625 “New WID: High power UE for FR1 for FDD single band(s) with power class 2”, China Unicom, 3GPP TSG-RAN Meeting #97-e, Online, September 12th – 16th, 2022

R4-2215661 “MSD analyses for PC2 FDD bands with 2Tx”, Apple, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 10th – 19th, 2022