3GPP RAN WG4 Meeting #106		R4-2300370

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.3.3

Source:	Apple

Title:	On FR1 2UL inter-band CA coexistence requirements

WI/SI:	FS_SimBC

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Along the line of the Rel-18 SI to study on the simplification of band combination specifications for NR and LTE [1], in RAN4 #104-e meeting, it has been proposed that the 2UL inter-band CA UE coexistence requirements is specified only with a normative text [2] which had led to a WF to encourage companies to investigate whether the 2UL inter-band CA UE co-existence requirements can be specified based on the intersection set of the protected bands and frequency ranges from each constituent band without an explicit coexistence table [3]. In RAN4 #104bis-e meeting, several aspects on both single-band and inter-band UL CA coexistence requirements which may lead to the simplification on specifications structure were discussed [4-6]. The discussions had led to an approved WF [7] to encourage companies to:

- Investigate whether the non-3GPP RATs protection can follow the intersection set rule or should be considered as exceptions for FR1 2UL inter-band CA coexistence requirements.

- Identify the cases where the protected bands do not follow the intersection set rule for FR1 2UL inter-band CA coexistence requirements in the current specifications and determine whether they are exceptions or errors.

In last RAN4 meeting, we had proposed that the non-3GPP-RATs protection under 2UL inter-band CA also follows the intersection set rule [8] which however was not consented. In the same contribution [8], we also identified the cases where the protected bands do not follow the intersection set rule for FR1 2UL inter-band CA coexistence requirements in the current specifications only for combinations with n1. Some of the cases were confirmed either as error in the 2UL coexistence requirements or single-band UE coexistence requirements itself.

In this contribution, we first summarize the findings in last RAN4 meeting on the identified cases where the protected bands do not follow the intersection set rule and continue the investigation for the rest of the band combinations.

# Discussion

## 2.1	Summary of findings in last RAN4 meeting

In last RAN4 meeting, we identified the cases where the protected bands do not follow the intersection set rule for FR1 2UL inter-band CA coexistence requirements in the current specifications for the combinations with n1. During the meeting, we received the feedback from one company to confirm that some exception cases are needed and some exception cases are either error in the 2UL coexistence requirements or single-band UE coexistence requirements itself. The findings are summarized in the following observations which can also be visualized in Table 2.1-1 in the “Note” column.

Observation 1: For the protection from CA_n1-n40 to PHS system, the exception was an error. There is no need to leave PHS protection in the list.

Observation 2: For the protection from CA_n1-n74 to PHS system, the exception was an error. There is no need to leave PHS protection in the list.

Observation 3: For the protection from CA_n1-n18 to B40, the exception was an error. There is no need to leave B40 protection in the list. The error has been corrected in RAN4 #105 meeting.

Observation 4: For the protection from CA_n1-n78 to B74, B74 needs to be protected by the combination. The error came from that B74 protection from n78 was missing in single-band UE coexistence requirement which needs to be corrected.

Observation 5: For the protection from CA_n1-n79 to B26, B26 needs to be protected by the combination. The error came from that B26 protection from n79 was missing in single-band UE coexistence requirement which needs to be corrected.

Observation 6: For the protection from CA_n1-n28 to DTV system, further check is needed.

| NR CA Combination | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Protected | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | E-UTRA Band 45 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n1-n5 | E-UTRA Band 22 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 45 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n1-n7 | B39 | 1880 | - | 1895 | -40 | 1 | Yes |  |
|  |  | 1895 | - | 1915 | -15.5 | 5 |  |  |
|  |  | 1915 | - | 1920 | +1.6 | 5 |  |  |
| CA_n1-n8 | E-UTRA Band 52 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n1-n18 | E-UTRA Band 40 | FDL_low | - | FDL_high | -50 | 1 | No | Error |
| CA_n1-n20 | E-UTRA Band 52 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n1-n28 | DTV | 470 | - | 694 | -42 | 8 | Yes | Need further check |
|  |  | 470 | - | 710 | -26.2 | 6 | Yes |  |
|  |  | 662 | - | 694 | -26.2 | 6 | Yes |  |
| CA_n1-n40 | E-UTRA Band 73 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | PHS | 1884.5 | - | 1915.7 | -41 | 0.3 | Yes | Error |
| CA_n1-n74 | PHS | 1884.5 | - | 1915.7 | -41 | 0.3 | Yes | Error |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | Yes |  |
| CA_n1-n78 | E-UTRA Band 32,75,76 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 74 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error in n78 |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n1-n79 | E-UTRA Band 26 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error in n79 |

Table 2.1-1 2UL inter-band CA protected bands not following the intersection set rule

Unfortunately there were still many exception cases in Table 2.1-1 which need to be confirmed.

Proposal 1: RAN4 to confirm whether the remaining exceptions listed in Table 2.1-1 are errors or indeed exceptions in this meeting.

## 2.2	Protected bands not following the intersection set rule

Table 2.2-1 summarizes the protected bands not following the intersection set rule for the band combinations which we have continued investigating, starting from the combinations with Band n2 in Table 6.5A.3.2.3-1 in TS 38.101-1 [9]. As this manual checking process is rather time consuming, we only finished checking the combinations with Band n2. Notice that in the column labelled as “Protected”, “Yes” means the band is protected by the combination but not belonging to the intersection set, “No” means the band is not protected by the combination but belonging to the intersection set.

| NR CA Combination | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Protected | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2-n5 | E-UTRA Band 24 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n105 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n2-n7 | E-UTRA Band 4 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 7 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error in n2 |
|  | E-UTRA Band 10 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error |
|  | E-UTRA Band 38 | 2570 | - | 2575 | +1.6 | 5 | Yes |  |
|  |  | 2575 | - | 2595 | -15.5 | 5 |  |  |
|  |  | 2595 | - | 2620 | -40 | 1 |  |  |
|  | NR Band n105 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n2-n12 | E-UTRA Band 48 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n2-n14 | E-UTRA Band 10 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NB PS | 769 | - | 775 | -35 | 0.00625 | Yes |  |
|  |  | 799 | - | 805 | -35 | 0.00625 |  |  |
| CA_n2-n30 | E-UTRA Band 28 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 50 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 51 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 74 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
| CA_n2-n48 | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 53 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
| CA_n2-n66 | E-UTRA Band 10 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error |
|  | E-UTRA Band 24 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 53 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n105 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n2-n77 | E-UTRA Band 24 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 27 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 28 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 53 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 74 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | E-UTRA Band 85 | FDL_low | - | FDL_high | -50 | 1 | No |  |
|  | NR Band n105 | FDL_low | - | FDL_high | -50 | 1 | No |  |
| CA_n2-n78 | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error in n78 |
|  | E-UTRA Band 7 | FDL_low | - | FDL_high | -50 | 1 | Yes | Error in n2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 13 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 24 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 66 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |
|  | E-UTRA Band 103 | FDL_low | - | FDL_high | -50 | 1 | Yes |  |

Table 2.2-1 2UL inter-band CA protected bands not following the intersection set rule

Observation 7: Band n105 protection could be a miss in the current specifications as the band was just recently introduced.

Observation 8: Band 10 is an obsolete band which should be removed from all the coexistence tables.

Observation 9: n2 should protect E-UTRA band 7 if CA_n2-n7 is a valid band combination.

Observation 10: n78 should protect E-UTRA band 2 if CA_n2-n78 is a valid band combination.

Proposal 2: Remove Band 10 protection from all coexistence tables.

Proposal 3: Add Band 7 to n2 protection band list.

Proposal 4: Add Band 2 to n78 protection band list.

Proposal 5: RAN4 to confirm whether the exceptions listed in Table 2.2-1 are errors or indeed exceptions in this meeting.

# 3	Conclusion

In this contribution, we first summarize the findings in last RAN4 meeting on the identified cases where the protected bands do not follow the intersection set rule and continue the investigation for the rest of the band combinations.

Observation 1: For the protection from CA_n1-n40 to PHS system, the exception was an error. There is no need to leave PHS protection in the list.

Observation 2: For the protection from CA_n1-n74 to PHS system, the exception was an error. There is no need to leave PHS protection in the list.

Observation 3: For the protection from CA_n1-n18 to B40, the exception was an error. There is no need to leave B40 protection in the list. The error has been corrected in RAN4 #105 meeting.

Observation 4: For the protection from CA_n1-n78 to B74, B74 needs to be protected by the combination. The error came from that B74 protection from n78 was missing in single-band UE coexistence requirement which needs to be corrected.

Observation 5: For the protection from CA_n1-n79 to B26, B26 needs to be protected by the combination. The error came from that B26 protection from n79 was missing in single-band UE coexistence requirement which needs to be corrected.

Observation 6: For the protection from CA_n1-n28 to DTV system, further check is needed.

Proposal 1: RAN4 to confirm whether the remaining exceptions listed in Table 2.1-1 are errors or indeed exceptions in this meeting.

Observation 7: Band n105 protection could be a miss in the current specifications as the band was just recently introduced.

Observation 8: Band 10 is an obsolete band which should be removed from all the coexistence tables.

Observation 9: n2 should protect E-UTRA band 7 if CA_n2-n7 is a valid band combination.

Observation 10: n78 should protect E-UTRA band 2 if CA_n2-n78 is a valid band combination.

Proposal 2: Remove Band 10 protection from all coexistence tables.

Proposal 3: Add Band 7 to n2 protection band list.

Proposal 4: Add Band 2 to n78 protection band list.

Proposal 5: RAN4 to confirm whether the exceptions listed in Table 2.2-1 are errors or indeed exceptions in this meeting.

# 4	References

RP-221790 “Revised SID: Study on simplification of band combination specification for NR and LTE”, ZTE Corporation et al, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

R4-2212357 “On FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022

R4-2214448 “WF on FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022

R4-2215665 “On FR1 2UL inter-band CA UE coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 10th – 19th, 2022

R4-2215737 “Views on FR1 inter-band UL CA co-existence requirements”, Samsung, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 10th – 19th, 2022

R4-2216043 “Discussion on 2UL inter-band CA coexistence requirements”, Xiaomi, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 10th – 19th, 2022

R4-2217716 “WF on FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 10th – 19th, 2022

R4-2218116 “On FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14th – 18th, 2022

3GPP TS 38.101-1 V17.7.0 (2022-09)