3GPP TSG-RAN4 Meeting #106		R4-2300392

# Athens, Greece, 27th  Feb. – 3rd  March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7187 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PUR requirement for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TA validation in PUR requirement is FFS and the legacy RSRP-based TA validation is not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify TA validation in PUR requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear PUR requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6A.3, 4.7A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access

#### 4.6A.3.1	Introduction

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331].

#### 4.6A.3.2	Requirements on UE synchronization for transmission using PUR

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage.

The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331].

#### 4.6A.3.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers.

The UE shall acquire SystemInformationBlockType31-NB before initiating transmission using PUR and determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

<Next Change>

### 4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access

#### 4.7A.4.1	Introduction

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331].

#### 4.7A.4.2	Requirements on UE synchronization for transmission using PUR

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage area of the serving cell served by a satellite access node.

The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331].

#### 4.7A.4.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers.

The UE shall acquire SystemInformationBlockType31 before initiating transmission using PUR and determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

<End of Change>