3GPP TSG-RAN WG4 Meeting#106      	R4-2300417

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	Double SUL Acronym

Agenda item:	8.14.2

Source:	Nokia

Document for:	Discussion

# 1	Background

In this contribution we discuss dual-SUL acronym related aspects.

# 2 	Discussion

There was a request from China Telecom sent to reflector to define new CA configurations with dual-SUL, see below.

| New | CA_SUL_n78A-n81A_SUL_n78A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C | PC2 for band n78 | Only FR1 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| New | CA_SUL_n78A-n80A_SUL_n78A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C | PC2 for band n78 | Only FR1 | China Telecom |

The acronyms for “DL CA_SUL_n78A-n81A_SUL_n78A-n84A” and “CA_SUL_n78A-n80A_SUL_n78A-n84A” are something new and without RAN4 agreement how to interpret them. For example, is it so that DL is non-contiguous or contiguous for n78? Looking the UL CA configurations and especially CA_n78C one could think that CA in question is contiguous in nature. If it is so then DL acronym needs some attention as one can easily interpret that in is non-contiguous “A-A” If the intention is that DL is actually non-contiguous then UL cannot be contiguous hence CA_n78C needs to be removed from UL configuration list.

In any case the DL acronym could be smarter as it is not desirable to repeat SUL word multiple times even if SUL is considered as truly essential and useful feature.

So in case DL is contiguous then RAN4 could write

CA_n78C_SUL_n81A-n84A

SUL prefix indicating that preceding bands are not normal. Although it would not be necessary to advertise SUL in acronym as n81 and n84 are SUL bands thus RAN4 could just do

CA_n78C_n81A-n84A

in case DL is non-contiguous then RAN4 could write

CA_n78(2A)_SUL_n81A-n84A or CA_n78(2A)_n81A-n84A

Whatever RAN4 decides discussion is also needed what to do with SUL band combination. Note term band combination is not according to RAN4 custom it is what RAN2 uses. In RAN4 it should be CA configuration.

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n24A-n99A | n24 | 5, 10 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n41A-n80A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |

# 3	Conclusion

In this contribution we have highlighted Double SUL Acronym related problems which are linked into new CA configurations proposed in this meeting. These issues neds to solved before relevant configurations [1] can be added into specifications.

# 4	Reference

[1] RP-223515, New WID: NR CA band combinations with two SUL cells in Rel-18