3GPP TSG-RAN WG4 Meeting # 106                                         R4-2300472

Athens, Greece, February 27 – March 3, 2022

Agenda Item:	9.10.2.2

Source:	Intel Corporation

Title:	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs

Document for:		Discussion

# Introduction

In the last RAN4 meeting, Rel18 NR measurement gap enhance WI was initially discussed. In this paper, the further considerations on requirements for combinations of pre-configured MGs and multiple concurrent MGs under objective 1 in [1]was provided.

The following objectives are considered in this WI: - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps

# Discussion on RRM requirements for Case 1

# Con-MGs rules for multiple Pre-MGs

In the last meeting, there were extensive discussion on Pre-MG activation/deactivation mechanism.

## Issue 3-1-4: [Case 1] Whether to revisit the Con-MGs rules among the following Pre-MG status change < Wayforward >: - Activation/activation - Activation/deactivation - Deactivation/deactivation - Deactivation/activation

In the previous meeting, RAN4 agreed that [5]

### Issue 2-14: Required changes for Pre-MG on collision < Way forward/Agreement >: - For Case 1 (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated. - FFS whether Pre-MG priority can be further decided by the associated MOs being measured

That is when there are multiple pre-MGs within concurrent MGs, the individual pre-MG which was activated will be taken count into concurrent rules (e.g. collision or not).

Observation 1: For the pre-configured individual gap instances within concurrent MGs, the one in the transition status (e.g. activationdeactivation) needs not to be considered when defining concurrent MGs rules(e.g. collision rules).

Proposal 1: It is unnecessary to revisit the Con-MGs rules among Pre-MG status change.

# Gap association

## Issue 3-3-2: [Case 1] Pre-MG association clarification < Wayforward >: - FFS: When NW configures a Pre-MG and a Type-2 MG in ConMGs, RAN4 to further study whether to clarify the UE's behaviour in the following scenarios: - FFS: the MO associated with an activated Pre-MG which doesn’t need to be measured within gap - FFS: the MO associated with a deactivated Pre-MG

As discussed in the previous meetings[2], if there is more than one carrier to be measured (MOs) associated with the same gap instance (e.g. same “MeasGapId-r17” ), the way to (de)activat the pre-MG within concurrent gaps needs to be enhanced.

However, as highlighted the Pre-MG activation rules in TS38.133 below, the pre-MG status shall be determined by all conditions for all MOs associated indeed. As a result, the pre-MG will be always be activated if any of MO still needs the measurement gap.

“A measurement can be performed by the UE without measurement gaps if any of the following conditions is met: - The UE is configured with SSB based intra-frequency measurements, and the conditions defined for SSB based intra-frequency measurement without gaps in Clause 9.2.1 are met, or - The UE is configured with SSB based inter-frequency measurements, and the conditions defined for SSB based inter-frequency measurement without gaps in Clause 9.3.1 are met, or - The UE is configured with CSI-RS based intra-frequency measurements”

Observation 2 : It is sufficient to justify the pre-MG status when the multiple MOs associated with the same Pre-MG. And the changes on current rules of Pre-MG activation/deactivation will extremely increase the total RAN4 efforts.

Therefore, we can propose that:

Proposal 2:  RAN4 needs NOT to further study whether to clarify the UE's behaviour in the following scenarios:

- the MO associated with an activated Pre-MG which doesn’t need to be measured within gap

- the MO associated with a deactivated Pre-MG

## Issue 3-3-1: [Case 1] Explicit and implicit association < Agreement >: - RAN4 to focus on high-level issue and discuss whether to consider implicit association of intra-frequency layers with Pre-MG? - Option 1: RAN4 shall not define implicit association of intra-frequency layers with Pre-MG (RAN4 to extend the explicit association from Rel-17 MGE for defining Case 1 requirements). - Option 2: RAN4 shall consider defining implicit association of intra-frequency layers with Pre-MG.

In Rel17, since there is no joint configuration between pre-MG and concurrent-MG, there is ONLY one available measurement gap. Thus, when UE needs switch the gapless measurement to gap based measurements when any conditions in TS38.133 satisfied, the pre-MG configured can be activated without any ambiguity. In Rel18, if the multiple measurement gaps can be configured together with the pre-MG, when the conditions to trigger the pre-MG activation satisfied, which types of MGs or which exact one MGs shall be used for the measurement object which remanded the gap based measurement after the trigger events occurred could be known explicitly by the association via “MeasObjectNR” IE [4] below also.

Meanwhile, for the “intra-frequency” measurement objects which can be measured within or outside of gap can be defined by “MeasObjectNR IE in which the carrier index (ARFCN value) is same as that of serving cells. UE can perform the measurements on this intra-f measurement object outside of gap if it can be contained by UE’s active BWP. Otherwise, the measurement within gap is needed.

MeasObjectNR information element -- ASN1START -- TAG-MEASOBJECTNR-START MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need R absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, -- Need R nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need R nrofCSI-RS-ResourcesToAverage INTEGER (2..maxNrofCSI-RS-ResourcesToAverage) OPTIONAL, -- Need R quantityConfigIndex INTEGER (1..maxNrofQuantityConfig), offsetMO Q-OffsetRangeList, cellsToRemoveList PCI-List OPTIONAL, -- Need N cellsToAddModList CellsToAddModList OPTIONAL, -- Need N excludedCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N excludedCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N allowedCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N allowedCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N ..., [[ freqBandIndicatorNR FreqBandIndicatorNR OPTIONAL, -- Need R measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL -- Need R ]], [[ smtc3list-r16 SSB-MTC3List-r16 OPTIONAL, -- Need R rmtc-Config-r16 SetupRelease {RMTC-Config-r16} OPTIONAL, -- Need M t312-r16 SetupRelease { T312-r16 } OPTIONAL -- Need M ]], [[ associatedMeasGapSSB-r17 MeasGapId-r17 OPTIONAL, -- Need R associatedMeasGapCSIRS-r17 MeasGapId-r17 OPTIONAL, -- Need R smtc4list-r17 SSB-MTC4List-r17 OPTIONAL, -- Need R measCyclePSCell-r17 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1} OPTIONAL, -- Cond SCG cellsToAddModListExt-v1710 CellsToAddModListExt-v1710 OPTIONAL -- Need N ]], [[ associatedMeasGapSSB2-v1720 MeasGapId-r17 OPTIONAL, -- Cond AssociatedGapSSB associatedMeasGapCSIRS2-v1720 MeasGapId-r17 OPTIONAL -- Cond AssociatedGapCSIRS ]] }

Observation 3: With the current Rel17 gap association with the measurement objects, the intra-f measurement can be associated with the configured measurement gaps, but whether UE performs measurements within or outside gap based on UE’s active BWP.

Proposal 3: RAN4 shall not define implicit association of intra-frequency layers with Pre-MG.

# Overlapping and gap dropping rules

## Issue 3-2-3: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion < Wayforward >: - FFS the options.

For this issue above, it shall be noted that for the pre-MG there is some extra (de)activation delay required (e.g. [5m]ms after BWP switching completion) in comparison Type1/2 MG within the concurrent MGs. That is if there are any other gaps which is overlapped with the pre-MG (de)activation duration (e.g. t2~t3 in Figure 2) whether these gap instances (e.g. one pre-MG being activated/deactivate and other Type2 MG) are collided or not can be FFS.

Figure 1. Overlapping with pre-MG activation duration

Observation 4: When pre-MG being activated duration overlapped with other gaps occasion, there is some ambiguity to justify collisions for these concurrent MGs.

However, in our views during the pre-MG status transition time UE may not to return to other RF chains for the carriers to be measured (e.g. UE RX @f2 in Figure 2) because UE can do this in RTT time which is included in the measurement length. On the other words, the impacts on the ongoing measurements with other gaps(e.g. MG R16 above) because of pre-MG status transition can be avoid.

Observation 5: When justifying collisions due to RF operation on Pre-MG with other measurement gaps, the occasions of pre-MG activated can be considered only.

Proposal 4: It is unnecessary to define any additional capability if collisions on Pre-MG is only considered when Pre-MG being activated.

## Issue 3-2-4: [Case 1] dynamic collisions < Wayforward/Agreement >: - Support of gap combinations including pre-configured MGs (Case 1) that cause dynamic collisions will be subject to new UE capability(ies). - FFS: Dynamic collisions are gap collisions involving a pre-configured MG, where gap instances of other MGs are dropped. - FFS: Gap combinations that cause dynamic collisions when at least one Pre-MGs with higher priority are involved in gap collision. - FFS: Gap combinations that does not cause dynamic collisions when at most one Pre-MG involved in the gap collision, and the Pre-MG is assigned the lowest priority level among all the colliding gaps. - FFS: Define separate UE capability for the scenario where pre-MG is colliding with the other component gap and pre-MG has higher priority

Particularly, some concerns for the dynamic collision were raised, in which the pre-MG switched between ON and off. The priority of the pre-configured MG may be invalid if it was to be deactivated. Thus, whether the new dropping rules shall be defined is FFS.

Figure 2. dynamic collision

For examples, the enhanced dropping can be defined. e.g.

- The pre-MG with higher priority can be dropped when it was to be deactivated. The other concurrent MG collided with the "the higher priority legacy MG or activated pre-MG will be kept.

- The deactived MG can be dropped or not dropped when UE support UE capability

Thus, we can conclude that:

Proposal 5: It is necessary to define some enhanced dropping rules for the dynamic collision in case 1.

# Activation delay

## Issue 3-2-5: [Case 1] Activation/deactivation delay < Wayforward >: - Option 1: RAN4 shall extend the activation when multiple Pre-MG are activated. - FFS whether condition is needed. - FFS: if statuses of multiple Pre-MGs are changed due to the different events, e.g. before completion of the first (de)activation the second Pre-MG is (de)activated, additional delay is expected. - Option 2: RAN4 shall reuse the Pre-MG (de)activation delay from Rel-17 when the (de)activation procedures of multiple pre-MG overlap. - FFS whether condition is needed. - FFS: if statuses of the two Pre-MGs are changed simultaneously, e.g. due to the same event, existing Pre-MG (de)activation delay requirements can be reused

The additional RRM requirements impacted includes the Pre-MG activation delay in 8.19 of TS38.133[3]. In case of multiple activation procedures happened overlap, if the simultaneous activation is not allowed the extension on the delay requirements may be needed.

Figure 3. Concurrent gaps which can be simultaneously activated

Proposal 6: In case of the activation procedures of multiple pre-configured gaps being overlapped, the pre-configured gap activation delay requirements in [3] need to be extended.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI in Rel18 are discussed. The proposals can be summarized as:

Observation 1: For the pre-configured individual gap instances within concurrent MGs, the one in the transition status (e.g. activationdeactivation) needs not to be considered when defining concurrent MGs rules(e.g. collision rules).

Proposal 1: It is unnecessary to revisit the Con-MGs rules among Pre-MG status change.

Observation 2 : It is sufficient to justify the pre-MG status when the multiple MOs associated with the same Pre-MG. And the changes on current rules of Pre-MG activation/deactivation will extremely increase the total RAN4 efforts.

Proposal 2:  RAN4 needs NOT to further study whether to clarify the UE's behaviour in the following scenarios:

- the MO associated with an activated Pre-MG which doesn’t need to be measured within gap

- the MO associated with a deactivated Pre-MG

Observation 3: With the current Rel17 gap association with the measurement objects, the intra-f measurement can be associated with the configured measurement gaps, but whether UE performs measurements within or outside gap based on UE’s active BWP.

Proposal 3: RAN4 shall not define implicit association of intra-frequency layers with Pre-MG.

Observation 4: When pre-MG being activated duration overlapped with other gaps occasion, there is some ambiguity to justify collisions for these concurrent MGs.

Observation 5: When justifying collisions due to RF operation on Pre-MG with other measurement gaps, the occasions of pre-MG activated can be considered only.

Proposal 4: It is unnecessary to define any additional capability if collisions on Pre-MG is only considered when Pre-MG being activated.

Proposal 5: It is necessary to define some enhanced dropping rules for the dynamic collision in case 1.

Proposal 6: In case of the activation procedures of multiple pre-configured gaps being overlapped, the pre-configured gap activation delay requirements in [3] need to be extended.

# References

- RP-222337 New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps

- R4-2216460, WF on further enhancements on measurement gaps and measurements without gaps

- TS38.133 v17.7.0

- TS38.331 v17.1.0

- R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps