3GPP TSG-RAN WG4 Meeting # 106                                                        R4-2300546			R4-22xxxxx

# Athens, Greece, February 27 – March 3, 2023

Title: 	SBFD adjacent channel co-existence simulation results

Source: 	CATT

Agenda item:	9.19.2.1

Document for:	Discussion

- Background

This contribution provides the SBFD adjacent channel co-existence simulation results from our company. Simulation calibration will upload before the meeting.

- Discussion

  - SBFD adjacent channel co-existence simulation scenarios

SBFD adjacent channel co-existence simulation calibration scenarios is specified in table 1, SBFD is{DU}subband configuration.

Table 1: SBFD adjacent channel co-existence simulation calibration scenarios

| FR1 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| --- | --- |
| FR1 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR1 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR1 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |
| FR2 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR2 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |

  - FR1

2.2.1 FR1 SBFD Urban Macro UL

Simulation is under working .

2.2.2 FR1 SBFD Urban Macro DL

Table 2: FR1 SBFD Urban Macro DL SINR and Throughput

| FR1 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 7.02 | 25 | 1.57 | 4.99 |
| Baseline + ACI | 6.01 | 23.89 | 1.39 | 4.77 |
| ((Baseline + ACI) - Baseline)/Baseline | ____ | ____ | 0.1146 | 0.0441 |

Figure1: FR1 SBFD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 4.41%,  5% throughput degrades 11.46%.

2.2.3 FR1 legacy TDD Urban Macro UL

Table 3: FR1 legacy TDD Urban Macro UL SINR and Throughput

| FR1 legacy TDD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -6.79 | 13.37 | 0.11 | 1.8 |
| Baseline + ACI | -8.05 | 12.81 | 0.08 | 1.73 |
| ((Baseline + ACI) - Baseline)/Baseline | ____ | ____ | 0.2727 | 0.0389 |

Figure2: FR1 legacy TDD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 3.89%,  5% throughput degrades 27.27%.

2.2.4 FR1 legacy TDD Urban Macro DL

Table 4: FR1 legacy TDD Urban Macro DL SINR and Throughput

| FR1 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 6.56 | 23.36 | 1.48 | 4.66 |
| Baseline + ACI | 5.18 | 23.04 | 1.26 | 4.6 |
| ((Baseline + ACI) - Baseline)/Baseline | ____ | ____ | 0.1486 | 0.0129 |

Figure3: FR1 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 1.29%,  5% throughput degrades14.86%.

  - FR2

2.3.1 FR2 SBFD Urban Macro UL

Simulation is under working.

2.3.2 FR2 SBFD Urban Macro DL

Table 5: FR2 SBFD Urban Macro DL SINR and Throughput

| FR2 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 9.65 | 24.28 | 2.01 | 4.84 |
| Baseline + ACI | 8.46 | 23.05 | 1.8 | 4.6 |
| ((Baseline + ACI) - Baseline)/Baseline | ____ | ____ | 0.1045 | 0.0496 |

Figure4: FR2 SBFD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 4.96%,  5% throughput degrades10.45%.

2.3.3 FR2 legacy TDD Urban Macro UL

Simulation is under working .

2.3.4 FR2 legacy TDD Urban Macro DL

Table 6: FR2 legacy TDD Urban Macro DL SINR and Throughput

| FR2 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 9.97 | 24.67 | 2.07 | 4.92 |
| Baseline + ACI | 8.97 | 24.19 | 1.89 | 4.82 |
| ((Baseline + ACI) - Baseline)/Baseline | ____ | ____ | 0.0870 | 0.0203 |

Figure5: FR2 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 2.03%,  5% throughput degrades8.70%.

- Conclusion

Table 7: SBFD adjacent channel co-existence simulation results

|  | 5% throughput degradation (%) | 50% throughput degradation (%) |
| --- | --- | --- |
| FR1 SBFD Urban Macro UL |  |  |
| FR1 SBFD Urban Macro DL | 11.46 | 4.41 |
| FR1 legacy TDD Urban Macro UL | 27.27 | 3.89 |
| FR1 legacy TDD Urban Macro DL | 14.86 | 1.29 |
| FR2 SBFD Urban Macro UL |  |  |
| FR2 SBFD Urban Macro DL | 10.45 | 4.96 |
| FR2 legacy TDD Urban Macro UL |  |  |
| FR2 legacy TDD Urban Macro DL | 8.70 | 2.03 |

This contribution summarizes the simulation results from our company. The simulation results show 50% throughput degradation is within the 5% evaluation criteria.

Reference

[1] R4-2217466, “WF for adjacent channel co-existence evaluation of SBFD operation”,  RAN4#104-bis-e

[2] R4- 2220247, “Simulation assumptions for SBFD adjacent channel co-existence study”,  RAN4#105