3GPP TSG-RAN WG4 Meeting #106                                                                                           R4-2300581

Athens, GR, February 27 – March 3, 2023

Source: 	CATT

Title: 	Discussion on MU for adjacent channel selectivity, in-band blocking, in-channel selectivity for FR2-2

Agenda item:	9.13.3

Document for:	Approval

- Introduction

In RAN4 #105 meeting, A WF [1] on MU budget calculations for FR2-2 BS RF. The MUs in WF need further discussion in next meeting.

In this contribution, we provide our views on MU for EIS, ACS, IBB, and ICS for FR2-2.

- Discussion

Referring to MU budget Excel spreadsheets for FR2-2 RX (37941_FR2-2 RX MU calculation_v03.xlsx [2]) in RAN4#105, the MU budget for EIS are copied as below Table 2-1. From table 2-1, we can observe that the differences among different companies are MUPA uncertainty due to use of PA, uncertainty of RF signal generator and Mismatch of transmit chain.

For MUPA uncertainty due to use of PA, PA uncertainty was not used in previous EIS MU for 37<f<43.5GHz, another reason is that because EIS is defined for noise floor, so it is reasonable that MUPA uncertainty due to use of PA is set to 0.

For Uncertainty of the RF signal generator, 1.6dB proposed by Nokia was used for 43.5 GHz < f ≤ 48.2 GHz in TR 37.941,  the Uncertainty of RF signal generator for  52.6<f ≤71.0 GHz can be greater than 1.6dB. so we think the average value between 1.6dB and 2.37dB proposed by Keysight is acceptable. So we propose the Uncertainty of RF signal generator for  52.6<f ≤71.0 GHz as 1.99dB.

For Mismatch of transmit chain, we think that 1.09 dB is more reasonable.

Based on above our discussion, using 0dB MUPA uncertainty due to use of PA, 1.99dB Uncertainty of RF signal generator and 1.09dB Mismatch of transmit chain, we can calculate MU for EIS as 4.9dB (form 4.89dB) based on following table 2-1.

Proposal 1: Define MU for EIS for  52.6<f ≤71.0 GHz as 4.9dB.

Table 2-1: MU budget for EIS copied from 37941_FR2-2 RX MU calculation_v03.xlsx [2]

| Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Distribution of the probability | Divisor based on distribution shape | ci |  |  |  |  |
|  |  |  |  |  | Nokia 43.5GHz<f ≤71GHz | Ericsson 43.5GHz<f ≤71GHz | Keysight, 52.6<f ≤71.0 GHz | CATT 52.6<f ≤71.0 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |
| B2-1a | Misalignment and pointing error of BS | Exp. normal | 2.00 | 1 | 0.1 | 0.1 | 0.10 | 0.10 |
| B2-2 | Standing wave between BS and test range antenna | U-shaped | 1.41 | 1 | 0.15 | 0.15 | 0.15 | 0.15 |
| C1-2 | Uncertainty of the RF signal generator | Gaussian | 1.00 | 1 | 1.6 | 0.3 | 2.37 | 1.99 |
| B2-3 | RF leakage & dynamic range, test range antenna cable connector terminated | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 | 0.01 |
| B2-4a | QZ ripple experienced by BS | Gaussian | 1.00 | 1 | 0.4 | 0.4 | 0.40 | 0.40 |
| B2-9 | Miscellaneous uncertainty | Gaussian | 1.00 | 1 | 0 | 0 | 0.00 | 0.00 |
| MU pa | MUPA Uncertainty due to use of PA |  |  |  |  |  | 1.50 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | Gaussian | 1.00 | 1 | 0.3 | 0.4 | 0.85 | 0.85 |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) | U-shaped | 1.41 | 1 | 0.4 | 0.4 | 1.09 | 1.09 |
| B2-6 | Insertion loss of transmitter chain | Rectangular | 1.73 | 1 | 0.07 | 0.07 | 0.07 | 0.07 |
| B2-7 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 | 0.01 |
| B2-8 | Influence of the calibration antenna feed cable | U-shaped | 1.41 | 1 | 0.21 | 0.21 | 0.21 | 0.21 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | Rectangular | 1.73 | 1 | 0.3 | 0.3 | 0.30 | 0.30 |
| B2-11 | Misalignment positioning system | Exp. normal | 2.00 | 1 | 0 | 0 | 0.00 | 0.00 |
| B2-4b | QZ ripple experienced by calibration antenna | Gaussian | 1.00 | 1 | 0.1 | 0.1 | 0.10 | 0.10 |
| B2-10 | Rotary joints | U-shaped | 1.41 | 1 | 0 | 0 | 0.00 | 0.00 |
| B2-1b | Misalignment and pointing error of calibration antenna | Exp. normal | 2.00 | 1 | 0 | 0 | 0.00 | 0.00 |
| B2-12 | Standing wave between SGH and test range antenna | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| B2-13 | Switching uncertainty | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  | 1.78 | 0.87 | 3.18 | 2.49 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  | 3.48 | 1.71 | 6.24 | 4.89 |

We think that 1.5dB MUPA uncertainty due to use of PA proposed by Keysight can be used for MU for ACS, IBB and ICS for 52.6<f ≤71.0 GHz. Using 4.9dB MU for EIS, 1.99dB MU for test equipment, 1.5dB MU for PA, and 0.4dB  ACLR effect, we can get 7.3dB MU for ACS, IBB and ICS for 52.6<f ≤71.0 GHz shown as following Table 2-2.

Proposal 2: Define MU for ACS, IBB and ICS for 52.6<f ≤71.0 GHz as 7.3dB.

Table 2-2: MU for adjacent channel selectivity, in-band blocking, in-channel selectivity

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- |
|  | 24.25 GHz < f ≤ 29.5 GHz and 37 GHz < f ≤ 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz | 52.6 GHz < f ≤ 71 GHz |
| MUEIS (Expanded uncertainty) | 2.4 | 3.5 | 4.9 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.9 | 1.6 | 1.99 |
| MUPA (Uncertainty due to use of PA) | 0.2 | 0.2 | 1.5 |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.74 | 2.61 | 3.73 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.4 | 5.1 | 7.3 |

- Conclusion

This contribution provides our consideration on MU for EIS, ACS, IBB, and ICS for FR2-2. The following proposals are concluded as follows:

Proposal 1: Define MU for EIS for  52.6<f ≤71.0 GHz as 4.9dB.

Proposal 2: Define MU for ACS, IBB and ICS for 52.6<f ≤71.0 GHz as 7.3dB.

- References

- R4-2220280, WF on MU budget calculations for FR2-2 BS RF, Huawei, RAN4#105

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_105/Inbox/Drafts/%5B105%5D%5B306%5D%20NR_exto71GHz_BSRF/MU%20Excels/37941_FR2-2%20RX%20MU%20calculation_v03.xlsx

- TR 37.941 V17.0.0