3GPP TSG-RAN4 Meeting #106	                                                                  R4-2300587

Athens, Greece, February 27 – March 3, 2023

Title:	Discussion on RRM requirements for Inter-RAT measurement without gap

Source:	CATT

Agenda Item:	9.10.3.2

Document for:	Discussion

1 Introduction

In the previous RAN4 meeting, RRM requirements for Inter-RAT measurement without gap were discussed and the conclusions are captured in the approved WF [1]. This contribution further discusses the topic and presents our views.

The following case differentiation in last meeting is used in this paper for discussion:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

2 Discussion

For the UE capability for case b-1, as we discussed in our companion paper, we think NeedForGapsInfoNR and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE. So for this case b-2, we think on top of ‘nogap-noncsg’ in NeedForNCSG-InfoEUTRAN is enough and no need to further consider NeedForGapsInfoNR. On the other hand, NeedForGapsInfoNR can only indicate intra-frequency and inter-frequency measurement, if we want to indicate inter-RAT LTE, the signaling need to be updated. We don’t see the necessity.

Proposal 1: ONLY on top of ‘nogap-noncsg’ in NeedForNCSG-InfoEUTRAN to define the UE capability to support Case b-1.

For inter-frequency measurement in R16, there is a UE capability interFrequencyMeas-NoGap-r16 and a network flag interFrequencyConfig-NoGap-r16 to indicate whether UE can perform inter-frequency measurement without gap when the SSB is completely contained in the activate BWP.

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

For case b-2, we think it should be similar as the scenario above. So we think we don’t need the capability of the gap information report (e.g. NeedForGap, NeedForNCSG etc.) but a new capability indicating whether UE support measurement without gap when the SSB bandwidth is completely contained in the active BWP can be defined, which is similar as interFrequencyMeas-NoGap-r16.

Proposal 2: For case b-2, define a new capability indicating whether UE support inter-RAT measurement without gap LTE CRS are fully contained within UE’s active BWP.

For inter-RAT measurement with mixed numerology, we understand it does not need additional capability. The same approach with existing requirement which define it as scheduling restriction can be used.

Proposal 3: No additional UE capability is defined for inter-RAT measurement with mixed numerology. Instead it can be considered for scheduling restriction.

For the inter-RAT NR measurement without gap (case a-1), the requirements of inter-frequency measurement without gap based on NeedForGapsInfoNR can be reused. And in the discussion of our companion paper [3], we have discussed the inter-frequency measurement without gap with/without interruption, and we think the conclusion can also be leveraged to this case.

Proposal 4: For the inter-RAT NR measurements without gap (case a-1), the requirements of inter-frequency measurement without gap based on NeedForGapsInfoNR in TS 38.133 can be reused including the interruption.

For inter-RAT LTE measurement without gap (case b-1 and b-2), the requirements for LTE intra-frequency measurement can be used as baseline since the LTE inter-frequency measurement can only be performed with gap.

Proposal 5: For inter-RAT LTE measurement without gap (case b-1 and b-2), the requirements for LTE intra-frequency measurement can be used as baseline.

3 Summary

This contribution discusses RRM requirements for Inter-RAT measurement without gap. The following proposals are made.

Proposal 1: ONLY on top of ‘nogap-noncsg’ in NeedForNCSG-InfoEUTRAN to define the UE capability to support Case b-1.

Proposal 2: For case b-2, define a new capability indicating whether UE support inter-RAT measurement without gap LTE CRS are fully contained within UE’s active BWP.

Proposal 3: No additional UE capability is defined for inter-RAT measurement with mixed numerology. Instead it can be considered for scheduling restriction.

Proposal 4: For the inter-RAT NR measurements without gap (case a-1), the requirements of inter-frequency measurement without gap based on NeedForGapsInfoNR in TS 38.133 can be reused including the interruption.

Proposal 5: For inter-RAT LTE measurement without gap (case b-1 and b-2), the requirements for LTE intra-frequency measurement can be used as baseline.

4 References

- R4-2220360, WF on NR_MG_enh2 Part 2, Intel, RAN4#105

- R4-2220067, Topic summary for [105][221] NR_MG_enh2_part2, Intel, RAN4#105

- R4-2300586, Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR, CATT, RAN4#106