| 3GPP TR 38.858 V0.1.1 (2023-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Evolution of NR Duplex Operation (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Introduction	9

5	Objectives of study	9

6	Subband non-overlapping full duplex (SBFD)	10

6.1	General aspects of SBFD schemes	10

6.2	gNB self-interference handling schemes	10

6.3	Inter-gNB and inter-UE CLI handling schemes	10

7	Feasibility and Performance Evaluation for SBFD	10

7.1	Deployment Scenarios	10

7.2	Evaluation Methodologies	11

7.2.1	System level simulation	11

7.2.2	Link level evaluation	13

7.3	Performance Evaluation Results	13

7.3.1	FR1	13

7.3.2	FR2-1	13

7.4	Feasibility Evaluation	13

7.4.1	FR1	13

7.4.1	FR2-1	13

8	Potential enhancements and analysis for dynamic/flexible TDD	14

8.1	Deployment Scenarios	14

8.2	Evaluation Methodologies	14

8.2.1	System level simulation	14

8.3	Inter-gNB CLI handling schemes	15

8.3.x	Inter-gNB CLI scheme x	15

8.3.x.1 Description of scheme	15

8.3.x.2 Performance evaluation or analysis	15

8.3.x.3 RAN1 specification impacts	15

8.4	Inter-UE CLI handling schemes	16

8.4.x	Inter-UE CLI scheme x	16

8.4.x.1 Description of scheme	16

8.4.x.2 Performance evaluation or analysis	16

8.4.x.3 RAN1 specification impacts	16

9	Feasibility of and impact on RF requirements	16

10	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum	16

11	Conclusions and recommendations	16

Annex <A>: Evaluation methodologies	17

A.1	Layout and UE distribution	17

A.1.1	Layout	17

A.1.2	UE distribution	19

A.2	Interference Modelling	20

A.2.1	gNB self-interference modelling	21

A.2.2	Co-site inter-sector co-channel inter-subband CLI	22

A.2.3	Inter-site gNB-gNB co-channel inter-subband CLI	22

A.2.4	UE-UE co-channel inter-subband CLI	24

A.2.5	gNB-gNB adjacent-channel CLI	25

A.2.6	UE-UE adjacent-channel CLI	25

A.3	Channel Modelling	26

A.4	Performance metrics	30

A.4.1	UPT related performance metrics	30

A.4.2	Latency related performance metrics	30

A.4.3	RU and Unfinished/dropped FTP packets rate	31

A.5	gNB antenna configuration	31

A.6	Traffic model	34

A.7	SBFD subband and slot configurations	35

A.8	Coupling loss definition	36

Annex <B>: System level simulation assumptions	38

Annex <C>: System level simulation calibration	40

C.1	SLS calibration methodology	40

C.2	SLS calibration assumptions	41

Annex <X>: Change history	45

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures the results and findings from the study item "Study on Evolution of NR Duplex Operation " [2]. The purpose of this TR is to document the follows for evolution of NR duplex operation:

-	applicable and relevant deployment scenarios.

-	evaluation methodology and assumptions.

-	possible schemes/enhancements, feasibility and performance evaluation results of subband non-overlapping full duplex and dynamic/flexible TDD.

-	summary of the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum.

This activity involves the Radio Access work area of the 3GPP studies and has potential impacts both on the Mobile Equipment and Access Network of the 3GPP systems.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-213591, New SI: Study on evolution of NR duplex operation.

[3]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[4]	3GPP TR 36.843: "Study on LTE Device to Device Proximity Services; Radio Aspects".

[5]	3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

[6]	3GPP TR 36.873: "3D channel model for LTE".

[7]	3GPP TR 36.814: "Further advancements for E-UTRA physical layer aspects".

[8]	3GPP TR 36.889: "Study on Licensed-Assisted Access to Unlicensed Spectrum".

[9]		3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[10]		3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[11]	Report ITU-R M.2412: "Guidelines for evaluation of radio interface technologies for IMT-2020".

[12]	RP-180524, Summary of calibration results for IMT-2020 self evaluation.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACIR	Adjacent Channel Interference Ratio

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AOA	Azimuth angle Of Arrival

AOD	Azimuth angle Of Departure

AS	Angular Spread

ASA	Azimuth angle Spread of Arrival

ASD	Azimuth angle Spread of Departure

CDF	Cumulative Distribution Function

CLI	Cross link interference

EIRP	Effective Isotropic Radiated Power

IBE	In-Band Emission

ICS	In Channel Selectivity

ISD	Intersite Distance

LOS	Line Of Sight

MIMO	Multiple-Input-Multiple-Output

NLOS	Non-LOS

O2I	Outdoor-to-Indoor

O2O	Outdoor-to-Outdoor

OFDM	Orthogonal Frequency-Division Multiplexing

PRB	Physical Resource Block

RMa	Rural Macro

RSI 	Ratio of self-interference

RSRP	Reference Signal Received Power

RU	Resource Utilization

Rx	Receiver

SBFD	Subband non-overlapping Full Duplex

SI	Self-Interference

SINR	Signal-to-Interference-plus-Noise Ratio

SLS	System Level Simulation

TRP	Transmission Reception Point

Tx	Transmitter

TxRU	Transceiver Unit

UMa	Urban Macro

UMi	Urban Micro

UPT	User Perceived Throughput

ZOA	Zenith angle Of Arrival

ZOD	Zenith angle Of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

# 4	Introduction

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time and CLI handling and RIM for NR were introduced in Rel-16. Nevertheless, further study may be required for CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. The inter-gNB CLI may be due to either adjacent-channel CLI or co-channel-CLI, or both, depending on the deployment scenario. One of the problems not addressed in the previous releases is gNB-to-gNB CLI.

This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

# 5	Objectives of study

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

-	Duplex enhancement at the gNB side

-	Half duplex operation at the UE side

-	No restriction on frequency ranges

The detailed objectives are as follows:

-	Identify applicable and relevant deployment scenarios (RAN1).

-	Develop evaluation methodology for duplex enhancement (RAN1).

-	Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

-	Identify possible schemes and evaluate their feasibility and performances (RAN1).

-	Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

-	Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

-	Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

-	Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

-	Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

-	Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

-	Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

# 6	Subband non-overlapping full duplex (SBFD)

Editor's note: More subsections may be added under section 6 depending on the discussion for SBFD

# 7	Feasibility and Performance Evaluation for SBFD

# 8	Feasibility and Performance evaluation for dynamic/flexible TDD

# 9	Feasibility of and impact on RF requirements

Editor's note: This section captures the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE, as well as feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). This section also captures the evaluation assumptions, methodologies and results developed by RAN4.

## 9.1 SBFD Feasibility on FR1 Wide Area BS aspects

### 9.1.1	Self-interference cancellation analysis

#### 9.1.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9.1.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9.1.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.1.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.1.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9.1.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.1.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.1.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.1.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.1.3	Co-channel inter-sub-band inter-site interference analysis

#### 9.1.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9.1.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9.1.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.1.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.2 SBFD Feasibility on FR1 Medium Range BS aspects

### 9.2.1	Self-interference cancellation analysis

#### 9.2.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9.2.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9.2.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.2.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9.2.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.2.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.2.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.2.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.2.3	Co-channel inter-sub-band inter-site interference analysis

#### 9.2.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9.2.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9.2.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.2.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.3 SBFD Feasibility on FR1 Local Area BS aspects

### 9.3.1	Self-interference cancellation analysis

#### 9.3.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9. 3.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9.3.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.3.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9.3.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.3.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.3.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.3.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.3.3	Co-channel inter-sub-band inter-site interference analysis

#### 9.3.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9.3.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9.3.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.3.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.4 SBFD Feasibility on FR2 Wide Area BS aspects

### 9.4.1	Self-interference cancellation analysis

#### 9.4.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9.4.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9.4.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.4.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.4.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9.4.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.4.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.4.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9. 4.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9. 4.3	Co-channel inter-sub-band inter-site interference analysis

#### 9. 4.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9. 4.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9. 4.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9. 4.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.5 SBFD Feasibility on FR2 Medium Range BS aspects

### 9. 5.1	Self-interference cancellation analysis

#### 9. 5.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9. 5.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9. 5.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9. 5.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9. 5.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.5.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.5.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.5.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.5.3	Co-channel inter-sub-band inter-site interference analysis

#### 9.5.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9.5.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9.5.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.5.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.6 SBFD Feasibility on FR2 Local Area BS aspects

### 9.6.1	Self-interference cancellation analysis

#### 9.6.1.1	analysis approach/framework

Editor's note: This section captures the overall description of all potential approaches. Besides, the limitation for certain approach could also be added if there are strong arguments.

#### 9.6.1.2	interference suppression assumption

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived, e.g. output power, antenna configuration (use the same or half antenna elements), IIP3, SBFD configuration, guard band.

#### 9.6.1.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.6.1.4	feasibility conclusion

Editor's note: This section captures REFSENSE desensitization and the conclusion of SI feasibility.

### 9.6.2	Co-channel inter-sub-band co-site inter-sector interference analysis

#### 9.6.2.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach, which may be very similar to RSIC.

#### 9.6.2.2	interference suppression assumption

Editor's note: This section captures the typical assumption of RF requirements, e.g. antenna configuration (use the same or half antenna elements), inter-sector isolation distance.

#### 9.6.2.3	interference suppression results

Editor's note: This section captures all companies’ analysis results. One example, this section can be separated by different RSIC component as below.

- Tx Frequency isolation capability

- Spatial isolation capability

- Tx Beam nulling capability

- Tx RF IC capability and other tech (before LNA)

- Rx blocker suppression capability

- Rx beam nulling/isolation capability

- Rx digital IC capability

- any other considerations

Besides, multi-carrier/multi-band component can also be added here after finishing single carrier case.

#### 9.6.2.4	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. For example, transfer interference cancellation capability to REFSENSE desensitization with target value to conclude whether it is feasible or not. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.6.3	Co-channel inter-sub-band inter-site interference analysis

#### 9.6.3.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solutions.

#### 9.6.3.2	Co-channel inter-subband Inter-site gNB-gNB CLI modeling

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

#### 9.6.3.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of such feasibility criteria. Not sure whether we need this sub-section.

### 9.6.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility. But since we have separate SBFD feasibility conclusion for SI\inter-sector and inter-site respectively, not sure whether we need this section or not. One alternative option is to only retain this section but deleting the sub-section of feasibility for SI, inter-sector and inter-site.

## 9.7 SBFD Feasibility on FR1 UE aspects

### 9.7.1	Co-channel inter-sub-band interference analysis

#### 9.7.1.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solution.

#### 9.7.1.2	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

#### 9.7.1.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of feasibility criteria.

## 9.8 SBFD Feasibility on FR2 UE aspects

### 9.8.1	Co-channel inter-sub-band interference analysis

#### 9.8.1.1	analysis approach/framework

Editor's note: This section captures the overall description and limitation of all potential approach. Maybe frequency isolation is the only feasible solution.

#### 9.8.1.2	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

#### 9.8.1.3	feasibility conclusion

Editor's note: This section captures the conclusion of feasibility. Noted: until now there is no discussion of feasibility criteria.

## 9.9 Impact on BS RF requirements

Editor's note: This section will also capture adjacent channel co-existence simulation results, i.e. ACLR, ACS, ACIR. About simulation parameters and methodology, they are suggested to be moved into Annex D to make this section clearly.

### 9.9.1	FR1

### 9.9.2	FR2

## 9.10 Impact on UE RF requirements

Editor's note: This section will also capture adjacent channel co-existence simulation results, i.e. ACLR, ACS, ACIR. About simulation parameters and methodology, they are suggested to be moved into Annex D to make this section clearly.

### 9.10.1	FR1

### 9.10.2	FR2

# 10	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum

Editor's note: This section captures the summary of the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

## 10.1 Region 1

## 10.2 Region 2

## 10.3 Region 3

# 11	Conclusions and recommendations

Annex <A>:
Evaluation methodologies

# A.1	Layout and UE distribution

Editor's note: This section captures the layout and UE distribution for SBFD and dynamic/flexible TDD evaluation.

## A.1.1	Layout

Table A.1.1-1: Layout for SBFD and dynamic/flexible TDD evaluation

|  | Layout | Inter-BS (2D) distance | Minimum BS-UE (2D) distance | Minimum UE-UE (2D) distance | BS antenna height |
| --- | --- | --- | --- | --- | --- |
| Indoor office | Single layer - Indoor floor: (12BSs per 120m x 50m) | 20m | 0m | 1m | 3 m |
| Urban Macro/ Dense Urban Macro layer | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | 500m for Urban Macro, 200m for Dense Urban Macro layer | 35m | 1m | 25 m |
| Dense Urban with 2-layer | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer (see Figure A.3.1-1): - Step 1: Randomly drop X micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center). - X=1, 3, 6 or 9 - Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center - Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center. | Macro-to-Macro: 200m Minimum Macro-to-micro-center distance: 42m Minimum Micro-center-to-micro-center distance: 40m | Macro-to-UE: 35m Micro-to-UE: 10m | 1m | 25m for Macro cells and 10m for Micro cells |
| Dense Urban Micro layer | Single layer Only consider the Micro TRPs of Dense Urban with 2-layer network. All users communicate with micro TRPs, i.e. macro cell is only used for determining position of micro TRP. |  |  |  |  |
| 2-layer Scenario B | Two layer (see Figure A.1.1-2) - Layer 1: Urban Macro - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m - Layer 2: Indoor office (baseline) - Only one building randomly dropped in the whole network as in figure A.1.1-2. The building has to be confined within one macro cell area. - 12 (baseline) or 3 (optional) TRPs per 120m x 50m x 3m - The distance between two indoor TRPs: 20m for 12 TRPs, 40m for 3 TRPs - The orientation of the building is fixed as in Figure A.1.1-2 (i.e., the long side of the rectangular is along the x-axis) | Macro-to-Macro: 500m Minimum Macro-to- indoor office center distance: 100m | Macro-to-UE: 35m Indoor TRP-to-UE: 10m | 1m | 25m for Macro cells and 3m for Indoor TRP |

Figure A.1.1-1: Layout for dense urban (3 Micro TRPs per Macro TRP)

Figure A.1.1-2: Layout for 2-layer Scenario B

For SBFD Deployment Case 4, consider 0% and 100% grid shift between two networks for Urban Macro and Dense Urban Macro layer scenarios as shown in Figure A.1.1-3.

Figure A.1.1-3: 0% and 100% grid shifts between two networks

## A.1.2	UE distribution

Table A.1.2-1: UE distribution for SBFD and dynamic/flexible TDD evaluation

|  |  | UE distribution |
| --- | --- | --- |
| FR1 | Indoor office | - 10 users per TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
|  | Urban Macro / Dense Urban Macro layer | - Baseline: UE clustering distribution (NOTE 1) - Baseline: M=20, X=2; Optional: M=10, X=1. - R’ = 25 m. Evaluation results for other values of R’ can be submitted. - Dmacro-to-cluster = 35m+ R’, Dinter-cluster = 2R’ m - UEs dropped within the UE cluster(s) are indoor with 3km/h. UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h. - UE height is 1.5m for all UEs. - Optional: Uniform UE distribution - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell - 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | Dense Urban with 2-layer | - 2/3 users randomly and uniformly dropped around micro TRP centers with radius of R (R = [28.9m]), 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area. - 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | 2-layer Scenario B | - Layer 1: Urban Macro - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell outside the Indoor office / Indoor factory - Baseline: 100% outdoor without car penetration loss (3km/h), UE height is 1.5m - Optional: 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) - Layer 2: Indoor office (baseline) - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
| FR2-1 | Indoor office | - 10 users per TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
|  | Dense Urban Macro layer | - Baseline: UE clustering distribution (NOTE 1) - Baseline: M=10, X=1; Optional: M=20, X=2. - R’ =20m. Lower values of R’ are not excluded. - Dmacro-to-cluster = 35m+R’, Dinter-cluster = 2R’ m - All the UEs (including UEs in the clusters and out of the clusters) are outdoor UEs without car penetration loss (3km/h). - UE height is 1.5m for both outdoor and indoor UEs. - Optional: Uniform UE distribution - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell. - Baseline: 100% outdoor without car penetration loss (3km/h). Outdoor UE height is 1.5 m. - Optional: 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m; - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | Dense Urban Micro layer | - All users are randomly and uniformly dropped around Micro TRP center with the radius of R (R = [28.9m]) |
| NOTE 1: The UE clustering distribution is assumed as below: - Randomly drop X (X =1 or 2) UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Assuming M (M=10 or 20) users per macro TRP per direction, 80% UEs are randomly and uniformly dropped within the UE clusters with the radius of R’, 20% users randomly and uniformly dropped in the macro geographical area outside the clusters. If each UE is assigned both UL traffic and DL traffic, there are 8 UEs in one UE cluster. If each UE is either assigned UL traffic or DL traffic, there are 8 UEs with DL traffic and 8 UEs with UL traffic. - The UEs in the same cluster are in the same building. For the case with M=20 and X=2, the UEs in different clusters are in different buildings. - Note that the UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area). |  |  |

# A.2	Interference Modelling

Editor's note: This section captures the interference modelling methods for duplex evolution study. The details of interference modelling described in Annex A.2.1~A.2.6 will be further revised based on RAN4 reply LS and more progress in RAN1 and RAN4.

Consider the following definitions of interference types for duplex evolution study.

Table A.2-1: Definition of interference types

| Interference types |  |  | Definition |
| --- | --- | --- | --- |
| Co-channel interference | Co-channel intra-subband interference | gNB-UE co-channel intra-subband interference | This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. |
|  |  | UE-gNB co-channel intra-subband interference | This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. |
|  |  | (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI | CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. |
|  |  | (inter-cell) co-site inter-sector co-channel intra-subband CLI | CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. |
|  |  | (inter-cell) UE-UE co-channel intra-subband CLI | CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. |
|  | Co-channel inter-subband interference | gNB self-interference (SI) | Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency (see Annex A.2.1 for detailed modelling). |
|  |  | (inter-cell) co-site inter-sector co-channel inter-subband CLI | CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.2 for detailed modelling). |
|  |  | (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI | CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.3 for detailed modelling). |
|  |  | (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI | CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.4 for detailed modelling). |
| Adjacent channel interference |  | gNB-gNB adjacent-channel CLI | CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier. This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI (see Annex A.2.5 for detailed modelling). |
|  |  | UE-UE adjacent-channel CLI | CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier (see Annex A.2.6 for detailed modelling). |
| NOTE: This does not imply all the above interference types are needed in evaluation for SBFD. Some of the interferences may not be used according to the deployment scenarios, e.g., whether the SBFD subband configurations are the same or different across gNBs. |  |  |  |

## A.2.1	gNB self-interference modelling

For gNB self-interference modelling in SLS of SBFD in RAN1, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference.

For SLS of SBFD in RAN1, the RSI is modelled as frequency flat within the UL subband. The residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power is computed by

ISImax=PtxmaxαSI*NULRBNDLRB#A-1

where

- ISImax is the residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power (in linear scale).

- Ptxmax is the maximum gNB DL Tx Power on the two DL subbands (in linear scale).

- NDLRB is the total number of DL RBs in the DL subbands.

- NULRB is the total number of UL RBs in the UL subband.

- αSI is the overall RSI value provided by RAN4 in linear scale. RAN1 assumes frequency isolation value in the overall RSI value ranges provided by RAN4 is based on the assumption of SBFD subband configuration with {DUD=40MHz:20MHz:40MHz} at least for FR1 and all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power. Candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. Companies shall report what values of the individual components are assumed in order to achieve the αSI value corresponding to 1 dB desense. Other approaches of determining values for αSI are not precluded and can be used and reported by companies.

Assuming αSI doesn’t change when DL RBs are not fully allocated for DL transmission, the residual self-interference power on one UL RB when DL RBs are not fully allocated for DL transmission is computed by

ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB#A-2

where

- ISIper-RB is the residual self-interference power on one UL PRB when all the DL RBs in the DL subbands are not fully allocated for DL transmission (in linear scale).

- Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB.

- Nused-DL-RB is the number of DL RBs allocated for DL transmission.

## A.2.2	Co-site inter-sector co-channel inter-subband CLI

For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows.

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB#A-3

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB#A-4

where

-	Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

-	Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

-	NDLRB is the total number of DL RBs in the DL subbands.

-	Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

-	αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

-	10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for Tx leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

-	Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived.

## A.2.3	Inter-site gNB-gNB co-channel inter-subband CLI

For discussion of gNB-gNB co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, the power of inter-site gNB-gNB co-channel inter-subband CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as

IInter-Site-CLIA'→A,per-RB=PtxBS A',per-RB*Nused-DL-RBBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS*1NDLRB#A-5

where

-	IInter-Site-CLIA'→A,per-RB is the power of inter-site gNB-gNB co-channel inter-subband CLI from aggressor gNB A' to victim gNB A on each receiver chain at one UL RB (linear value).

-	PtxBS A',per-RB is DL transmission power of gNB A' across all transmit chains per RB (linear value). PtxBS A',per-RB=Ptxmax/NDLRB.

-	Nused-DL-RBBS A' is the number of DL RBs allocated for DL transmission by gNB A'.

-	CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB.

-	CLBS A'→BS A=1ST∙p=0ST-11SR∙u=0SR-1CLp,uA',AwA',gA, wherein, ST is the number of Tx antenna ports of gNB A', and SR is the number of Rx antenna ports of gNB A.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e., ACLRBS) is provided as the candidate for Tx leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment.

-	Note: the model is based on the assumption that the same transmission power across different DL RBs is used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS.

-	Note: This model is not applicable to the RBs in the guardband.

-	Note: This model is not applicable for some candidate gNB-gNB CLI handling schemes (for example, spatial digital beam coordination, advanced receivers).

For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB A can be modelled as

IInter-Site-CLIn=Ileakagen+Iselectivity#A-6

where

-	Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB A'.

-	HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn.

-	NT is the number of Tx chains at aggressor gNB.

-	NR is the number of Rx chains at victim gNB.

-	yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB.

-	NT is the number of Tx chains at aggressor gNB.

-	ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise.

-	σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB  is the total leakage power at UL RB n at aggressor gNB.

-	Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB.

-	Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	W is the NT×NT normalized identity matrix with unit norm, WF=1.

-	It is up to companies to report other values of W and the corresponding applicable conditions.

-	Iselectivity is the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB A. It can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT#A-7

-	zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise.

-	σz,n2=PblockerICSBS×NDLRB.

-	Pblocker=1NR*m∈Used DL RBsHCLImWmsm2.

-	HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm.

-	Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1.

-	sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received.

-	Note: Pblocker can be reported by companies.

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment.

-	Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS.

-	Note: This model is not applicable to the RBs in the guardband.

## A.2.4	UE-UE co-channel inter-subband CLI

For discussion of UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g., receiver blocking at the victim, overload of the receiver dynamic range, etc)

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point.

## A.2.5	gNB-gNB adjacent-channel CLI

Regarding gNB-gNB adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g., receiver blocking at the victim, overload of the receiver dynamic range, etc).

For inter-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modelling with gNB ACLR for TX leakage and gNB ACS for Receiver impairment.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for inter-site gNB-gNB adjacent-channel channel model, the power of inter-site gNB-gNB adjacent-channel CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as

IgNB-gNB-adjacent-channel-CLIA'→A,per-RB=PtxBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS*1NRBtotal#A-8

where

-	IgNB-gNB-adjacent-channel-CLIA'→A,per-RB is the power of inter-site gNB-gNB adjacent-channel CLI from gNB A' to gNB A on each receiver chain at one UL RB (linear value).

-	PtxBS A' is DL transmission power of gNB A' across all transmit chains over all DL RBs (linear value).

-	CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB.

-	NRBtotal is the total number of RBs of the channel bandwidth (e.g., 100MHz for FR1) of the aggressor gNB.

-	Note: ACLRBS  (i.e., gNB ACLR) and ACSBS (i.e., gNB ACS) are in linear scale. With this assumption, in absence of further RAN4 inputs, gNB ACLR and gNB ACS in current specification are used for both inter-site gNB-gNB co-channel inter-subband CLI modelling and inter-site gNB-gNB adjacent-channel CLI modelling. The values of ACLRBS and ACSBS used in inter-site gNB-gNB co-channel and adjacent-channel CLI modelling can be revisited based on further RAN4 inputs.

-	Note: This model is not applicable to the RBs in the guard band between the two adjacent channels.

## A.2.6	UE-UE adjacent-channel CLI

Regarding UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

For UE-UE adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modelling with UE ACIR used in Rel-16 CLI study.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE adjacent-channel channel model, the power of UE-UE adjacent-channel CLI experienced by the victim UE on each receiver chain at one DL RB can be modelled as

IUE-UE-adjacent-channel-CLIB'→B,per-RB=PtxUE B'*CLUE B'→UE B*1ACIRUE*1NRBtotal#A-9

where

-	IUE-UE-adjacent-channel-CLIB'→B,per-RB is the power of UE-UE adjacent-channel CLI from UE B' to UE B on each receiver chain at one DL RB (linear value).

-	PtxUE B' is UL transmission power of UE B' across all transmit chains over all UL RBs (linear value).

-	CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE.

-	NRBtotal is the total number of RBs of the channel bandwidth (e.g., 100MHz for FR1) of the aggressor UE.

-	Note: ACIRUE are in linear scale. In RAN4 reply LS, RAN4 agree on UE ACLR based model on Tx and UE ACS based model on Rx which is the same ACIR model as Rel-16 CLI study as starting point.

-	Note: This ACIR-based model is not applicable to the guard band between the two adjacent channels.

# A.3	Channel Modelling

Editor's note: This section captures the channel modelling for SBFD and dynamic/flexible TDD evaluation, including gNB-UE channel model, gNB-gNB channel model and UE-UE channel model.

Table A.3-1: gNB-UE channel model

|  | Urban Macro, Dense Urban | Indoor office | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1) | FR1: - TRP-to-UE: InH-Office in TR 38.901 - Penetration loss is not modelled. FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - Penetration loss is not modelled. | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1) - Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 (NOTE 1) |
| Fast fading parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 | FR1: - TRP-to-UE: InH-Office in TR 38.901 FR2-1: - TRP-to-UE: InH-Office in TR 38.901 | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 |
| NOTE 1: For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; Otherwise the building type is high loss. |  |  |  |

Table A.3-2: gNB-gNB channel model

|  | Urban Macro, Dense Urban | Indoor office | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters (NOTE 2) | FR1: - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) | FR1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) - Penetration loss is not modelled. FR2-1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) - Penetration loss is not modelled. | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 (NOTE 1) - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| Fast fading parameters | FR1: - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 | FR1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| NOTE 1: For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; Otherwise the building type is high loss. NOTE 2: LOS probability of gNB-gNB channel: - For Macro-to-Macro, if the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to 75%; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. |  |  |  |

Table A.3-3: UE-UE channel model

|  | Urban Macro, Dense Urban | Indoor hotspot | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters | FR1: - Option 1 (baseline): UE-to-UE: A.2.1.2 in TR36.843 (NOTE 1), penetration loss between UEs follows Table A.2.1-12 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 - Use NLOS when two indoor UEs are in different buildings. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 - Use NLOS when two indoor UEs are in different buildings. | FR1: - Option1 (baseline): UE-to-UE: A.2.1.2 in TR36.843 (NOTE 1) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) | FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (NOTE 1) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-12 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (NOTE 1). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (NOTE 1). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-12 in TR38.802 |
| Fast fading parameters | FR1: - Option 1 (baseline): UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Option 1 (baseline): UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA | FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| NOTE 1: Regarding Option 1 of UE-UE channel model for FR1, - For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-12 in TR38.802. - For Indoor to Indoor case, additional 6dB should be added in pathloss to support 4GHz carrier frequency. |  |  |  |

# A.4	Performance metrics

Editor's note: This section captures the definition of performance metrics for SBFD and dynamic/flexible TDD evaluation.

## A.4.1	UPT related performance metrics

UPT is defined as the size of an FTP packet divided by the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver [Refer to TR36.814]. Unfinished FTP packets should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished FTP packet by the end of the simulation is divided by the served time (simulation end time – file arrival time) [Refer to TR36.889]. Consider zero bit for dropped FTP packets.

Mean/5%/50%/95% Average-UPT:

-	Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

-	Average-UPT CDF: the CDF of the Average-UPTs for all users.

-	Mean/5%/50%/95% Average-UPT: the mean/5%/50%/95% value of Average-UPTs for all users.

Mean/5%/50%/95% Tail-UPT:

-	Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

-	Tail-UPT CDF: the CDF of the Tail-UPTs for all users.

-	Mean/5%/50%/95% Tail-UPT: the mean/5%/50%/95% value of Tail-UPTs for all users.

Mean/5%/50%/95% Median-UPT:

-	Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

-	Median-UPT CDF: the CDF of the Median-UPTs for all users.

-	Mean/5%/50%/95% Median-UPT: the mean/5%/50%/95% value of Median-UPTs for all users.

## A.4.2	Latency related performance metrics

Packet latency is defined as the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver. HARQ re-transmission should be considered for latency evaluation. Unfinished/dropped FTP packets are not incorporated in the packet latency calculation.

Mean/5%/50%/95% Packet-Latency:

-	Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

-	Packet-Latency CDF is the CDF of the packet latencies of all the packets from all the UEs.

-	Mean/5%/50%/95% Packet-Latency is the mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

Mean/5%/50%/95% UE-Average-Latency:

-	Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

-	UE-Average-Latency CDF is the CDF of the UE-Average-Latency for all users.

-	Mean/5%/50%/95% UE-Average-Latency is the mean/5%/50%/95% value of UE-Average-Latency for all users.

## A.4.3	RU and Unfinished/dropped FTP packets rate

Type-1 RU:

-	DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time.

Type-2 RU:

-	DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time.

For both type-1 RU and type-2 RU, in case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once.

Unfinished/dropped Packet Rate:

-	Unfinished/dropped Packet Rate is defined as the number of the unfinished packets for all users divided by the total number of generated packets for all users

# A.5	gNB antenna configuration

Editor's note: This section captures the gNB antenna configuration for SBFD evaluation.

For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption.

The separate-Tx/Rx antenna array for description of evaluation assumption can be modelled by two panel groups as in Figure A.5-1.

-	Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group:

-	M: Number of vertical antenna elements within a panel, on one polarization

-	N: Number of horizontal antenna elements within a panel, on one polarization

-	P: Number of polarizations

-	Mg: Number of panels in a column within a panel group.

-	Ng: Number of panels in a row within a panel group.

-	dg,H: Antenna panel spacing in horizontal direction within a panel group.

-	dg,V: Antenna panel spacing in vertical direction within a panel group.

-	Companies are to report the separation of the two panel groups. Introduce new parameters da,H,da,V as illustrated in the following Figure A.5-1.

-	da,H: Panel group spacing in the horizontal direction. Typically, da,H = 0.

-	da,V: Panel group spacing in the vertical direction.

Figure A.5-1: Separate-Tx/Rx antenna array model

For evaluation and comparison between SBFD and legacy TDD, three options can be used.

-	SBFD antenna configuration option-1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

-	SBFD antenna configuration option-2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

-	SBFD antenna configuration option-3: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

These options are further clarified with examples in the following:

As shown in Figure A.5-2 (a), for legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is M,N,P,Mg,Ng;Mp,Np. The total number of TxRUs is K=PMpNpMgNg, and the total number of antenna elements is L=PMNMgNg.

For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 1 (see Figure A.5-2 (b)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2.

-	In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is 2L=2PMNMgNg(two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 2-1 (see Figure A.5-2 (c)):

-	In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

-	In UL slots, L antenna elements on panel group#2 are connected to K Rx chains.

-	In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

-	Method 2-2 (see Figure A.5-2 (d)):

-	In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

-	In UL slots, L antenna elements on panel group#1 are connected to K Rx chains.

-	In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K/2=PMpNpMgNg/2 (half of that for legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 3-1 (see Figure A.5-2 (e)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains.

-	In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

-	Method 3-2 (see Figure A.5-2 (f)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1.

-	In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1.

(a) Legacy TDD with shared-Tx/Rx antenna array

(b) SBFD antenna configuration Option-1 (Method 1)

(c) SBFD antenna configuration Option-2 (Method 2-1)

(d) SBFD antenna configuration Option-2 (Method 2-2)

(e) SBFD antenna configuration Option-3 (Method 3-1)

(f) SBFD antenna configuration Option-3 (Method 3-2)

Figure A.5-2: Illustration of antenna configuration of legacy TDD and SBFD

# A.6	Traffic model

Editor's note: This section captures the traffic model for SBFD and dynamic/flexible TDD evaluation.

Table A.6-1: Traffic model of FTP model 3 for SBFD evaluation

|  | 1-layer scenario for SBFD Deployment Case 1 | 2-layer scenario for SBFD Deployment Case 1 and Case 3-2 | SBFD Deployment Case 4 |
| --- | --- | --- | --- |
| General | UL and DL are simulated simultaneously. Companies to report which option is used (NOTE 1). - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL - Option 2: Each UE is assigned both UL traffic and DL traffic. |  |  |
| FTP packet size | Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL |  |  |
| UL/DL arrival rate for baseline legacy TDD case | The UL/DL packet arrival rates are selected so that baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL packet arrival rates are selected for each layer independently so that each layer of baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL packet arrival rates are selected for each operator independently so that each operator of baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). |
| UL/DL arrival rate for SBFD case | The packet arrival rates for SBFD case are the same as that for baseline legacy TDD case. | The packet arrival rates are kept the same for each corresponding layer in baseline legacy TDD case (i.e., legacy TDD for both layer-1 and layer-2) and SBFD deployment case (i.e., SBFD for both layer-1 and layer-2 for SBFD Deployment Case 1, or layer-1 with legacy TDD and layer-2 with SBFD for SBFD Deployment Case 3-2) respectively. | The packet arrival rates are kept the same for each corresponding operator in baseline legacy TDD case (i.e., legacy TDD for both Operator#1 and Operator#2) and SBFD deployment case 4 (i.e., legacy TDD for Operator#1 and SBFD for Operator#2) respectively. |
| NOTE 1: For M UEs distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs. |  |  |  |

Table A.6-2: Traffic model of FTP model 3 for dynamic/flexible TDD evaluation

|  | 1-layer scenario | 2-layer scenario |
| --- | --- | --- |
| General | UL and DL are simulated simultaneously. Companies to report which option is used (NOTE 1). - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL Option 2: Each UE is assigned both UL traffic and DL traffic. |  |
| FTP packet size | Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL |  |
| UL/DL arrival rate determination method | The UL/DL arrival rates are selected so that network using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL arrival rates are selected for each layer independently so that each layer using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). |
| NOTE 1: For M UEs distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs. |  |  |

# A.7	SBFD subband and slot configurations

Editor's note: This section captures the SBFD subband and slot configurations for SBFD evaluation.

For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation, one or more of the following alternatives can be considered:

-	Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

-	Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

-	Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 25% of the channel bandwidth.

-	Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

Regarding the SBFD subband configuration for Alt 1/2/4, SBFD Subband configuration#1 with {DUD} pattern as following is assumed.

-	For FR1

-	Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5>

-	Optional: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <106, 51, 5>

-	For FR2-1

-	Baseline: 100MHz channel bandwidth and 120kHz SCS (66 PRB) < ND, NU, NG > = <25, 14, 1>

-	Optional: 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <47, 32, 3>

wherein,

-	ND is the number of RBs in one DL subband.

-	NU is the number of RBs in one UL subband.

-	NG is the number of RBs in one guard band between one UL subband and one DL subband.

-	Other values of < ND, NU, NG > are not precluded and can be reported by companies.

Regarding the SBFD slot configurations, companies should report the guard symbols assumed in the SBFD operation.

# A.8	Coupling loss definition

Consider following for the definition of coupling loss CLp,uA,BwA,gB from Tx antenna port p of transmitter A to Rx antenna port u of receiver B:

If both large scale fading and small scale fading are modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (1) which is based on formula (B.1-2) in TR 37.910.

CLp,uA,BwA,gB=PL∙SF∙α0,u,p2+n=1Nm=1Mαn,m,u,p2#A-10

If only large scale fading is modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (2).

CLp,uA,BwA,gB=PL∙SF∙α'0,u,p2#A-11

α'0,u,p=Frx,u,θθLOS,ZOA,ϕLOS,AOAFrx,u,ϕθLOS,ZOA,ϕLOS,AOAT∙100-1∙Ftx,p,θθLOS,ZOD,ϕLOS,AODFtx,p,ϕθLOS,ZOD,ϕLOS,AOD#A-12

where

-	wA (wA=w1A,w2A,⋯,wNTA) represents a complex weight vector used for virtualization of Tx antenna port p of transmitter A, and gB (gB=g1B,g2B,⋯,gNRB) represents a complex weight vector used for virtualization of Rx antenna port u of receiver B.

-	Formula (A-12) can be understood according to equation (7.5-29) in TR38.901.

Annex <B>:
System level simulation assumptions

Table B-1: SLS assumptions common to SBFD and Dynamic/flexible TDD

| Parameters | FR1 | FR2-1 |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| BS antenna array configuration | Companies to report the BS antenna configurations |  |
| UE antenna configuration | Companies to report the UE antenna configurations |  |
| BS antenna radiation pattern | - InH: reuse Table 10 in Report ITU-R M.2412 - Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 Companies can also consider evaluation with other realistic BS antenna radiation pattern |  |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | Reuse Table 11 in Report ITU-R M.2412 |
| Channel Model | Refer to Annex A.3 |  |
| Traffic model | Refer to Annex A.6 |  |
| Layout | Refer to Annex A.1.1 |  |
| UE distribution | Refer to Annex A.1.2 |  |
| System bandwidth | 100MHz | Baseline: 100MHz Optional: 200MHz |
| Numerology | 14 OFDM symbol slot, SCS = 30kHz | 14 OFDM symbol slot, SCS = 120kHz |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Open loop power control parameters | Companies to report power control parameters. |  |
| BS receiver noise figure | 5dB | 7dB |
| UE receiver noise figure | 9 dB | 13 dB |
| UE receiver | MMSE-IRC as the baseline receiver. NOTE: Advanced receiver is not precluded. |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Companies to report the option used. Option 1: Ideal Option 2: Realistic |  |
| UE processing capability | UE processing capability 1 as baseline |  |
| Handover margin | 3 dB |  |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |
| DL/UL Modulation | Up to 256QAM |  |
| Transmission scheme | Companies to report transmission schemes (e.g., SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO, etc) |  |
| Scheduling | PF |  |
| Overhead | Companies to report the overhead assumption |  |

Table B-2: SLS assumptions specific to SBFD Deployment Case 1

|  | FR1 |  |  | FR 2-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters | Indoor office | Urban Macro / Dense Urban Macro layer | Dense Urban with 2-layer | Indoor office | Dense Urban Macro layer | Dense Urban Micro layer |
| BS transmit power for legacy TDD | 24 dBm for 100MHz | Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Dense Urban Macro layer - Option 1: 53 dBm for 100MHz - Option 2: 44 dBm for 100MHz | Dense Urban Macro layer - Option 1: 53 dBm for 100MHz - Option 2: 44 dBm for 100MHz Dense Urban Micro layer - 38 dBm for 100MHz | 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. | 40 dBm for 100MHz or 43dBm for 200MHz | 30 dBm for 100MHz or 33dBm for 200MHz. EIRP should not exceed 68 dBm. |
| BS transmit power for SBFD | Companies to report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation. |  |  |  |  |  |
| SBFD subband and slot configurations | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. |  |  |  |  |  |

Table B-3: SLS assumptions specific to SBFD Deployment Case 3-2 (FR1)

| Parameters | 2-layer Scenario B |
| --- | --- |
| BS transmit power for legacy TDD | Layer 1: Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Layer 2: Indoor office - 24 dBm for 100MHz |
| BS transmit power for SBFD | Companies to report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation. |
| SBFD subband and slot configurations | Layer 1; Layer 2. baseline legacy TDD network (Baseline for comparison with SBFD Deployment Case 3-2); Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]. SBFD Deployment Case 3-2; Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]; Companies to report which option is used: - Option 1: SBFD Frame structures in Alt2(XXXXU) in Annex A.7 - Option 2: SBFD Frame structures in Alt4(XXXXX) in Annex A.7. |
| UE attachment | UE selected macro TRP or indoor TRP is determined based on the RSRP from port 0, i.e., the UE in the indoor office/indoor factory can select the Macro cell as serving cell, and the UE outside the indoor office/indoor factory can select the indoor TRP as serving cell |

Table B-4: SLS assumptions specific to SBFD Deployment Case 4 (FR1 & FR2)

| Parameters | Urban Macro (FR1) | Dense Urban Macro layer (FR2-1) |
| --- | --- | --- |
| BS transmit power | - Different power levels in adjacent carriers can be simulated, and it is up to company to report the power levels. - Company to report the power levels on DL slots and SBFD slots in SBFD operation. |  |
| SBFD subband and slot configurations | Operator#1; Operator#2. baseline legacy TDD network (Baseline for comparison with SBFD Deployment Case 4); Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]. SBFD Deployment Case 4; Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]; Companies to report which option is used: - Option 1: SBFD Frame structures in Alt2(XXXXU) in Annex A.7 - Option 2: SBFD Frame structures in Alt4(XXXXX) in Annex A.7. |

Table B-5: SLS assumptions specific to dynamic/flexible TDD

| Parameters | FR1 |  |  | FR2-1 |  |
| --- | --- | --- | --- | --- | --- |
|  | Indoor office | Urban Macro | 2-layer Scenario B | Indoor office | Dense Urban Macro layer |
| BS transmit power | 24 dBm for 100MHz | - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz | Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Indoor office/indoor factory - 24 dBm for 100MHz | 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. | 40 dBm for 100MHz or 43dBm for 200MHz |

Annex <C>:
System level simulation calibration

# C.1	SLS calibration methodology

Calibration scenarios

The SLS calibration focuses on the following scenarios of SBFD Deployment Case 1:

-	FR1

-	Urban Macro

-	Indoor office

-	FR2-1

-	Dense Urban Macro layer

-	Indoor office

Calibration metrics

The following metrics are considered for SLS calibration:

-	CDF of gNB-UE coupling loss

-	CDF of Inter-gNB coupling loss

-	CDF of Inter-UE coupling loss

-	Optional: CDF of DL SINR for legacy TDD/ DL SINR in DL-only slots for SBFD

-	Optional: CDF of DL SINR in SBFD slots

-	Optional: CDF of UL SINR for legacy TDD/ UL SINR in UL-only slots for SBFD

-	Optional: CDF of UL SINR in SBFD slots

Table C.1-1: Statistic method for SLS calibration

| SLS calibration metrics | Statistic method |
| --- | --- |
| gNB-UE coupling loss | Only the coupling losses between each UE and its serving cell are collected for CDF statistic. - wA and gB are determined by selecting the best beam pair of the UE and its serving cell with the criteria of maximizing receive power of the UE. |
| Inter-gNB coupling loss | For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair - The two gNBs in each gNB pair should be from different sites. - Both wA and gB are randomly selected for calculating the coupling loss for each gNB pair. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| Inter-UE coupling loss | For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair - If the 2D distance between two UEs in a UE pair is larger than 50m, the UE pair is not considered for statistic. - For each UE, wA and gB is determined based on the best beam pair of the UE and its serving cell. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| NOTE 1: Formula (2) in Annex A.8 for CL with averaging across all the Tx/Rx ports is used for coupling loss calculation above, i.e., CLA,BwA,gB=1Sp=0S-11Uu=0U-1CLp,uA,BwA,gB=1Sp=0S-1PL∙SF∙1Uu=0U-1α'0,u,p2 NOTE 2: The beams for above cases are selected based on a defined set of beams for FR1 and FR2 in the table for calibration assumptions. |  |

# C.2	SLS calibration assumptions

Table C.2-1: SLS calibration assumptions

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- |
| Carrier frequency | 4 GHz | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  |  |  |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  | 12 TRPs per 120m x 50m x 3m |  |
| Inter-BS (2D) distance | 500m | 200m | 20m |  |
| Minimum BS-UE (2D) distance | 35m | 35m | 0m |  |
| Minimum UE-UE (2D) distance | 1m | 1m | 1m |  |
| BS antenna height | 25m | 25m | 3m |  |
| UE distribution | UE Clustering | UE Clustering | Uniform |  |
| UE number per macro/indoor TRP (per direction) (M) | 20 | 10 | 10 |  |
| UE cluster number per macro cell (X) | 2 | 1 | - |  |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 100% outdoor without car penetration loss: 3km/h | 100% indoor in houses: 3km/h |  |
| Indoor UE height (m) | 1.5m | 1.5m | 1.5m |  |
| Outdoor UE height (m) | 1.5m | 1.5m | - |  |
| Radius of cluster (R) | 25m | 20m | - |  |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m | 55m | - |  |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m | - | - |  |
| gNB-UE Channel model | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |
| BS antenna array configuration for Legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) =(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,16,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (16,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 |  | reuse Table 10 in Report ITU-R M.2412 |  |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 |
| BS receiver noise figure | 5dB | 7 dB | 5dB | 7 dB |
| UE receiver noise figure | 9 dB | 13 dB | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 | P0= -60 dBm, alpha = 0.6 |  |
| Handover margin (dB) | 3 dB | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {5*pi/8, 7*pi/8} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {pi/4, 3*pi/4} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

# D.1	RAN4 co-existence simulation scenarios

# D.2	RAN4 co-existence simulation assumption

# D.3	RAN4 co-existence simulation methodology

# D.4	RAN4 co-existence simulation results

# Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109-e | R1-2205399 |  |  |  | Skeleton | 0.0.1 |
| 2022-05 | RAN1#109-e | R1-2205692 |  |  |  | Endorsed skeleton | 0.0.2 |
| 2023-02 | RAN1#112 | R1-22xxxxx |  |  |  | Capture evaluation methodology agreements until RAN1#111 meeting | 0.1.0 |