3GPP TSG-RAN WG4 Meeting #106                                           	R4-2300831

Athens, Greece, February 27 – March 3, 2023

Source:	Samsung

Title:	initial simulation results on 8Rx UE demodulation requirements

Agenda item:	9.6.6.1

Document for:	Discussion

- Introduction

In last RAN4 #105 meeting, initial test scope and simulation assumptions for 8Rx UE demodulation requirements have agreements in WF [1]. In this contribution, we share our initial simulation results.

- Simulation Assumptions

As agreed in last RAN4 #105 meeting, this contribution focus on TDD PDSCH cases as first priority. Basic simulation assumptions used for this contribution are as below.

- Applicable test scenarios: Single CC with PDSCH mapping type A

- SCS/Bandwidth: 30 kHz/40 MHz

- UL-DL pattern: 7D1S2U with S: 6D+4G+4U

- MIMO layers: Rank 2, 4, 8

- Propagation conditions: TDLA30-10 for rank>=2 and TDLC300-100 for rank=2

- Antenna configuration: rank2 2T8R, rank4 4T8R, rank8 8T8R

- Correlation matrix: ULA Low

- DMRS symbol indexes for rank 8: (2, 3, 10, 11)

- TX EVM for QPSK/16QAM/64QAM: 6%

- Rank8 N1=4, N2=1

- MCS index table for PDSCH: Table 5.1.3.1-1 in 3GPP38.214

Other parameters specified in 3GPP 38.101-4 Table 5.2-1. Test cases for rank2, rank4 and rank8 are specified as Table 2-1 - Table 2-3.

Table 2-1 Test cases for rank2

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | TBD |

Table 2-2 Test cases for rank4

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |

Table 2-3 Test cases for rank8

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |

- Simulation results

Simulation results for cases defined in Table 2-1 - Table 2-3 are as below.

Figure 3-1 Simulation results of test cases 1-1 and 1-2

Thus the SNR point of 70% maximum throughput for cases 1-1 and 1-2 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |

Figure 3-2 Simulation results of test cases 2-1 and 2-2

Thus the SNR point of 70% maximum throughput for cases 2-1 and 2-2 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |

Figure 3-3 Simulation results of test cases 3-1, 3-2 and 3-3

Thus the SNR point of 70% maximum throughput for cases 3-1, 3-2 and 3-3 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

In summary, the SNR point of 70% maximum throughput for rank2, 4, 8 cases could be summarized as table 3-1.

Table 3-1 SNR point of 70% maximum throughput for rank2, 4, 8 cases

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

- Conclusion

According simulation results in section 3, we could get the simulation results of SNR point of 70% of max throughput as below.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

- Reference

[1] R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, 3GPP RAN4 #105 meeting