3GPP TSG-RAN WG4 Meeting #106                                           R4-2300904

Athens, Greece, February 27 – March 3, 2023

Source: 	Xiaomi

Title: 	Discussion on network B requirements for Rel-18 MUSIM gaps

Agenda Item:	9.24.2.4

Document for:	Discussion

# 1 Introduction

In RAN4#105, a WF on RRM requirements for Rel-17 MUSIM gaps was approd in [1]. The progress on network B requirements is captured below:

- Sub-topic 4-1 On network B requirements Issue 4-1-1: Whether to define network B requirements - Proposals - Option 1: Deprioritize NW B requirement at R18. (Apple xiaomi) - Option 1a: If necessary, only idle/inactive mode measurement/cell reselection requirements need to be considered. (Apple oppo) - Option 2: No measurement requirements in network B will be defined by RAN4 at R18 (vivo oppo Huawei MTK Qualcomm) - Option 3: RAN4 need to define specific UE requirements for Network B when UE is allocated MUSIM gaps (Nokia) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode when no MUSIM gap collision happens. (Ericsson) - WF - Non consensus and continue discussion Issue 4-1-2: Network B requirements if it will be defined - Proposals - P1: Framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example: (Apple) - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) (oppo Huawei) - P3: The UE requirements for Network B are the same as/re-use the existing UE requirements (Nokia) - WF - Non consensus and continue discussion

In this paper, we would like to further provide our views on the network B requirements for Rel-17 MUSIM gaps.

- Discussion

During the discussion in last RAN4 meeting, companies have different opinion on whether to define requirements for Network B. From our perspective, the measurements on Network B are conducted based on the throughout loss performance of Network A, it is reasonable to define RRM requirements for Network B. However, we think the discussion on requirements for Network A should be prioritized and the requirement on network B can be discussion after the UE behavior is clear on network A.

If RAN4 agree to define the requirements for Network B in RRC idle/inactive, we prefer to restrict to cell reselection requirement, which we think could use the existing requirement as baseline.

Proposal 1: The discussion on requirements for Network B in RRC idle/inactive could be de-prioritized.

- Conclusion

Proposal 1: The discussion on requirements for Network B in RRC idle/inactive could be de-prioritized.

- Reference

[1] R4-2220443, WF on RRM requirements for Rel-17 MUSIM gaps, vivo