3GPP TSG-RAN WG4 Meeting # 106							R4-2300913

Athens, Greece, February 27 – March 3, 2022

Agenda Item:	9.8.3.5

Source: 	Xiaomi

Title: 	on the TCI state switching delay with dual TCI

Document for:	Approval

- Introduction

During the RAN4#105 meeting, the TCI state switching delay requirement has been discussed. Couple of issues and scenarios have been discussed. We have also provided initial discussion on the TCI state switching delay requirements for all the scenarios in our previous paper. Hence to combine the agreed scenarios and the analysis of the requirements, we try to provide further discussion on the TCI state switching delay requirement.

- Discussion

The agreement of the WF [1] is captured as below:

Background on the terminology:

Single DCI:

•	One DCI for two TCI states (PDSCH single DCI)

•	One MAC CE for two TCI states (PDCCH SFN)

Multiple DCI:

•	Two DCI one for each TCI state (PDSCH multiple DCI)

•	Two MAC CE one for each TCI state (PDCCH non-SFN)

Issue 1-1-3: Definition of Dual TCI state switch

  - Dual TCI state switching refers to the case when two active TCI states with QCL type D for simultaneous reception of PDCCH/PDSCH are involved before and/or after TCI switching command.

  - In other words, following are considered as dual TCI state switching to discuss the requirements

    - Single DCI to Dual TCI

    - Dual TCI to Dual TCI

    - FFS: Dual TCI to Single TCI

  - Sub-topic 1-2: sDCI based TCI state switching requirements.

Issue 1-2-1: TCI state switch indication method for PDCCH and PDSCH

  - PDCCH non-SFN

    - Only single PDCCH is supported. So Single MAC CE should be used

  - FFS: PDCCH SFN

    - One MAC CE is used to indicate both TCI states

  - PDSCH

    - For both SFN and non-SFN, only one DCI code point is indicated using single DCI and the code point may indicate one or two TCI states

  - Sub-topic 1-4: known and unknown conditions.

Issue 1-4-1: Known and unknown states combination

- Dual TCI state switching requirements are defined for simultaneous reception. It is FFS whether we need to define any new requirements for non-simultaneous reception. The dual TCI states known, or unknown are FFS.

  - Option A: known and known

  - Option B: Known and unknown

  - Option C: unknown and unknown

Firstly, for the definition of TCI state switching, it is still FFS for dual TCI to single TCI scenario. In our view, this scenario is quite similar to single TCI to dual TCI and it should be considered as a valid scenario for dual TCI state switching.

Proposal 1: To agree the dual TCI to single TCI as a valid scenario and define corresponding requirement.

Secondly, for the PDCCH SFN, one MACE CE is sued to indicate both TCI states. In our previous discussion paper, the enhanced TCI states indication for UE-specific PDCCH MAC CE is referred and captured again below from TS 38.321 sub-clause 6.1.3.44. So it can be seen that the single MAC CE can contain two different TCI state.

Observation 1: For PDCCH SFN, enhanced TCI state indication is used and hence one MAC CE can indicate two different TCI.

Proposal 2: The TCI state indication is proposed as:

Two MAC CE one for each TCI state (PDCCH non-SFN): each MAC CE contains one TCI state

One MAC CE for two TCI states (PDCCH SFN): one enhanced MAC CE contains two TCI states

One DCI for two TCI states (PDSCH single DCI): one enhanced DCI contains one or two TCI states

Two DCI one for each TCI state (PDSCH multiple DCI): each DCI contains one TCI state

For the known and un-known state combination, we support all the three cases as: both known, both un-known, one known + one un-known. The corresponding MAC CE based requirements have been discussed in our previous paper and the summary table is captured as below:

Table 1: MAC CE based TCI state switching delay requirement

| Original TCI state | Target TCI state | Requirement |
| --- | --- | --- |
| Single TCI or Dual TCI for PDCCH SFN | Dual TCI for PDCCH SFN both known | Legacy single TCI known requirement apply |
|  | Dual TCI for PDCCH SFN one or both un-known | Legacy single TCI un-known requirement apply |
| Single TCI or Dual TCI for PDCCH non-SFN | Dual TCI for PDCCH non-SFN | Legacy single TCI requirement apply for each RX chain |
| Dual TCI for PDCCH SFN or Dual TCI for PDCCH non-SFN | Single TCI | Legacy single TCI requirement apply |

This table 1 has clearly included all the single TCI, PDCCH SFN and PDCCH non-SFN of MAC-CE based TCI state switching and corresponding requirements are defined. We can see the legacy MAC-CE based TCI state switching delay requirements apply for each case, hence the spec can be corrected correspondingly.

Proposal 3: To agree table 1 for the MAC CE based TCI state switching delay requirement.

3	Conclusions

In this paper, we give further discussion on TCI state switching delay, the observations and proposals are captured as below:

Proposal 1: To agree the dual TCI to single TCI as a valid scenario and define corresponding requirement.

Observation 1: For PDCCH SFN, enhanced TCI state indication is used and hence one MAC CE can indicate two different TCI.

Proposal 2: The TCI state indication is proposed as:

Two MAC CE one for each TCI state (PDCCH non-SFN): each MAC CE contains one TCI state

One MAC CE for two TCI states (PDCCH SFN): one enhanced MAC CE contains two TCI states

One DCI for two TCI states (PDSCH single DCI): one enhanced DCI contains one or two TCI states

Two DCI one for each TCI state (PDSCH multiple DCI): each DCI contains one TCI state

Proposal 3: To agree table 1 for the MAC CE based TCI state switching delay requirement.

4 References

[1] R4-2220402		WF on NR FR2 multi-Rx chain DL reception – Part 3, Ericsson