3GPP TSG-RAN WG4 Meeting #106	R4-2300995

# Athens, Greece, February 27 – March 3, 2023

Title:	Discussion and reply LS on prioritization of test cases

Source:	Samsung

Agenda item:	9.15.1

Document for:	Approval

1.	Introduction

ETSI sends LS to RAN4 regarding 3GPP work priority on BHH requirements for wide UE (72~92mm) in [1]:

- Overall description Dear all, ETSI TC MSG/ERM TFES is responsible to set TRP and TRS limits for LTE and NR in European markets as captured by EN 301 908-13 and EN 301 908-25, respectively. Currently EN 301-908-13 contains BHH (Beside Head and Hand) LTE TRP and TRS requirements for devices with width between 56 mm and 72 mm. However, BHH LTE and NR requirements for devices wider than 72 mm and narrower than 92 mm are still absent. BHH mode is important for European markets, especially for emergency calls, which are legal requirements for operators. Therefore, ETSI TC MSG/ERM TFES would like to know the schedule of 3GPP RAN WG4 for BHH NR TRP and TRS requirements, including VoNR (Voice over NR), for devices wider than 72 mm and narrower than 92 mm and would suggest to prioritize them if those requirements cannot be completed by the end of 2023. TC MSG/ERM TFES looks forward to further cooperation with 3GPP RAN WG4 on this matter. Best regards, Dominique Everaere ETSI MSG TFES Chair - Actions ETSI TC MSG/TFES respectfully ask 3GPP TSG RAN to consider the above information, and to provide status of related standardization work and prioritize them if necessary.

Besides, in WID [2] and work plan [3], prioritization of test cases are also mentioned.

In this contribution, our views on prioritization of test cases are presented especially about prioritizing talk mode (BHH) requirements and deprioritizing 2Tx requirements. A draft reply LS is also provided in Annex.

2. 	Discussion

2.1	test setup

Rel-18 TRP TRS work item involves so many test cases including both SA and NSA, both Size 1 UE and Size 2 UE, both 1Tx and 2Tx, different UE type, different power class, more test bands, and so on. It is obviously necessary to prioritize some important test cases which are urgently needed. According to work plan, discussion on further prioritization of test cases are scheduled to start since this meeting.

The ETSI LS shows the urgent need of talk mode (BHH) requirements for Size 1 UE (72~92mm width). According to the WID, Size 1 UE is already prioritized and further prioritization between browsing mode and talk mode is not precluded.

- Handheld UE type is the first priority - Consider UE size 1 and size 2 - UE size 1 is the first priority - Consider both browsing mode and talk mode - Further prioritization not precluded - Consider Power class 2 and 1.5 for 2Tx requirements, Power class 3 and 2 for 1Tx requirements - Requirement applicability of TxD and single-layer UL MIMO of the same UE shall be specified - Consider further prioritization of test cases

Regarding the prioritization between browsing mode and talk mode, in our view it is important to guarantee the minimum requirement for the network in the worst case which is talk mode. With only browsing mode requirements, it is not sure how much performance drop would occur under talk mode compared with browsing mode.

Observation 1:	talk mode is the worst case and it is important to guarantee the minimum requirement for the network

On one hand talk mode is the worst case for the network, on the other hand user usually requires higher communication robustness in real-time in talk mode than in browsing mode. So the browsing mode requirements are better to have, but talk mode requirements are the most critical.

Observation 2:	talk mode is the most critical case in which users require higher communication robustness in real-time

Consequently, no matter UE width is narrow or wide, talk mode requirement should be prioritized than browsing mode.

Proposal 1:	Talk mode (BHH) requirement work are prioritized than browsing mode in Rel-18 TRP TRS WI.

One more aspect is about 2Tx requirements. It can be seen that test method discussion for 2Tx OTA is still controversial and it seems more time is needed before test method will be stable and test system will be matured.

Moreover, TRP TRS requirements will be derived based on measurement campaign which requires large number of test models. So far the 2Tx models are not sufficient enough for measurement campaign.

Base on that, we think it should be reasonable to prioritize 1Tx requirement work than 2Tx in Rel-18.

Proposal 2:	1Tx requirement work are prioritized than 2Tx in Rel-18 TRP TRS WI.

As ETSI TC MSG/ERM TFES is waiting for RAN4 response on BHH prioritization, a draft reply LS is provided in Annex for the convenience of discussion.

3. 	Conclusion

Observation 1:	talk mode is the worst case and it is important to guarantee the minimum requirement for the network

Observation 2:	talk mode is the most critical case in which users require higher communication robustness in real-time

Proposal 1:	Talk mode (BHH) requirement work are prioritized than browsing mode in Rel-18 TRP TRS WI.

Proposal 2:	1Tx requirement work are prioritized than 2Tx in Rel-18 TRP TRS WI.

A draft reply LS is provided in Annex.

4. 	References

- TEFS(23)074029 (R4-2300042)    “LS to 3GPP RAN WG4 on NR TRP and TRS requirements”, ETSI TC MSG/TFES

- R4-223112    “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)”, vivo

- R4-2217455    “Workplan of Rel-18 TRP TRS WI”, vivo

5. 	Annex: reply LS

Title:	Reply LS to ETSI on NR TRP and TRS requirements

Response to:	   TEFS(23)074029 (R4-2300042)

Release:		Rel-18

Work Item:		NR_FR1_TRP_TRS_Enh

Source:		3GPP RAN4

To:		ETSI TC MSG/ERM TFES

Cc:

Contact person:		Bozhi Li, bozhi (dot) li (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

1. Overall Description:

3GPP RAN4 thanks ETSI TC MSG/ERM TFES for the LS TEFS(23)074029 (R4-2300042) on NR TRP and TRS requirements.

Prioritization for devices wider than 72 mm and narrower than 92 mm is already included in the Rel-18 FR1 TRP TRS WID. Regarding the prioritization between browsing mode and talk mode, after discussion RAN4 achieved the consensus that talk mode is the most critical scenario and agreed to prioritize BHH requirements in Rel-18 FR1 TRP TRS. Regarding requirements for VoNR (Voice over NR), it is not in scope of Rel-18 FR1 TRP TRS yet and the scope change is subject to TSG RAN plenary.

Regarding the planned schedule, lab alignment is expected to start since May 2023, measurement campaign is expected to start since November 2023, and BHH requirements for wider than 72 mm and narrower than 92 mm are expected to be concluded in May 2024.

2. Actions:

To RAN4:

ACTION: 	RAN4 respectfully requests ETSI TC MSG/ERM TFES to take the above responses into account in the future work.

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #106-bis-e	April 17 – 26, 2023	eMeeting

TSG RAN WG4 Meeting #107	May 22 – 26, 2023	Incheon, Korea