3GPP TSG-RAN WG4 Meeting #106	R4-2301085

Athens, Greece, 27th February – 3rd March 2023

Source:	Ericsson, BT plc

Title:	TP for TR 36.718-02-01: CA_3-20-67

Agenda item:			10.1.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.718-02-01 to include CA_3A-20A-67A and CA_3C-20A-67A.

This TP is depending on approval for fallback in “TP for 36.718-02-01 to include CA_3-67” submitted in agenda item 10.1.2.2

# 2. Text Proposal

# ---Start of changes---

### 5.4.x	CA_3-20-67

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 67 |  | N/A |  | 738 MHz | – | 758 MHz | FDD2 |
| NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-20A-67A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 67 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-20A-67A | CA_3C CA_3A-20A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 67 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 5.4.x.2	Co-existence studies

Co-existence studies show that UL CA_3-20 does not affect DL 67

#### 5.4.x.3	∆TIB and ∆RIB values

The following ∆TIB and ∆RIB values are drawn from CA_3-20-28 in TS 36101:

Table 5.4.x.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-20-67 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | 67 | N/A |

Table 5.4.x.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-20-67 | 3 | 0 |
|  | 20 | 0.1 |
|  | 67 | 0.1 |

#### 5.4.x.4	REFSENS Requirements

Based on the co-existence studies there is no need to define additional REFSENS requirements,

---End of changes---

# Reference

[1] 		R4-2219906, Revised WID Rel-18 LTE-A CA for x(x<=6) DL y(y<=2) UL, Huawei

[2]		R4-2219908, TR 36.718-02-01 LTE-A CA for x(x=123456) DL y(y=12) UL, Huawei