3GPP TSG-RAN4 Meeting #106	R4-2301171
Athens, Greece, 27 Feb – 3 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0834 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38101-3 CR on clarification of UE coexistence frequency range (R17 CAT-A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO; Anritsu; Keysight; Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The applicable frequency range of UE coexitence requirement is unclear especially when the frequency range is overlapped with the SEM range. And it is found some test systems test the UE coexistence requirements within the range of FOOB while some other test systems test the range out of FOOB. This needs to be aligned in the industry and also in the spec desciption, otherwise, UE may pass the testing in some TE but fail in others. - Besides, the NOTE 3 in table 6.5B.3.1.2-1 has referred the FOOB range of LTE spec and NR spec separately, however, it should refer to the dedicated SEM range for intra-band contiguous EN-DC defined in the clause 6.5B.2.1 of 38.101-3. This need to be corrected. Similar changes are needed for NOTE3 in table 6.5B.3.2.2-1 for intra-band non-contiguous EN-DC and NOTE2 in Table 6.5E.3.2.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the applicable frequency range of UE coexistence requirements. - Corrected the NOTE3 in table 6.5B.3.1.2-1 about the UE coexistence frequency range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The misalignment of specification understanding and the implementation of test system and UE implementation will still exists. Incorrect UE co-existence frequency range will still be there. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3; 6.5E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.5B.3	Spurious emissions for DC

6.5B.3.1	Intra-band contiguous EN-DC

6.5B.3.1.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.1apply.

6.5B.3.1.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.1.2-1 apply on each component carrier with all component carriers are active. Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.1.1 apply.

Table 6.5B.3.1.2-1: Requirements for intra-band contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_(n)71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | -- | FDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_(n)41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that overlap with the frequency ranges where out-of-band emissions requirements are defined in clause 6.5B.2.1.1. NOTE4: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

6.5B.3.2	Intra-band non-contiguous EN-DC

6.5B.3.2.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.2 apply. If for some frequency an individual CC spurious emission requirement overlaps with the general spectrum emission mask or the bandwidth of another CC then it does not apply.

6.5B.3.2.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.2.2-1 apply with all component carriers are active. Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.2.1 apply.

Table 6.5B.3.2.2-1: Requirements for intra-band non-contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| DC_41_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 and n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that overlap with the frequency ranges where out-of-band emissions requirements are defined in clause 6.5B.2.2.1. NOTE 4: Void. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

6.5B.3.3	Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both both uplink carriers active. Limits on configured maximum output power for the uplink according to clause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.1-1: (Void)

6.5B.3.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active. Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are more than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 of 3GPP TS 36.101 [4] or in Table 6.5.3.1-1 of 3GPP TS 38.101-1 [2] from the edge of the channel bandwidth.

NOTE:	For inter-band EN-DC with uplink assigned to one LTE band and one NR band the requirements in Table 6.5B.3.3.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur;

<<< Unchanged sections omitted >>>

6.5E.3	Spurious emissions

6.5E.3.1	Intra-band V2X

6.5E.3.1.1	General spurious emissions

For intra-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5C.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

6.5E.3.1.2	Spurious emission band UE co-existence

For intra-band V2X, the spurious emissions band UE co-existence requirements specified in clause 6.6.3.2 of TS 36.101 [4] and clause 6.5C.3.2 of TS 38.101-1 [2] apply for each frequency range respectively.

6.5E.3.2	 Inter-band V2X con-current operation

6.5E.3.2.1	General spurious emissions

For inter-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5E.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

6.5E.3.2.2	Spurious emission band UE co-existence

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.2.2-1 apply for the corresponding inter-band con-current operation with transmission assigned to both E-UTRA uplink in licensed band and sidelink in NR Band n47. Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are beyond any frequencies for which the out-of-band emissions requirements in clause 6.5E.2.2 apply.

Table 6.5E.3.2.2-1: Requirements for inter-band con-current V2X operation

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_1A_n47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_n1A_47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_5A_n47A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, 34, 40, 41, 42, 45, 65, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_n5A_47A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, 34, 40, 42, 45, 65, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_8A_n47A | E-UTRA Band 1, 3, 7, 8, 22, 28, 34, 39, 40, 41, 42, 45, 65, 68, 72, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_n8A_47A | E-UTRA Band 1, 3, 7, 8, 28, 34, 39, 40, 45, 65, 68, 72, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| V2X_3_n47 | E-UTRA Band 1, 5, 7, 8, 26, 28, 34, 39, 40, 41, 44, 45, 65, 68, 72, 73 NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| V2X_20_n38 | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| V2X_n39_47 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_39_n47 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n40_47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 42, 44, 45, 65, 68, 72 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_40_n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 27, 28, 34, 39, 41, 42, 44, 45, 65, 68, 72, 73 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n71_47 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n78_47 | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n79_47 | E-UTRA Band 1, 3, 5, 8, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that overlap with the frequency ranges where out-of-band emissions requirements are defined in clause 6.5E.2.2. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

<<< END OF CHANGES >>>