# 3GPP TSG-RAN WG4 Meeting # 106                               R4-2301198

# Athens, Greece, February 27 – March 3, 2022

# Title: 	Discussion on UE demodulation performance for ATG

# Source: 	ZTE Corporation

# Agenda item:	9.13.5.2

# Document for:	Approval

# 1 Introduction

In RAN#95e meeting, the work item[1] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions has been reached for ATG coexistence assumption. We share some views on ATG demodulation requirement.

# 2	Discussion

## 2.1 Scenario

Figure 2-1 shows the scenario of ATG of TN deployment and ATG deployment.

Figure 2-1(a). Scenario of ATG of TN deployment

Figure 2-1(b). Scenario of ATG of ATG deployment

For the perspective of demodulation, it is necessary to consider the both scenarios.

The difference between the two scenario is listed in table 2-1.

Table 2-1. Differences for different scenarios

| index | Parameter | TN deployment | ATG deployment |
| --- | --- | --- | --- |
| 1 | Propagation delay | Continuity | Discontinuity |
| 2 | Doppler shift | Jumping from negative value to positive value or vice versa | Not jumping from negative value to positive value or vice versa |
| 3 | Switching point | In the middle of the BS | Near to one of the BS |

With regard to propagation delay, we can find that the propagation delay is from big to small and then from small to big when the aircraft flies into the coverage area of a BS then it leaves the coverage area of one BS and enters the coverage area of another BS if for TN deployment. While for ATG deployment, There will be a large timing gap when the aircraft switch from one BS to another.

With regard to Doppler shift, we can find that the Doppler shift jumps from negative value to positive value for TN deployment. While for ATG deployment, there is no Doppler shift jumping from negative value to positive value or vice versa which can be found in figure 2-2. The starting TTI in figure 2-2 corresponds the position just above one BS and the flying direction in figure 2-2 is from left to right as depicted in figure 2-1.

Figure 2-2. Doppler shift experienced by an airplane for TN(left) and ATG(right) deployment

With regard to switching point, it is a natural choice for the switching point to be in the middle of two BS for TN deployment. For ATG deployment, the switch point needs to be decided with some factors such as similar Rx power, change rate of Rx power and change rate of Doppler shift etc,. And the switching point concluded in coexistence study[2] can be considered.

Proposal 1. To take the switching point concluded in coexistence study as a starting point for demodulation evaluation for demodulation requirement:

Proposal 2. To take the following factors into consideration for demodulation evaluation for both TN deployment and ATG deployment:

- Propagation delay

- Doppler shift

## 2.2 Channel model

The ATG base station is designed to serve the air line coverage, so the LOS propagation condition can be expected for ATG CPE. Two alternatives of channel model can be considered for demodulation requirement evaluation of ATG. One is static channel with Doppler shift trajectories corresponding to different scenarios. The other is using NTN channel  model or modified NTN channel model for ATG as NTN channel model is also used for communication between the the devices on ground and in the air.

Proposal 3. To consider static channel or NTN channel model or modified NTN channel model for ATG demodulation requirement evaluation.

## 2.3 Simulation parameters

Table 2-2 lists the parameter assumption for link level simulation and figure 2-3 shows the initial simulation results for n1 band with 40MHz BW under AWGN channel model with Doppler shift corresponding to ATG scenario.

Table 2-2 Simulation parameters

| Duplex mode |  | FDD TDD(2D4S6U+38D) |
| --- | --- | --- |
| SCS |  | 15kHz for n1 30kHz for n78, 79 |
| BW |  | 100MHz for n78,n79 40MHz for n1 |
| speed |  | 1200kmph |
| Antenna config |  | 2*2 |
| PDSCH | Mapping type | Type A |
|  | k0 | 0 |
|  | Start symbol | 2 |
|  | Length | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundle type | static |
|  | PRB bundle size | 2 |
|  | Resource allocation type | Type0 |
|  | VRB-to-PRB mapping type | Non-interleaved |
| PDSCH-DMRS | DMRS-type | Type 1 |
|  | # additional DMRS | DL slot: 2 S slot: 0 |
|  | Max #OFDM symbol for front loaded DMRS | 1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
| SSB | Period | 20ms |
| CSI-RS for tracking | Period | 20ms |
|  | k0 | 0 |
|  | OFDM symbol | l = 6, 10 |
| Channel |  | AWGN or static channel NTN-TDL-D/E or modified NTN channel |
| # MIMO layer |  | 1 |
| MCS Table 5.1.3.1-1: MCS index table 1 for PDSCH |  | 13 20 |
| Number of Max Re-transmission |  | 4 |
| Test metric |  | SNR @ %70 of maximum Throughput |

Figure 2-3. Throughput under AWGN channel model with Doppler shift experienced in TN deployment @2.1GHz

Another issue for demodulation evaluation is the TTI number to simulate, take 200km ISD as an example, the airplane needs to fly for 600s which corresponds to 6*10^5 TTIs for 15kHz SCS.

The TTI number to simulate is quite large. Maybe a simplified method needs to be considered.

Observation 1. If the whole coverage area of a BS is simulated, the TTI number will be very large.

In order to evaluate the Doppler shift tracking ability of a receiver, we can consider the following methods:

Proposal 4. To consider the following alternatives to evaluate the Doppler shift tracking ability of a receiver:

- Alternative 1: Evaluate a period during which the airplane experiences a large Doppler shift.

- Alternative 2: Evaluate a period during which the airplane experiences large Doppler changes.

- Alternative 3: The combination of the above two situations.

# 3 Conclusion

In this contribution, we give some initial discussions on ATG DL demodulation and initial simulation results for reference.

The observation and proposals are given:

Observation 1. If the whole coverage area of a BS is simulated, the TTI number will be very large.

Proposal 1. To take the switching point concluded in coexistence study as a starting point for demodulation evaluation for demodulation requirement:

Proposal 2. To take the following factors into consideration for demodulation evaluation for both TN deployment and ATG deployment:

- Propagation delay

- Doppler shift

Proposal 3. To consider static channel or NTN channel model or modified NTN channel model for ATG demodulation requirement evaluation.

Proposal 4. To consider the following alternatives to evaluate the Doppler shift tracking ability of a receiver:

- Alternative 1: Evaluate a period during which the airplane experiences a large Doppler shift.

- Alternative 2: Evaluate a period during which the airplane experiences large Doppler changes.

- Alternative 3: The combination of the above two situations.

# 4	Reference

[1]  RP-221369 Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2220542 WF on ATG co-existence evaluation, CMCC