# 3GPP TSG-RAN WG4 Meeting # 106	R4-2301263

# Athens, Greece, Feb 27 – Mar 03, 2023

Title:	Reply LS on clarification for ue-PowerClassPerBandPerBC-r17

Response to:	R2-2211023

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Wubin, Zhou

E-mail Address:  zhou.wubin@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS to inquire the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing different power class parameters.

Regarding RAN2’s request to RAN4 for the responses to the following questions:

- Whether R4 16-8 is applicable to only inter-band CA?

- What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610)?

RAN4 would like to respectfully provide the following responses for RAN2 consideration. According to the discussion during RAN4#106 meeting, the responses are:

Response to 1): It is applicable to only NR inter-band UL CA containing only single CC or intra-band UL CA for each individual band within an NR CA band combination.

Response to 2): powerClassNRPart-r16 applies for MR-DC, while ue-PowerClassPerBandPerBC-r17 is for only NR inter-band UL CA, there is no interaction between ue-PowerClassPerBandPerBC-r17 and powerClassNRPart-r16.

If ue-PowerClassPerBandPerBC-r17 is absent, the interaction between ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700 and powerClass/powerClass-v1610 have already been described in RAN2 spec.

If ue-PowerClassPerBandPerBC-r17 is present, and if the power class of this band combination indicated by powerClass/powerClass-v1610 is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassPerBandPerBC-r17), the maximum UE power available in each band shall be determined by ue-PowerClassPerBandPerBC-r17.

# 2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

# 3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106-bis-e	17 - 26 April 2023             E-meeting

TSG-RAN WG4 Meeting #107	22 - 26 May 2023             Incheon, KR