3GPP TSG-RAN WG4 Meeting #106									    		                   R4-2301364

27th Feb – 3rd March, 2023, Athens, Greece

Agenda item:			9.25.2

Source:	Samsung

Title:	Simulation assumptions for above 10GHz NTN co-existence study

Document for:	Discussion

- Introduction

According to the Rel-18 NR NTN enhancement WID, RAN4 has been mandated to study and identify NTN example band in above 10 GHz.

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminals and scenarios exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

For co-existence studies, following considerations should be taken into account:

- Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow.

- For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands.

In the RAN4 #104-bis-e and 105 meeting, agreements on some assumptions have been achieved. And the remaining issues have been discussed in this paper.

- Discussion

Network layout model

TN Urban area should be considered in the coexistence study, cause the NTN UE deployment is not certain. The NTN UE also have the possiblity move into the TN Urban area, or the user of NTN UE opened in the Urban area. Those two scenario can generate interference to the TN base stations.  When a large area that covered by a NTN satellite beam is considered in the study, density can be a better value to reflect the deployment of the base station. Therefore:

Proposal 1: Consider Urban TN with density in the NTN satellite beam covered area.

System parameters

Proposal 2: propose Table 1 as the Set-1 NTN satellite Noise figure assumptions.

Table 1 Set-1 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | 5.9 | 0.9 | 0.9 |

Proposal 3: propose Table 2 as the parameters for the NTN Fixed VSAT UE.

Table 2 NTN Fixed VSAT UE parameter

| Characteristics |  |
| --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Efficiency |  |
| Rx Antenna gain | 39.7 dBi |
| Noise figure | 1.2 dB |
| Rx Feeder loss | -0.5dB |
| Antenna temperature | 150 K |
| Sky temperature | N/A |
| Ground temperature | N/A |
| G/T figure of merit | 15.35 (dB/K) (Note 2) |
| Rx Feeder loss |  |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |
| Output loss |  |
| EIRP | 45.2 dBW |
| UE height | FFS |
|  | NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna NOTE 2: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |

Proposal 4: Propose Table 3 as the antenna parameters for the NTN movable VSAT UE.

Table 3 Antenna parameters for the NTN movable VSAT UE

| Characteristics | ESIM (TR 38.821) | ESIM (ITU) |
| --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (64,32,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol |  |
| Element gain including loss | 5.5dBi |  |
| Rx Antenna gain | 3dBi per elemant | 45 dBi |
| Antenna temperature | 150 K |  |
| Noise figure | 11dB |  |
| Tx transmit power | 10*log10(64*32)+7 dBm) | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | 3 dBi per element | 45 dBi |

Proposal 5: Propose Table 4 as the Horizontal boresight and Vertical tilt assumptions of NTN UEs

Table 4 Horizontal boresight and Vertical tilt assumptions of NTN UEs

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | Following Serving satellite or random between (0-360) | Following Serving satellite or random between (0-360) |

Proposal 6: TN parameters of Dense Urban Micro base station need to be considered.

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Proposal 7: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

TN BS and UE antenna and beam forming pattern modelling

Proposal 8: Base station and UE antenna pattern modelling are proposed according to the TR 38.803.

- Conclusion

Proposal 1: Consider Urban TN with density in the NTN satellite beam covered area.

Proposal 2: propose Table 1 as the Set-1 NTN satellite Noise figure assumptions.

Proposal 3: propose Table 2 as the parameters for the NTN Fixed VSAT UE.

Proposal 4: Propose Table 3 as the antenna parameters for the NTN movable VSAT UE.

Proposal 5: Propose Table 4 as the Horizontal boresight and Vertical tilt assumptions of NTN UEs

Proposal 6: TN parameters of Dense Urban Micro base station need to be considered.

Proposal 7: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

Proposal 8: Base station and UE antenna pattern modelling are proposed according to the TR 38.803.

- Reference

[1] TR 38.803 Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

[2] TR 38.863 Non-terrestrial networks (NTN) related RF and co-existence aspects

[3] 3GPP, R4-2220241 WF on [313] NR_NTN_enh_Part2