3GPP TSG-RAN WG4 Meeting # 106 	R4-2301421

Athens, Greece, February 27 – March 3, 2022

Source: 	Huawei, HiSilicon

Title: 	TP on RF front end

Agenda Item:	9.4.2

Document for:	Approval

1 Introduction

In the meeting # 104-bis-e the WF [11] was approved with some agreed observations on WB RF front feasibility.

- RAN4 reached consensus on below observations:

  - Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

  - For RF front-end, commercially available TRX chips cover 24-29.5GHz. 27-41GHz RX is implemented. A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz.

The background from for (1) comes from contribution [2] and for (2) from [10].

This TP captures those agreements and attempts to capture some of the background technical information, as presented in papers in the last meeting, in the TR

2 TP for TR 38.877

5.2	Wideband RF architectures

5.2.1	RF Front end

5.2.1.1	Beam former and PA

Feasibility for RF front ends can be considered by looking at existing component capabilities which could be used for multi-band FR2 products now but also by studying literature for the direction of future capabilities.

There are a number of components available today which offer sufficient band widths to cover at least some of the FR2 multi-band options. In [x] a wide band beam former and PA was presented which covered 24 to 29GHz (bands n257/n258/n261).

Table 5.2.1.1-1: beamformer IC in 24-29.5GHz

| Parameter | Beamformer |
| --- | --- |
| Frequency Band | 24-29.5GHz |
| Tx Op1dB/OIP3 | 21dBm/ 25dBm |
| Tx Pdiss/Ch @ 12dBm Pout | 300mW |
| Pout @3% EVM w/ 64QAM | 12dBm |
| Rx Single Channel NF | 4dB |
| Rx Pdiss | 130mW |
| Instantaneous Bandwidth | 1600MHz |

Figure 5.2.1.1-1: PA bias optimized for P1dB for 24GHz (left), and 28GHz (right)

It is considered feasible that multi-band products covering this frequency range and associated bands can be achieved.

Looking further ahead in [3] a 24-42 GHZ wideband PA was presented it exhibits flat P1dB of 17.8 to 19.6dBm, within 1.6dB from Psat, and flat PAEP1dB of 36.6 to 44.3% over 24 to 40GHz, verifying the truly wideband large-signal matching

Figure 5.2.1.1-2 in [3] Top schematic of the prototype PA and small-signal S-parameters and large-signal CW

In [4] a 26-39GHz PA. The hybrid N/PMOS allows the PA deep Class-AB biasing and device cascade, substantially increasing PA Pout and efficiency.

Paper [5] presented a 24-44 GHz low-noise amplifier (LNA) and compared to a narrow-band 28 GHz LNA. The UWB LNA achieves a 4.2-5.5 dB NF and 18-20 dB gain across the full range, and exhibited about 0.5 dB increase in NF compared to the narrow-band 28 GHz LNA.

Paper [6] presented a LNA covers 23-29 GHz and 37-43 GHz with a NF of 3.1-3.7 dB at 24-43 GHz, an in-band IIP3 of -13.2 to -19 dBm at 20-40 GHz. The LNA is co-designed with embedded high-pass filters so as to result in a very sharp rejection at < 16 GHz, exhibiting an elliptic filter response.

We summarize these PAs/LNAs across different frequency groups in the following Table Table 5.2.1.1-2. It can be seen that some recent study shows PA cover 26-39GHz, 24-42GHz with average output power around 10dBm,PAEP1dB larger than 30%.

Table 5.2.1.1-2: PAs/LNAs performance from literatures

| Frequency | [3] 24 -42GHz PA | [4] 26 -39GHz PA | [5]24-44 GHz LNA | [6]24-43 GHz LNA |
| --- | --- | --- | --- | --- |
| Authors and year of publication | Fei Wang,2020 | J Park,2022 | Vikas Chauhan, 2018 | Li Gao, 2019 |
| Technology | 45nm SOI CMOS | 45nm SOI CMOS | 45nm SOI CMOS | 22 nm FDSOI |
| Gain | 20.5 | 18.9 | 20 | 23/18.2(peak) |
| S21BW-3dB（GHz） | 25.8-43.4 | 25.3-42.0 | 24-47.5 | / |
| P1dBBW-1dB（GHz） | 22.0-37.0 | 25.0-37.0 | / | / |
| PAEp1dB BW-1dB（GHz） | 24.0-41.2 | 29.9-33.6 | / | / |
| P1dB（dBm） | 17.8-19.6 | 16.3-18.4 | -19- -16 | / |
| Pavg（dBm） | 8.4-11.3 | 10.8-12.3 | / | / |
| PAEP1dB（%） | 36.6-44.3 | 29.9-34.9 | / | / |
| NF(dB) | / | / | 4.7~5.5 | 3.1-3.7 3.4-4.3 |

These papers indicate that devices covering greater frequency ranges from 26 to 40GHz or more are at least feasible in the research environment and may become available in the longer term.

5.2.1.2	Receiver front end

In [7] a 27-41GHz RX was designed and a proof-of-concept mm-Wave four-input–four-output MIMO RX array is implemented in a 45-nm CMOS SOI process with a total chip size of 3.6 mm×6.5 mm. Figure 4 is System architecture and Table 3 summarized performance。

Figure 4 in [7] (a) System architecture of the 27–41-GHz N-input–N-output MIMO RX with scalable cascadable array-based high-order ASFs for instinctual full-FoV signal/blocker management. (b) Scaling to 1 ×N MIMO arrays by assembling multiple unit chips along the zero-phase symmetric reference plane.

In [8] a 24.25-to-71GHz phased-array receiver is introduced, which covers the whole FR2 frequency band 3GPP have defined. A harmonic-selection technique is proposed to extend the operating bandwidth with low power consumption.

The LNA can be configured into operating Mode 1 covering 24 to 44GHz and operating Mode 2 covering 44 to 71GHz.

Figure 5 in [8] Block diagram of proposed multi-band phased-array receiver

Paper [9] presented a 22–44 GHz 2×2 phased-array receive beamformer. The RX channel includes a LNA, a 5-bit vector modulator (VM) phase shifter, an attenuator, and a variable gain amplifier (VGA). The phased-array channel results in a peak gain of 26.3 dB and a 3-dB bandwidth of 20.5–44 GHz. The measured NF is 3–3.6 dB at 22–44 GHz with an IP1dB of −27.5 to −24.5 dBm.

Figure 1 in [9] : Block diagram of a wideband 2 × 2 beamformer chip

For phase-array receive beamformer, we summarize these good performance receivers across different frequency groups in the following Table 5.2.1.1-3. They achieved low NF in a wide bandwidth.

Table 5.2.1.1-3: Phased-array receiver chips performance

| Frequency | [9]22–44 GHz | [7]27-41GHz | [8]24-71GHz |
| --- | --- | --- | --- |
| Authors and year of publication | Li Gao,2020 | Min-Yu Huang,2019 | Jian Pang,2022 |
| Tech. | 45nm CMOS SOI | 45nm CMOS SOI | 65nm CMOS Bulk |
| BW(GHz) | 22-44 | 27-41 | 24.25-71 |
| Gain(dB) | 26.2 | 36/element | / |
| NF(dB) | 3-3.6 | 4.3-6.3 | 3.6-8.0 (at 24.25-35GHz) 4.0-7.6(at 35-44GHz) |
| Gain Tuning(dB) | 16 | 15 | / |
| Phase Shift Res | 11.25 | / | / |
| Phase/Gain RMS Error(º /dB) | 6/1.9 | / | / |
| IP1dB(dBm) | -25.4 | -34/-27.3 | -17.6 (at 28GHz) -20.9(at 39GHz) |
| IIP3(dBm) | -18 | / | / |
| Pdc(mW) | 112 | / | / |

5.2.2	Summary

It has been shown that:

  - Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

  - For RF front-end, TRX chips covering 24-29.5GHz. 27-41GHz RX are shown. A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz.

Multi-band FR2 RF front end is hence feasible today for certain frequency ranges and band numbers (n257/n258/n261) and it seems probable that wider frequency range products will be feasible in the future.

References

[1] RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon

[2] R4-2215413, General consideration on mmWave multi-band BS, CATT

[3]An Instantaneously Broadband Ultra-Compact Highly Linear PA with Compensated Distributed-Balun Output Network Achieving >17.8dBm P1dB and >36.6% PAEP1dB over 24 to 40GHz and Continuously Supporting 64-/256-QAM 5G NR Signals over 24 to 42GHz, Fei Wang, Hua Wang, ISSCC 2020 / session 24 / RF & mm-Wave power amplifiers / 24.6, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=9063157

[4]A 26-to-39GHz Broadband Ultra-Compact High-Linearity Switchless Hybrid N/PMOS Bi-Directional PA/LNA Front-End for Multi-Band 5G Large-Scaled MIMO System，ISSCC 2022 / Session 19 / power amplifiers and building blocks / 19.4, Jeongsoo Park1,

[5]A 24–44 GHz UWB LNA for 5G Cellular Frequency Bands, Vikas Chauhan and Brian Floyd,2018 11th Global Symposium on Millimeter Waves (GSMM)

[6]A 24-43 GHz LNA with 3.1-3.7 dB Noise Figure and Embedded 3-Pole Elliptic High-Pass Response for 5G Applications in 22 nm FDSOI, Li Gao, 2019 IEEE Radio Frequency Integrated Circuits Symposium

[7] A Mm-Wave Wideband MIMO RX With Instinctual Array-Based Blocker/Signal Management for Ultralow-Latency Communication, Min-Yu Huang, et al. , IEEE Journal Of Solid-State Circuits, Vol. 54, No. 12, December 2019

[8] A Power-Efficient 24-to-71GHz CMOS Phased-Array Receiver  Utilizing Harmonic-Selection Technique Supporting 36dB  Inter-Band Blocker Rejection for 5G NR, Jian Pang et al., ISSCC 2022 / SESSION 27 / mm-Wave & Sub-6GHz receivers and transceivers for 5g radios / 27.2, https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=9731619

[9]A 22–44-GHz Phased-Array Receive Beamformer in 45-nm CMOS SOI for 5G Applications With 3–3.6-dB NF, Li Gao et al., IEEE Transactions on Microwave Theory and Techniques, Vol. 68, No. 11, November 2020

[10] R4-2216242,	Discussion of mmWave multi-band BS feasibility,	Huawei, HiSilicon

[11] R4-2217451, WF on feasibility of FR2-1 multi-band BS, Huawei, HiSilicon