3GPP TSG-RAN4  Meeting #106	R4-2301466

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Source:				Ericsson

Title:					NTN enhancements – Simulation assumptions for NTN FR2 co-existence study

Agenda item:		9.25.2

Document for:		Approval

# Introduction

This document discusses the remaining open issues related to the coexistence simulation assumptions for the NTN enhancements frequency band in the scope of the NTN enhancements WI ([1]), based on the Way Forward [3] agreed in last RAN4#105 meeting.

# Discussion

## Coexistence scenarios

### DL coexistence

RAN4 reached to the following agreement:

The considered frequency range to be discussed in the scope of this WI is defined as:

- Space-to-Earth (DL): 17.7 – 20.2 GHz

- Earth-to-Space (UL): 27.5 – 30.0 GHz

For the coexistence studies, RAN4 only needs a reference frequency to determine the propagation conditions, there is no need to define an exact frequency band. Here, as the distance between UL and DL is large, 2 reference frequencies would be needed to get more accurate simulation results. To simplify, as suggested in [6], we would then propose to use the following reference frequencies for the coexistence studies: 20 GHz when considering NTN DL and 30GHz when considering NTN UL.

Proposal1: Study NTN-TN coexistence by assuming a reference frequency of 20GHz for NTN DL cases and 30 GHz NTN UL cases.

Also, RAN4 should assume ACLR/ACS values for BS/UE TN. Such values are specified for BS/UE operating in the 30 GHz frequency area but, as there is no TN band in the 20 GHz, RAN4 would still need to assume values for that that frequency range.

The following Table 1 shows the different ACLR/ACS BS/UE values currently specified in the mentioned frequency range.

Table 1: Agreed ACLR/ACS BS/UE and ACIR values at reference frequencies

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| Below 5 GHz | 45 | 46 | 30 / 43 | 33 |  |  |
| 6 GHz (n104) | 38 | 42 | 30 | 33 | 31.8 | 29.7 |
| 10 GHz (TR 38.921) | 37 | 40 | 24 | 31 | 30.0 | 23.9 |
| 30 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |
| 45 GHz | 26 | 23 | 16 | 22 | 20.5 | 15.2 |

Based on these inputs, we would make the following proposal for BS/UE ACLR/ACS in the scope of this coexistence study.

Proposal2: Consider the following BS/UE ACLR/ACS values for the NTN-TN coexistence study:

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 20 GHz | 30 | 26 | 19 | 25 | 23.8 | 18.2 |
| 30 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

## Propagation model between NTN UE and TN UE

In the RAN4#105 way forward ([3]), the propagation model between NTN UEs and TN UEs is still listed as an open issue.

We would then propose the following:

Proposal3: Use UMi propagation model between NTN UEs and TN UEs.

## Network Layout

The Way Forward captured the following agreements and open issues:

Like NTN FR1 coexistence study, it’s not realistic to consider the satellite beam will be full of urban macro TNs. As we suggested in [7], we make the following proposal as starting point:

Proposal4: Urban macro TNs shall occupy 50% maximum of a satellite beam.

Regarding the above open issue on NTN UE deployment, the 2 options are not exclusive. RAN4 might want to consider dropping NTN UEs at the edge of TN clusters in a first step but also, if no coexistence issue is found, check if it would also be possible to drop UEs with TN clusters.

Following this, the deployment and observation rules used for NTN FR1 simulations have been reused in Annex 1 (NTN UEs dropped at TN cluster edge) or updated in Annex 2 (NTN UEs dropped in TN) to reflect this.

Proposal5: Agree on the deployment an observation rules as listed in Annex 1 (when NTN UEs are dropped at TN cluster edges) and Annex 2 (when NTN UEs are dropped in TN).

## NTN UE types and parameters

### Type of NTN UE devices

As discussed in our other contribution ([5]), RAN4 should consider 2 main types of NTN UEs:

- NTN VSAT which is fixed UE. Also, this device might not point accurately to the satellite, generating unexpected interference to the adjacent network.

- NTN ESIM which is mobile UE. This device embeds a self-monitoring feature that makes sure the UE will also point to the satellite with a certain accuracy.

For the 2 types of NTN UEs, RAN4 should still finalize the assumptions agreement. Nevertheless, for NTN VSAT RAN4 should further discuss how to simulate this type of device:

- UE antenna deviation from the ideal direction when pointing to satellite. When transmitting, such device could then transmit in an unexpected direction, not pointing to the satellite then.

- Fixed device consideration. For coexistence studies, RAN4 uses Monte-Carlo methodology where UEs are randomly distributed in predefined deployment scenario, this to model UE mobility. With the considered NTN VSAT fixed type of devices, there is no mobility anymore but, on the other hand, the fixed UEs might still be installed anywhere, at any location in the considered area.

Proposal6: RAN4 should define an antenna pointing accuracy assumption when considering NTN VSAT devices.

Proposal7: RAN4 should further discuss how to simulate NTN VSAT (fixed) devices.

### NTN UE antenna parameter

In RAN4#104bis-e meeting, Thales proposed ([4]) to use the following model for VSAT UE antenna:

Fθ=2J1uu

Where:

- J1x  is the Bessel function of first type and first order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

This model seems aligned with the literature dealing with parabolic antenna modelization for simulations purpose.

Proposal8: Reuse the proposed antenna model described above.

### NTN UE other parameters

We proposed above to use 20 GHz and 30 GHz respectively for the reference DL and UL frequency for the coexistence simulations. Based on TR 38.821, we would then suggest the following NTN UE parameters.

Proposal9: Adopt the following parameters for fixed and mobile VSAT:

| Characteristics | Fixed or mobile VSAT |
| --- | --- |
| Polarisation | circular |
| Rx Antenna gain | 39.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |

## TN BS antenna parameters

The Way Forward [3] from last RAN4#105 meeting has still listed the TN antenna parameters as an open issue. As already proposed in our contribution [6]in that meeting, we make the following proposal again:

Proposal10: Adopt the following TN antenna parameters:

|  | Macro urban |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 l dv = 0.5 l |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 10 |

## Preliminary results

## Based on the previously agreed assumptions and the proposals made in this document, we are early sharing here after some preliminary results. In the following, the NTN UEs are dropped at TN clusters edge. Only NTN UL at 30 GHz has been considered.

### Aggressor: NTN UL – Victim: TN DL

| Required ACIR |  | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | 0.5 | 0.5 | 0.4 | 0.4 |
|  | 5%-tile | 6.7 | 2.8 | 2.7 | 2.4 |

### Aggressor: TN DL – Victim: NTN UL

| Required ACIR |  | 2 | 4 | 6 |
| --- | --- | --- | --- | --- |
| Throughput Loss | Average | NA | NA | NA |
|  | 5%-tile | 1.6 | 0.2 | 0.1 |

### Aggressor: NTN UL – Victim: TN UL

| Required ACIR |  | 32 | 33 | 34 | 35 | 36 |
| --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 |
|  | 5%-tile | 12.1 | 8.3 | 8.2 | 4.0 | 0.7 |

### Aggressor: TN UL – Victim: NTN UL

| Required ACIR |  | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Throughput Loss | Average | NA | NA | NA |
|  | 5%-tile | 0 | 0 | 0 |

# Conclusion

In this contribution, we analyzed the remaining open issues for the NTN Ka-band coexistence study. We made the following proposals:

Proposal1: Study NTN-TN coexistence by assuming a reference frequency of 20GHz for NTN DL cases and 30 GHz NTN UL cases.

Proposal2: Consider the following BS/UE ACLR/ACS values for the NTN-TN coexistence study:

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 20 GHz | 30 | 26 | 19 | 25 | 23.8 | 18.2 |
| 30 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Proposal3: Use UMi propagation model between NTN UEs and TN UEs.

Proposal4: Urban macro TNs shall occupy 50% maximum of a satellite beam.

Proposal5: Agree on the deployment an observation rules as listed in Annex 1 (when NTN UEs are dropped at TN cluster edges) and Annex 2 (when NTN UEs are dropped in TN).

Proposal6: RAN4 should define an antenna pointing accuracy assumption when considering NTN VSAT devices.

Proposal7: RAN4 should further discuss how to simulate NTN VSAT (fixed) devices.

Proposal8: Reuse the proposed antenna model described above.

Proposal9: Adopt the following parameters for fixed and mobile VSAT:

| Characteristics | Fixed or mobile VSAT |
| --- | --- |
| Polarisation | circular |
| Rx Antenna gain | 39.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |

Proposal10: Adopt the following TN antenna parameters:

|  | Macro urban |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 l dv = 0.5 l |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 10 |

# Reference

- RP-222564, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), Rel-16

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2, Samsung

- R4-2215348, VSAT UE Characteristics and Initial Simulation Parameters, Thales

- R4-2301468, NTN enhancements – UE RF requirements, Ericsson

- R4-2218490, NTN enhancements – Simulation assumptions for NTN FR2 co-existence study, Ericsson

- R4-2205045, NTN – SAN ACS and case 6 – NB-IoT coexistence, Ericsson

# Annex 1. Consideration of Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

# Annex 2. Consideration of Network and UE deployment – NTN UE in TN cluster

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s). |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | Consider an active rate of 20% for TN. | Only the active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped in TN clusters | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) . |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE NTN UEs dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |