3GPP TSG-RAN4 Meeting #106	R4-2301484

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Source:			Ericsson

Title:			LS Reply to RAN1 on NR support for dedicated spectrum less than 5MHz for FR1

Agenda item:		9.14.1

Document for:		Approval

# Introduction

A new Work Item ([1]) was approved in RAN#95e to add NR support for dedicated spectrum less than 5 MHz for FR1, introducing new channel bandwidth(s) narrower than 5 MHz.

In the scope of this WI, RAN1 sent a LS to RAN4 ([2]) requesting some information on 3 MHz channel bandwidth signals. This contribution is discussing the answers to RAN1 questions and propose a LS reply to RAN1 in annex.

# Discussion

RAN1 asked RAN4 the following 2 questions:

Regarding the question 1, as proposed and explained in our other contribution ([3]), we propose to reply to RAN1 that the maximum transmission bandwidth configuration for 3 MHz channel bandwidth signal shall be equal to 15 RBs.

Regarding the question 2, as also discussed in our other contribution ([3]), a finer sync raster would be needed to give a full flexibility when deploying network supporting 3 MHz channel bandwidth. Also, based on our proposals ([3]), such finer raster would be feasible. Nevertheless, several options exist depending on RAN1 decision on the punctured SSB design. We propose so to request RAN1 the following inputs:

- What would be the size (number of PRBs) of the punctured SSB?

- Would the punctured SSB always have the same design (position of PSS/SSS and PBCH in the punctured SSB)?

Based on those inputs, we have drafted in annex a LS Reply to RAN1 for approval.

# References

- RP-222645, WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell

- R1-2212919, LS on NR support for dedicated spectrum less than 5MHz for FR1, Nokia

- R4-2301483, Spectrum less than 5 MHz - System parameters, Ericsson

# Annex: LS Reply to RAN1

3GPP TSG-RAN4 Meeting #106	R4-230xxxx

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Title:	LS Reply on NR support for dedicated spectrum less than 5MHz for FR1

Response to:	LS on NR support for dedicated spectrum less than 5MHz for FR1

Release:	Rel-18

Work Item:	-

Source:	RAN WG4

To:	RAN WG1

Cc:	-

Contact person:	Dominique Everaere

dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

# 1	Overall description

RAN4 would like to thank RAN1 for the LS R1-2212919 on NR support for dedicated spectrum less than 5MHz for FR1, early sharing as well the RAN1 agreements made in last RAN1#111 meeting.

RAN4 has discussed the 2 RAN1 questions in RAN4#106 meeting and would like to give RAN1 the following answers.

Question 1: RAN1’s understanding is that in addition to reusing 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 response on the maximum transmission bandwidth (the number of PRBs) for this channel BW.

RAN4 answer1: RAN4 agreed that the maximum transmission bandwidth for a 3 MHz channel bandwidth signal shall be 15 PRBs.

Question 2: RAN1 have discussed aspects related to synch raster in the spectrum of interest. RAN1 would like to ask RAN4 if finer synch raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as if RAN4 needs any input from RAN1.

RAN4 answer2: To provide a maximum flexibility when deploying 3 MHz channel bandwidth operation, RAN4 agreed to specify a finer synchronization raster. RAN4 would kindly request to RAN1 the following inputs to be able to conclude on this new raster synchronization scheme:

- Question 1: Based on RAN4 answer to RAN1 question1, what would be the size (number of PRBs) of the punctured SSB?

- Question 2: Would the punctured SSB always have the same design (e.g. same position of PSS/SSS and PBCH in the punctured SSB)?

# 2	Actions

To RAN WG1

ACTION: 	RAN4 kindly requests RAN1 to answer to questions 1 and 2.

# 3	Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #106bis-e	April 17-26, 2023	Online

TSG-RAN4 Meeting #107	May 22-26, 2023	Seoul, Korea