# 3GPP TSG-RAN WG4 Meeting #106	R4-2301533

Athens, Greece, February 27 – March 3, 2023

Source: 	vivo

Title: 	Simulation results for adjacent channel co-existence evaluation

Agenda Item:	9.19.2.1

Document for:	Approval

Introduction

In the last meeting, the adjacent channel co-existence scenarios and detailed simulation assumptions were further discussed. The agreements and way-forwards for adjacent channel co-existence simulation of non-overlapping subband full duplex (SBFD) were captured in [1]. Just before this meeting, the calibration work for adjacent channel co-existence evaluation was performed offline. In this contribution, we provide our simulation results for adjacent channel co-existence evaluation after calibration.

Discussion

In this contribution, we only considered the scenarios for SBFD co-existence study with high priority, as shown in Table 1. The Urban Macro scenario is agreed as baseline scenario for SBFD co-existence study with high priority.

Table 1: Scenarios for SBFD co-existence study

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
| FR2 (30GHz) | 3 | Urban Macro -> Urban Macro | High |

The aggressor and victim combinations are listed in Table 2 below.

Table 2: Victim, aggressor and aggressor baseline for SBFD co-ex study

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 |  | High |
| SBFD (DUD) | NR TDD DL | Case 7 | No system in adjacent channel | High |
| SBFD(DU) |  | Case 8 |  | High |

The detailed simulation assumptions for FR1 and FR2 Urban Macro scenarios were given in [2]. The grid shift assumption is 100% for all the cases of the urban macro deployments. Both SBFD antenna configuration 1 and configuration 2 are considered in the SLS.

The simulation results for SINR and throughput distributions are given for Case 1,2,7,8 for FR1 and FR2 as in Section 2.1~2.4.

Case 1

FR1

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 4.67 | 25.26 | 1.18 | 5.04 |
| ACI | 4.67 | 25.26 | 1.19 | 5.06 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.11% | 0.52% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.62 | 23.86 | 0.66 | 4.76 |
| ACI | 0.62 | 23.86 | 0.66 | 4.76 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.14% | 0.02% |

FR2

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 3.01 | 23.85 | 0.95 | 4.76 |
| ACI | 3.01 | 23.85 | 0.95 | 4.83 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.21% | 1.51% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 3.28 | 27.28 | 0.99 | 5.44 |
| ACI | 3.28 | 27.28 | 1.00 | 5.48 |
| Gain (acl-baseline)/basline | N/A | N/A | 1.56% | 0.67% |

Case 2

FR1

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.97 | 23.94 | 0.82 | 4.77 |
| ACI | 1.97 | 23.94 | 0.82 | 4.77 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.03% | 0.00% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.97 | 25.04 | 0.82 | 4.99 |
| ACI | 1.97 | 25.04 | 0.82 | 4.99 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.00% | 0.00% |

FR2

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.58 | 23.67 | 0.77 | 4.72 |
| ACI | 1.58 | 23.67 | 0.77 | 4.73 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.00% | 0.10% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.76 | 26.75 | 0.68 | 5.33 |
| ACI | 0.76 | 26.75 | 0.68 | 5.35 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.00% | 0.35% |

Case 7

FR1

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.23 | 26.46 | 0.73 | 5.28 |
| ACI | 1.23 | 26.46 | 0.73 | 5.20 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.01% | -1.39% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -5.01 | 8.77 | 0.16 | 1.24 |
| ACI | -5.01 | 8.77 | 0.16 | 1.13 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.04% | -8.30% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.36 | 23.60 | 0.64 | 4.71 |
| ACI | 0.36 | 23.60 | 0.64 | 4.70 |
| Gain (acl-baseline)/basline | N/A | N/A | 0.00% | -0.17% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.91 | 9.11 | 0.20 | 1.28 |
| ACI | -3.91 | 9.11 | 0.17 | 1.14 |
| Gain (acl-baseline)/basline | N/A | N/A | -15.49% | -10.53% |

FR2

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 2.03 | 26.24 | 0.83 | 5.23 |
| ACI | 2.03 | 26.24 | 0.78 | 5.03 |
| Gain (acl-baseline)/basline | N/A | N/A | -6.12% | -3.78% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -1.10 | 12.74 | 0.33 | 1.72 |
| ACI | -1.10 | 12.74 | 0.33 | 1.71 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.01% | -0.48% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.55 | 25.23 | 0.55 | 5.03 |
| ACI | -0.55 | 25.23 | 0.55 | 4.95 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.01% | -1.57% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.57 | 12.54 | 0.21 | 1.70 |
| ACI | -3.57 | 12.54 | 0.21 | 1.69 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.03% | -0.26% |

Case 8

FR1

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 2.31 | 25.96 | 0.86 | 5.18 |
| ACI | 2.31 | 25.96 | 0.86 | 5.11 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.06% | -1.25% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -4.36 | 9.59 | 0.18 | 1.33 |
| ACI | -4.36 | 9.59 | 0.14 | 1.22 |
| Gain (acl-baseline)/basline | N/A | N/A | -23.98% | -8.73% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.34 | 24.23 | 0.74 | 4.83 |
| ACI | 1.34 | 24.23 | 0.74 | 4.81 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.03% | -0.40% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -4.26 | 9.54 | 0.18 | 1.33 |
| ACI | -4.26 | 9.54 | 0.14 | 1.20 |
| Gain (acl-baseline)/basline | N/A | N/A | -23.03% | -9.87% |

FR2

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.14 | 25.85 | 0.59 | 5.15 |
| ACI | -0.14 | 25.85 | 0.55 | 5.04 |
| Gain (acl-baseline)/basline | N/A | N/A | -5.81% | -2.26% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -1.44 | 12.57 | 0.31 | 1.70 |
| ACI | -1.44 | 12.57 | 0.31 | 1.69 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.49% | -0.77% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 2.70 | 23.81 | 0.91 | 4.75 |
| ACI | 2.70 | 23.81 | 0.90 | 4.72 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.81% | -0.67% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.91 | 12.46 | 0.20 | 1.69 |
| ACI | -3.91 | 12.46 | 0.20 | 1.68 |
| Gain (acl-baseline)/basline | N/A | N/A | -0.17% | -0.63% |

Conclusion

This contribution provides initial simulation results of SINR and throughput distributions for adjacent channel co-existence in FR1 and FR2, considering Urban Macro scenario as high priority. The following observation is made:

For Urban Macro scenario

- SBFD (DUD/DU) Aggressor->NR TDD DL Victim, the throughput losses are within 5%, which shows these two systems can co-exist;

- References

[1] R4-2220246, WF for co-existence study, CMCC, Samsung, RAN4#105.

[2] R4-2220247, Simulation assumptions for SBFD adjacent channel co-existence study, Samsung, RAN4#105.