3GPP TSG-RAN WG4 Meeting #106                                                                 R4-2301578

Athens, Greece, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on SL-U impact on RRM requirements

Agenda item:	9.29.3

Document for:	Discussion

Introduction

In this paper we discuss RRM aspects related to the SL-U for Rel-18. In RAN4#105 we had the initial discussions related to the WI and this is a continuation of the discussion. We address the objectives of the work item and based on those, observations and proposals are derived concerning assumed architecture, deployment, and

The updated WID [1] of NR sidelink evolution from 3GPP TSG RAN Meeting #98-e is presented below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

Discussion

Overview of RRM impacts due to SL-U

- In NR-U, the key aspect impacting RAN4 RRM work was the Listen-Before-Talk (LBT) both on the DL and UL, which caused discontinuous transmissions in the unlicensed spectrum. In addition, other aspects related to operation in unlicensed band are transmit power constraints, power spectrum density (PSD) and occupied bandwidth (OCB) requirements. Similar aspects of LBT could be considered in SL-U.

The RAN4 RRM requirements for NR-U could be used as reference for SL-U as much as possible.

Current RRM measurement accuracy requirements in SL are defined for the TDD band (38.133 [2], section 10.4). To accommodate for the LBT mode of operation in SL-U, RAN4 can study if the measurement accuracy requirement defined for NR-U for the CCA band i.e., n46 and n96 can be reused.

RAN4 to study if the measurement accuracy requirement defined for NR-U for the CCA band can be reused.

In shared channel access operation, initiating UE which intends to perform the SL transmission needs to successfully complete the LBT procedure check before being able to initiate the same transmission by performing the channel occupancy measurement in CCA slots of 9 µs duration. If the RSSI at the UE while performing channel occupancy measurement meets certain conditions with respect to the configured channelOccupancyThreshold then the channel is deemed to be available for transmission. RAN4 can study if the similar channel occupancy measurement under CCA for NR-U could be reused for SL-U operation.

Figure 1: CCA slot with duration Tsl = 9 µs, where the energy sensing takes place during 4 µs.

The general approach that can be adopted by RAN4 RRM when defining the SL-U requirements is to extend the measurements/evaluation periods and timing requirements to account for LBT failures.

Measurements periods and timings requirements need to account for LBT failure in SL-U.

The uncertainty in the transmission and/or reception caused by the LBT procedure has an immediate impact on the periodic signals such as sidelink synchronization signal block (S-SSB), and channel state information reference signal (CSI-RS), thus, the presence of these signals cannot be guaranteed. Therefore, as in NR-U, the uncertainty of LBT access will have an impact on all the procedures that rely on successful LBT access, and such impact needs to be considered in a similar way as in NR-U.

The uncertainty of LBT access will impact all the procedures or features of SL-U, in a similar way that the NR-U got impacted.

In the following, we provide the detailed impact on each clause of SL-U features or/and mechanisms of TS 38.133 [2]. Table 1 shows the expected impact on TS 38.133 [2], taking NR-U as the baseline for SL-U. According to the WI, for Rel-18, the SL-U cell can operate in FR1, thus all the requirements are defined on FR1. All the discussed requirement impact below should take this into account.

In Rel-18, an SL-U is always operating in FR1, so all requirements for SL-U should only be defined for FR1.

In general, based on our observation and analysis, we have identified some RRM requirements that needs to be studied further, as provided in Table 1.

Table 1 – Overview of potential SL-U impacts on TS 38.133 [2]

| Section | Comments |
| --- | --- |
| 12.2 UE Transmit Timing | In SL-U, a SyncRef UE transmitting sidelink synchronization signals may suffer LBT failure. Thus, this requirement might be impacted for SL-U when the UE transmit reference timing used for deriving sidelink transmission is from SyncRef UE. Hence, RAN4 should study the transmission timing error for sidelink transmissions and may define the timing error limit to cope with LBT failure. |
| 12.3 Initiation/Cease of SLSS Transmissions | This requirement might be impacted for SL-U when SyncRef UE is used as the synchronization reference source. RAN4 should discuss if measurement requirement should be modified to cope for LBT failures on SyncRef UE, at least if additional S-SSBs are not enabled. |
| 12.4 Selection / Reselection of V2X Synchronization Reference Source | The measurement period needs to be further studied when the synchronization reference source is configured as the highest priority. Additionally, the UE transmission drop rate needs to be updated to account for LBT failures. |
| 12.6 Congestion Control measurements | Congestion control measurements requirements needs to be studied further as the SL-RSSI measurement performed for congestion control required to meet the SL-RSSI measurement accuracy requirements defined in Clause 10. RAN4 needs to study this aspect further and might need to align the SL-RSSI accuracy requirement similar to NR-U requirements. |
| 12.7 Interruption | The changes to the interruption requirements would be minimal when compared to NR-U. Thus, for SL-U, NR-U interruption requirements can be reused as a baseline, accounting all the scenarios and interruption types applicable for SL-U are properly covered. |

According to what was discussed in Table 1, the following observation and proposal can be made:

In unlicensed carriers, UEs cannot expect regular transmission of control and reference signals due to the uncertainty of the outcome of the LBT procedure. Therefore, the RRM measurement and reporting may be impacted by the reduced transmission opportunity of reference signals due to LBT failure.

RAN4 should study at least the potential RRM requirement impacts mentioned in Table 1 for sidelink unlicensed, based on RAN1 progression.

Conclusion

In this contribution, we have discussed potential impacts of SL-U on the TS 38.133 [2] specifications.

The following Observations and Proposals were made:

Observation 1: The RAN4 RRM requirements for NR-U could be used as reference for SL-U as much as possible.

Proposal 1: RAN4 to study if the measurement accuracy requirement defined for NR-U for the CCA band can be reused.

Observation 2: Measurements periods and timings requirements need to account for LBT failure in SL-U.

Observation 3: The uncertainty of LBT access will impact all the procedures or features of SL-U, in a similar way that the NR-U got impacted.

Observation 4: In Rel-18, an SL-U is always operating in FR1, so all requirements for SL-U should only be defined for FR1.

Observation 5: In unlicensed carriers, UEs cannot expect regular transmission of control and reference signals due to the uncertainty of the outcome of the LBT procedure. Therefore, the RRM measurement and reporting may be impacted by the reduced transmission opportunity of reference signals due to LBT failure.

Proposal 2: RAN4 should study at least the potential RRM requirement impacts mentioned in Table 1 for sidelink unlicensed, based on RAN1 progression.

- References

- RP-222806, WID on NR sidelink evolution, 3GPP TSG RAN#98-e.

- 3GPP TS-38.133, “Requirements for support of radio resource management (Release 17)”