3GPP TSG-RAN WG4 Meeting #106	R4-2301614

Athens, Greece, Feb 27th – March 3rd, 2023

Agenda Item:	8.33.1

Source:	MediaTek Inc.

Title: 	On system parameters for n54

Document for:	Discussion

# Introduction

In RANP#98e, a new band request for n54 was approved [1] with the following objective regarding BS/UE requirements:

- Specify UE/BS RF requirements for a new NR TDD operating band: - BS transmit frequency range: 1670 MHz –1675 MHz, UE transmit frequency range: 1670 MHz–1675 MHz - Channel bandwidths of 5 MHz with supported subcarrier spacing of 15 kHz

Accordingly, we discuss the system parameters for this new band, including operating band, channel bandwidth, channel raster and sync raster.

# Discussion

## 2.1 Operating band

It is clear according to the WID that a TDD band with a frequency range 1670 – 1675 can be specified:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n54 | 1670 MHz – 1675 MHz | 1670 MHz – 1675 MHz | TDD |

## 2.2 Channel bandwidth

Only 5MHz and 15kSCS is requested. For 30kSCS, it can be added upon any new request.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n54 | 15 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.3 Channel raster

Firstly we could calculate the range of NREF according to the following calculation on channel raster [2]:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

For the starting point 1670M, its NREF = 1670M/5k = 334000. And the ending point 1675M, its NREF = 1675M/5k = 335000. Therefore, the channel raster would be:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |

## 2.4 Sync raster

For sync raster, the applicable GSCN should follow [2]:

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Since the half size of an SSB is 1.8MHz for SCS = 15kHz, we could calculate GSCNmin = 4179 corresponding to SSREF = 1671.65MHz, and GSCNmax = 4187 corresponding to SSREF = 1674.25MHz. Therefore, the proposed sync raster is:

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 15 kHz | Case A | 4179 – <1> – 4187 |

# Conclusion

In this contribution we have the following proposals on system parameters for n54 :

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n54 | 1670 MHz – 1675 MHz | 1670 MHz – 1675 MHz | TDD |

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n54 | 15 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 15 kHz | Case A | 4179 – <1> – 4187 |

# References

- RP-223514, New WID: Introduction of NR TDD band in 1670 – 1675 MHz, Ligado Networks.

- TS 38.101-1, v17.8.0