3GPP TSG-RAN WG4 Meeting # 106										  R4-2301615

Athens, Greece, February 27 – March 3, 2022

Title: 	Discussion on CA_n5-n8

Source: 	Xiaomi

Agenda item:	9.2.2.1

Document for:	Approval

- Introduction

In RAN4#105 meeting, several concrete RF architectures are proposed including 2 antenna and 3 antenna. How to address the spectrum overlap issue between n8 UL and n5 DL is still FFS. Associated agreements and way forwards are captured in [1]. This contribution will further provide our view on CA_n5-n8.

- Discussion

Way forward: Companies to propose which of the methods is used to manage the overlap of n5 DL and n8 UL - Option 1: Restricting UL support to n5 UL only - Option 2: 2UL but Non-concurrent operation between n5 DL and n8 UL - NOTE: Check if this kind of configuration is allowed according to current RAN2 specifications

Option 1 is 1UL CA configuration that only allows n5 UL only. This is a feasible solution to avoid the interference between n8 UL and n5 DL but it precludes 2UL CA configuration.

Option 2 allows 2UL CA configuration but restricts the simultaneous n5 DL and n8 UL. While the number of UL band is larger than that of DL band, it can work with proper network scheduling. This kind of configuration is allowed according to current RAN2 specifications. So option 2 is also a feasible solution that enables more UL transmissions.

| CA_n5-n8 with 1UL (Option 1) |  |  |
| --- | --- | --- |
| UL configuration | DL configuration | Allowed or not |
| n5 UL | n5 DL n8 DL | Yes |
| n8 UL | n5 DL n8 DL | No |

| CA_n5-n8 with 2UL (Option 2) |  |  |
| --- | --- | --- |
| UL configuration | DL configuration | Allowed or not |
| n5 UL n8 UL | n5 DL n8 DL | No |
| n5 UL n8 UL | n8 DL | Yes |
| n5 UL n8 UL | n5 DL | No |

Proposal 1: Both option 1 and option 2 can be introduced for CA_n5-n8 with 1UL CA configuration and 2UL CA configuration respectively.

The following architectures have been discussed: Two antenna: Three antenna: Way forward: Companies to propose which RF architecture is used derive requirements NOTE: Requirements are not specified during the SI

Both 2 antenna and 3 antenna have identified several RF architectures for full band or restricted spectrum. The feasibility of each RF architecture should be evaluated before diving into the requirements. During the phase of study item, it would be difficult to propose which RF architecture is used to derive requirements. RAN4 also have achieved consensus that UE requirements should be specified in the follow-up work item. In our understanding, the worst case can be used to derive the minimum UE RF requirements. However, this can be further discussed in the phase of work item.

Proposal 2: How to specify RF requirements can be decided in the follow-up work item.

- Conclusion

This contribution gives our views on CA_n5A-n8A. The following proposals are provided:

Proposal 1: Both option 1 and option 2 can be introduced for CA_n5-n8 with 1UL CA configuration and 2UL CA configuration respectively.

Proposal 2: How to specify RF requirements can be decided in the follow-up work item.

- Reference

- R4-2220507, WF on UE architecture and other aspects for CA_n5-n8, Qualcomm, RAN4#105