3GPP TSG RAN WG4 Meeting #106		 R4-2301721

# 27 February - 3 March 2023, in Athens, Greece

Source:	MediaTek Inc., Telstra

Title:	TP for TR 38.718-02-01: CA_n26-n28

Agenda item:			x.x.x

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n26A-n28A as request by operator.

2. Discussion

The combo is a L-L band combination with small gap between uplink range of n26 and downlink range of n28. The maximum uplink bandwidth of n26 is up to 20MHz that its ACP1 would fall into DL range of n28 directly if uplink is allocated full RB. Below table lists our assumption for MSD characterization. Here we consider three antenna implementation since the frequency gap between n26 uplink and n28 downlink is only 11MHz that we don’t think a low band diplexer is feasible with existing technology. Based on previous RAN4 agreement, there shall not be frequency range constraint on n28 thus MSD shall be characterized with full frequency range.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection at n26 UL range | 15 | dB |
| n26 ACP1 | -30 | dBc |
| n26 CIM3 | -60 | dBc |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| PA RXBN noise | -115 | dBm/Hz |
| Thermal noise at n28 RX ANT port | -165.5 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

With above parameters, we calculate MSD due to cross band isolation as below:

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n26 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n26PA output noise power at n28 LNA input port | -71.3 | -71.3 | dBm/5MHz |
| n28 LNA IM2 due to n26 uplink | -53 | -53 | dBm/5MHz |
| Thermal noise at RX ant port | -162.5 |  | dBm/Hz |
| CIM3 noise at n28 LNA input port | -55.8 | -55.8 | dBm |
| Total noise level refer to B28 LNA input port | -51.1 | -51.1 |  |
| Combined |  |  |  |
| MSD (5MHz BW) | 47.4 |  | dB |

Table 2 Link analysis for Band 28 5MHz REFSENS calculation at fc = 800.5 MHz

With the above analysis we found that due to insufficient isolation of existing n28B filter that the degradation on REFSENS that makes uplink in n26 for the combo would block the n28 receiving signal that makes the carrier aggregation has very limited gain comparing to non-CA operation. We propose for the CA_n26-n28, uplink is only in n28.

Proposal 1: For CA_n26-n28, uplink is only in band n28

3. Text Proposal

---Start of changes---

5.x	n26-n28

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1		Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n26 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |

5.x.1.2		Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n26+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n28A | n28 | n26 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |

5.x.1.3		UE co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n28.

Table 5.x.1.3-1: Impact of UL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |

Table 5.x.1.3-2: Impact of DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 | 3790 | 4015 |

There’s no REFSENS impact due to UL harmonic or DL harmonic mixing.

5.x.1.4		∆TIB and ∆RIB values

For CA_n26-n77, the TIB,c and RIB,c values are same as for CA_n5-n28 and are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.5 |
|  | n28 | 0.5 |

Table 5.x.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.2 |
|  | n28 | 0.2 |

5.x.1.5 		REFSENS requirements

There’s no MSD exception needed due to uplink is only in n28

5.x.1.6	OOB blocking exception requirements

There’s no OOBB exception needed for the combo.

5.x.2		Specific for 2 bands UL CA

N/A since the combo does not target on 2 bands UL CA

---End of changes---

Reference

- R4-2219214	Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), RAN4#105