3GPP TSG-RAN WG4 Meeting#106                               R4-2301728

Athens, Greece,27th Feb – 3rd Mar, 2023

Agenda item:	9.13.2.2

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Rx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on ATG UE RF requirements based on the initial evaluation results obtained so far.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1. ACS requirement of ATG UE

Based on the initial simulation results obtained so far for Case 3 and Case 11 as shown in Figure 2 and Figure 3, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Figure 1. simulation results for Case 3: TN DL interfering ATG DL @4GHz

Figure 2. simulation results for Case 10: TN DL interfering ATG DL @2GHz

Observation 1: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

## 2.2. Other Rx RF requirements for ATG UE

In addition, in the following table 1, we share some further views on the ATG UE Rx RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | During the last RAN4 meeting, there were some offline discussions for 2Rx or 4Rx for ATG UE, however no conclusion was draw on the end. First of all, as we highlighted for power class part, to use similar approach as IAB-MT, this could give more freedom of implementation cater for the practical diverse market demand. For IAB-MT introduced in Rel-16, it’s also not mandatory to support 4Rx for IAB-MT and its reference requirement for IAB-MT is defined per antenna connector FR1 IAB-MT type 1-H. In addition, if we consider the 4Rx for ATG CPE, at least two panels with cross-polarization on each panel are needed, however due to the limitation of installation on ATG UE on the aircraft, it will be difficult to have 4Rx in practice at least based on our experience. Based on the above clarifications, we propose to have 2 Rx branches as baseline and it’s not necessary to mandate 4Rx for band n1/n78 and n79 since antenna assumptions for ATG UE and handheld smartphone are different. Proposal 1: to have 2 Rx branches as baseline and not to mandate 4Rx for band n1/n78 and n79. |
| Reference sensitivity | Yes | - To follow the existing Reference requirements of band n1/78/79 for 2Rx in TS 38.101-1. - Proposal 2: to reuse the existing reference requirement of band n1/n78/n79 fof 2Rx in TS38.101-1. |
| Maximum input level | No | This depend on the simulation results input to verify whether this requirement could be further relaxed similar as NR over NTN in Rel-17. Based on the coupling loss shown as following, the maximum input power could be further relaxed around 30dB compared with MCL 70dBc for Wide area BS class. Proposal 3: the maximum input power could be relaxed around [30dBc]. |
| ACS | No | Please see the simulation results in section 2.1 and its observations. |
| Blocking characteristics |  |  |
| - In-band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Narrow band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be much lower compared with the handheld smartphone |
| Spurious response | No | Expect the same as TN and its power level of interfering signal could be further investigated similar as in-band blocking and out of band blocking. |
| Intermodulation | No | Not applicable |
| Spurious emissions | No | To reuse General requirements defined in TS 38.101-1 |

Conclusions

In this contribution, we shared some further views for ATG UE Rx RF requirements and proposals are made as following:

Observation 1: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Proposal 1: to have 2 Rx branches as baseline and not to mandate 4Rx for band n1/n78 and n79.

Proposal 2: to reuse the existing reference requirement of band n1/n78/n79 fof 2Rx in TS38.101-1.

Proposal 3: the maximum input power could be relaxed around [30dBc].

Proposal 4: for in-band blocking, to use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed.

Proposal 5: for out of band blocking and NBB requirement, to use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed.

Proposal 6: for Spurious response, to use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.