3GPP TSG-RAN WG4 Meeting#106                              R4-2301731

Athens, Greece, Feb 27 – Mar 3,2023

Agenda item:	9.13.1.4

Source: 	ZTE Corporation

Title: 	Initial simulation results for ATG coexistence study

Document for:	Discussion

Introduction

- In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some initial simulation results for it based on the agreed simulation assumption. The simulation results for 4GHz is based on 7.5km ISD instead of 5km ISD. This could be updated later.

- Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1. Case 1,9 for ACLR requirement of ATG BS

Based on the initial simulation results obtained so far for Case 1 and Case 9 as shown in Figure 1 and Figure 2, ACI caused by ATG DL is quite limited due to both the sparse deployment sites of ATG BS and its uptilt beamforming towards the aircraft in the sky. In short, based on the initial simulation results in Case 1 and Case 9 dominating the ATG BS DL ACLR requirement, it should be sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Figure 1. simulation results for Case 1: ATG DL interfering TN DL @4GHz

Figure 2. simulation results for Case 9: ATG DL interfering TN DL @2GHz

Observation 1: based on the initial simulation results in Case 1 and Case 9, it’s sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

## 2.2. Case 4, 12 for ACS requirement of ATG BS

Based on the initial simulation results obtained so far for Case 4 and Case 12 as shown in Figure 3 and Figure 4, regardless of co-located deployment between ATG BS and the legacy TN BS, it should be sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

-

Figure 3a. simulation results for Case 4: TN UL interfering ATG UL @4GHz [non co-located deployment]

Figure 3b. simulation results for Case 4: TN UL interfering ATG UL @4GHz [co-located deployment]

Figure 4a. simulation results for Case 12: TN UL interfering ATG UL @2GHz [non co-located deployment]

- Figure 4b. simulation results for Case 12: TN UL interfering ATG UL @2GHz [co-located deployment]

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

## 2.3. Case 2,10 for ACLR requirement of ATG UE

Based on the initial simulation results obtained so far for Case 2 and Case 10 as shown in Figure 5 and Figure 6, ACI caused by ATG UL towards TN UL are still quite limited. In short, based on the initial simulation results in Case 2 and Case 10 dominating the ATG UE UL ACLR requirement, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Figure 5a. simulation results for Case 2: ATG UL interfering TN UL @4GHz [non co-located deployment]

Figure 5b. simulation results for Case 2: ATG UL interfering TN UL @4GHz [co-located deployment]

Figure 6a. simulation results for Case 10: ATG UL interfering TN UL @2GHz [non co-located deployment]

Figure 6b. simulation results for Case 10: ATG DL interfering TN UL @2GHz [co-located deployment]

Observation 3: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

## 2.4. Case 3 and 11 for ACS requirement of ATG UE

Based on the initial simulation results obtained so far for Case 3 and Case 11 as shown in Figure 7 and Figure 8, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Figure 7. simulation results for Case 3: TN DL interfering ATG DL @4GHz

Figure 8. simulation results for Case 10: TN DL interfering ATG DL @2GHz

Observation 4: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Conclusions

In this contribution, we shared some further views on coexistence study for ATG coexistence and proposals are made as following:

Observation 1: based on the initial simulation results in Case 1 and Case 9, it’s sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

Observation 3: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Observation 4: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214459,WF on co-existence evaluation for ATG, Huawei, Approved.

- R4-2217736, WF on FR1 co-existence evaluation for ATG, CMCC, Approved.