3GPP TSG-RAN WG4 Meeting # 106	R4-2301829

Athens, Greece, February 27 – March 3, 2022

Title:	Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility

Response to:	R1-2210727

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:	Han Jing

E-mail Address:	hw.hanjing@huawei.com

Attachments:

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on i L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility. On top of the reply LS [R4-2220733], RAN4 further discussed the second question, and provide the following feedback for RAN1’s questions:

Question 1 (to RAN4): RAN1 respectfully asks RAN4 if the restriction on e.g., SFN offset alignment, BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP, and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

RAN4 feedback:

RAN4 think the restrictions mentioned in Question 1 which are imposed on R17 ICBM can be somewhat relaxed in Rel-18 L1/L2 mobility.

Regarding SSB based L1 intra-frequency measurement for candidate cell,

- If UE ever reported time index when performed L3 measurement on the non serving cell, the restriction of sfn-SSB-Offset can be removed, otherwise SFN offset alignment compared with serving cell is needed for intra-frequency L1-RSRP.

- Both scenarios where non-serving cell SSB is covered within serving cell active BWP and not covered within active BWP are to be supported. The conclusion on FG 6-1a can be reused for SSB based intra-frequency L1-RSRP measurement where SSB from neighbor cell is not within active BWP.

- The scenario that RTD between the SSBs of serving cell and cell with different PCI is larger than CP length of the corresponding SCS is to be supported. UE capability can be considered.

2. To RAN WG1 and WG2 group.

ACTION: 	RAN4 respectfully ask RAN1 and RAN2 to take the above agreements into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #106-bis-e		Online

TSG-RAN4 Meeting #107		22nd - 26th May 2023     Incheon