3GPP TSG-RAN WG4 Meeting #106 	R4-2301846

Athens, Greece, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on applicability of requirements for RedCap UE

Agenda item:	5.2.5.1

Document for:	Discussion

Introduction

This discussion paper is regarding the response to R5-228034 [1].

During the last RAN5 meeting, RAN5 could not reach consensus on whether receiver requirements could be verified in case of SUL band combinations for RedCap UE and thus following two questions were raised to RAN4:

Question 1:  Are the requirements in clause 7.3C in TS 38.101-1 [2] valid for a RedCap UE, indicating SUL band combinations, to be verified with REFSENS specified in clause 7.3I?

If the answer to Question 1 is yes:

Question 2: How could the requirements in 7.3I be applied to 7.3C in respect to Reference sensitivity side conditions (UL/DL configuration), sensitivity allowance, SUL band combination with HD-FDD band.

This paper provides a discussion on the above questions.

Discussion

Following observations are made regarding RedCap and SUL according to TS 38.101-1 [2]

Section 5.2C, Table 5.2C-1 in [2], provides the operating band combinations for SUL in FR1 and is replicated below.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n802 | n41, n80 |
| SUL_n41-n812 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n842 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Note 2 of the table says, "For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory." i.e., for all the UEs operating in SUL, simultaneous RX and TX capability is mandatory. However, the Rel-17 RedCap WID [3] focuses on SA mode and single connectivity with operation in a single band at a time. This introduces a contradiction where a UE might be assigned to use for example, SUL_n24-n99 and thus will have to simultaneously use n24 for downlink and n99 for uplink i.e., use more than a single band at one time.

Applying NR band combinations for SUL to RedCap UE would introduce a contradiction where a UE might be assigned to use for example, SUL_n24-n99 and thus will have to simultaneously use n24 for downlink and n99 for uplink i.e., use more than a single band at one time as defined in the Rel-17 RedCap WID [3].

In case of a HD-FDD RedCap UE, no SUL band specific uplink configuration is defined in Table 7.3I.2-4 in [2], which is replicated below.

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | NOTE 1 |
|  | 30 |  | 24 | 36 | NOTE 1 |
|  | 60 |  | 10 | 18 | NOTE 1 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

Thus, a HD-FDD RedCap UE is untestable in an SUL band combination.

A HD-FDD RedCap UE is untestable in an SUL band combination.

- In addition, sensitivity allowance is unknown for a RedCap UE with a single antenna port for any of the SUL bands.

Observation 3: Sensitivity allowance is unknown for a RedCap UE with a single antenna port for any of the SUL bands.

Thus, there are several gaps in [2] regarding SUL support for RedCap.

Observation 4: There are several gaps in [2] regarding SUL support for RedCap.

The following proposal is made.

Proposal 1: The requirements in clause 7.3C in TS 38.101-1 [2] are not valid for a RedCap UE.

Conclusion

This paper provides a discussion on the two questions raised by RAN5 in [1].

Following observations are made.

- Applying NR band combinations for SUL to RedCap UE would introduce a contradiction where a UE might be assigned to use for example, SUL_n24-n99 and thus will have to simultaneously use n24 for downlink and n99 for uplink i.e., use more than a single band at one time as defined in the Rel-17 RedCap WID [3].

A HD-FDD RedCap UE is untestable in an SUL band combination.

Observation 3: Sensitivity allowance is unknown for a RedCap UE with a single antenna port for any of the SUL bands.

Observation 4: There are several gaps in [2] regarding SUL support for RedCap.

Based on the presented observations, following proposal is submitted:

Proposal 1: The requirements in clause 7.3C in 38.101-1 [2] are not valid for a RedCap UE.

- References

- R5-228034, “LS on applicability of requirements for RedCap UE”, RAN5 #97, Toulouse, France, November 14 – 18, 2022

- 3GPP TS 38.101-1 v17.8.0, ”User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)”

- RP-211574, “Revised WID on support of reduced capability NR devices”, RAN #92e, June 14 – 18, 2021