3GPP TSG-RAN WG4 Meeting #106	R4-2301857

Athens, Greece, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	UE RF requirements for Enhanced RedCap

Agenda item:	9.30.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on the impact on UE RF requirements for the revised work item description for Rel-18 RedCap in RP-223544 [1].

Discussion

Core objectives

According to the core objectives of the WID [1], as shown below, RAN4 should work on power saving and further reducing the UE complexity in FR1.

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

Impact to UE RF requirements

As shown, the revised WID in RP-223544 only foresees UE BB bandwidth reduction within RF bandwidth of 20 MHz. This will require to study if new UE demodulation requirements for PDSCH, and PUSCH and CSI reporting requirements for PDSCH are needed, however, no impact on the RF requirements is anticipated. BB bandwidth is restricted to 5 MHz. Based on this, following observations can be deduced.

No impact to UE RF requirements is expected.

Rel-18 will operate with same UE RF requirements as Rel-17.

Following proposals are submitted:

RAN4 to agree there is no impact to UE RF requirements from Rel-18 enhanced RedCap.

The agenda item shall be removed.

Conclusion

In this contribution we have provided our view on the impact of Rel-18 enhanced RedCap to UE RF requirements.

Following observations are made.

- No impact to UE RF requirements is expected.

- Rel-18 will operate with same UE RF requirements as Rel-17.

Following proposals are submitted:

- RAN4 to agree there is no impact to UE RF requirements from Rel-18 enhanced RedCap.

The agenda item shall be removed.

- References

- RP-223544, Revised WID: "Revised WID on Enhanced support of reduced capability NR devices", Ericsson, RAN#98-e