3GPP TSG-RAN WG4 Meeting #106		R4-2301870

Athens, Greece, 27th February – 03rd March 2023

Title:	Further reply to LS on applicability of timing error margin of Rx TEG

Response to:	R2-2210977 “Reply LS on applicability of timing error margin of Rx TEG”

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN1, RAN3

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:	N/A

1. Overall Description:

RAN2 asked 4 questions related to application of timing error margin of Rx TEG to RAN4 in R2-2210977. RAN4#105 discussed and provided its response to question #4 in R4-2220729. RAN4#106 discussed the remaining questions in R2-2210977 and agreed to provide following responses.

RAN4 answers to RAN2 questions:

Question 1: Will there also be definition of applicable values of timing error margin for Tx TEG and/or for RxTx TEG, similar to that for Rx TEG?

Answer: No. RAN4 will not define applicability rule for Tx TEG timing error margin value. For RxTx TEG refer to answer to question 3 below.

Question 2: Is the “applicability of timing error margin of Rx TEG” as included in the RAN4 LS (R4-2214493) supposed to be specified in LPP?

Answer: Applicability rule for Rx TEG timing error margin value as included in the RAN4 LS (R4-2214493) is not supposed to be specified in LPP. Based on the agreed Rx/RxTx TEG framework, timing error margin value selected by UE for Rx/RxTx TEG for positioning measurement is reported by UE to LMF. The relevant agreements are copied below for reference.

RAN4#104e agreement:

# Whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling?

Agreements:

- UE Rx/RxTx TEG margins are provided to LMF as LPP signaling parameters outside of UE capability signaling.

# Whether a single timing error margin value is provided per Rx TEG/RxTx TEG type in a single LPP message, even if it has multiple measurement instances?

Agreements:

- A single timing error margin value is provided per Rx TEG/RxTx TEG type per measurement instance in a single LPP message, if it has multiple measurement instances.

# Whether the timing error margin values for an Rx TEG/RxTx TEG type in different LPP messages can be different?

Agreements:

The timing error margin values for an Rx TEG/RxTx TEG type in different LPP messages can be different.

Question 3: Does the applicability of timing error margin of Rx TEG in the LS (R4-2214493) apply for UE Rx-Tx timing difference?

Answer: No. The applicability of timing error margin for Rx TEG in the LS (R4-2214493) does not apply for UE Rx-Tx timing difference. For RxTx TEG the following applicability rule applies:

The applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the group delay margins for the individual measurements plus the frequency drift margin.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to take the above answers into consideration.

3. References:

- R2-2210977, “Reply LS on applicability of timing error margin of Rx TEG”, CATT.

- R4-2220729, “Reply LS on applicability of timing error margin of Rx TEG”, Ericsson.

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #106bis-e		17 – 26 April 2023				E-Meeting

TSG RAN WG4 Meeting #107			22 – 26 May 2023				Incheon, South-Korea