3GPP TSG-RAN WG4 Meeting #106	                               R4-2302031

Athens, Greece, Feb. 27th – Mar. 3rd, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	PC2 FDD bands RSD

Agenda Item:	8.23.2 High power UE for FR1 for FDD single band(s) with PC2 	[HPUE_NR_FR1_FDD_R18]

Document for:	Approval

Introduction

In the RAN4#105 meeting, the Reference Sensitivity Degradation (RSD) of some Low bands and Middle bands added in the new WID [1] has been discussed. In this contribution, we provide our evaluation results for Low bands.

Discussion

Analysis conditions

The two kinds of FE architecture shown in the Figure 1 are analysis baseline. For 2Tx/2Rx architecture, the mutual influence between pTx and dTx shall be considered.

Figure 1. FE architecture for PC2 FDD

RxNoise measurement

PA calibration:

- For PC3：PA is calibrated to deliver - 30 dBc ACLR at 1dB MPR using full allocation 20MHz QPSK DFT-s-OFDM SCS15kHz.

- For PC2：PA is calibrated to deliver - 31 dBc ACLR at 1dB MPR using full allocation 20MHz QPSK DFT-s-OFDM SCS 15kHz.

Waveform:

- QPSK DFT-s-OFDM

- Image and Local Oscillator rejection is set to -28 dBc

- Counter-IM3 set to -60 dBc

- Counter-IM5 set to -70 dBc

UL configuration:

UL configuration is set according to [2].

Table 1 UL configuration for RxNoise measurement

|  |  | UL/DL BW, UL configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS [kHz] | 5 [MHz] | 10 [MHz] | 15 [MHz] | 20 [MHz] | 25 [MHz] | 30 [MHz] | 35 [MHz] |  |
| n5 | 15 | (0,25) | (27,25) | (59,20) | (86,20) | (86,20)* |  |  |  |
| n8 | 15 | (0,25) | (27,25) | (59,20) | (86,20) | (86,20)* | (86,20)* | (86,20)* |  |
| n26 | 15 | (0,25) | (27,25) | (54,25) | (81,25) | (81,25)* | (81,25)* |  |  |
| n28 | 15 | (0,25) | (27,25) | (54,25) | (81,25) | (108,25) | (135,25) |  |  |
| n71 | 15 | (0,25) | (0,25) | (0,20) | (0,20) | (0,20)* | (0,20)* | (0,20)* |  |
| *：DL only, UL BW is 20MHz. |  |  |  |  |  |  |  |  |

RSD analysis

Assumptions

Table 2 basic assumptions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 26 dBm | PC2 PA * 1Tx, at ANT1 |
|  | 23 dBm | PC3 PA * 2Tx, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1⇔ANT2 |
| MRC | ANT1&ANT2 | Correlated |

RSD calculation results

The RSD calculation results are shown in Table 3 and Table 4.

To account the reverse IMD between pTx and dTx, additional 1dB RxNoise contribution is added for 2Tx/2Rx architecture.

Table 3 Reference sensitivity degradation for 1Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n5 | 0.9 | 1.0 | 0.7 | 4.8 | 5.3 | - | - | - | - | - |
| n8 | 0.8 | 0.8 | 0.5 | 4.9 | - | - | 6.8 | - | - | - |
| n26 | 0.7 | 0.9 | 0.9 | 4.8 | 5.3 | 5.8 | - | - | - | - |
| n28 | 0.9 | 0.8 | 1.0 | 1.6 | 2.8 | 4.5 | - | - | - | - |
| n71 | 1.0 | 1.1 | 0.9 | 3.9 | 4.2 | 4.4 | 4.6 | - | - | - |

Table 4 Reference sensitivity degradation for 2Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n5 | 1.0 | 1.2 | 1.3 | 5.2 | 5.9 | - | - | - | - | - |
| n8 | 1.3 | 1.3 | 1.4 | 5.5 | - | - | 7.7 | - | - | - |
| n26 | 0.9 | 1.1 | 1.5 | 5.1 | 5.8 | 6.4 | - | - | - | - |
| n28 | 0.9 | 1.0 | 1.4 | 2.1 | 4.0 | 7.6 | - | - | - | - |
| n71 | 1.0 | 1.2 | 1.3 | 4.9 | 5.5 | 6.2 | 6.8 | - | - | - |

Proposal:

The RSD of n5, n8, n26, n28, n71 for PC2 FDD are proposed highlighted as Table 3 and Table 4.

Conclusion

In this contribution, we presented the analysis condition and RSD calculation results for PC2 FDD band.

Proposal:

The RSD of n5, n8, n26, n28, n71 for PC2 FDD are proposed highlighted as Table 3 and Table 4.

Table 3 Reference sensitivity degradation for 1Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n5 | 0.9 | 1.0 | 0.7 | 4.8 | 5.3 | - | - | - | - | - |
| n8 | 0.8 | 0.8 | 0.5 | 4.9 | - | - | 6.8 | - | - | - |
| n26 | 0.7 | 0.9 | 0.9 | 4.8 | 5.3 | 5.8 | - | - | - | - |
| n28 | 0.9 | 0.8 | 1.0 | 1.6 | 2.8 | 4.5 | - | - | - | - |
| n71 | 1.0 | 1.1 | 0.9 | 3.9 | 4.2 | 4.4 | 4.6 | - | - | - |

Table 4 Reference sensitivity degradation for 2Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n5 | 1.0 | 1.2 | 1.3 | 5.2 | 5.9 | - | - | - | - | - |
| n8 | 1.3 | 1.3 | 1.4 | 5.5 | - | - | 7.7 | - | - | - |
| n26 | 0.9 | 1.1 | 1.5 | 5.1 | 5.8 | 6.4 | - | - | - | - |
| n28 | 0.9 | 1.0 | 1.4 | 2.1 | 4.0 | 7.6 | - | - | - | - |
| n71 | 1.0 | 1.2 | 1.3 | 4.9 | 5.5 | 6.2 | 6.8 | - | - | - |

References

[1]	RP-222625 “New WID: High power UE for FR1 for FDD single band(s) with power class 2”, China Unicom, 3GPP TSG-RAN Meeting #97-e, Online, September 12th – 16th, 2022

[2] 3GPP TS 38.101-1 V17.7.0