3GPP TSG-RAN WG4 Meeting #106                                          R4-2302046

Athens, Greece, Feb 27 – Mar 03, 2023

Source: 	Huawei, HiSilicon

Title: 	On Rel-18 NR SL evolution

Agenda Item:	9.29.2

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#98e as the updated scope for the topic of NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#99) - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication) - The work is limited to FR1 licensed spectrum and ITS band in FR1. - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel. - Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only. - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] - The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI. - The exact frequency band combinations for both FR1 licensed and ITS-dedicated spectrum are to be determined based on company input during the WI. - Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements. Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum

In this contribution, we provide our consideration and initial analysis on RAN4 work for Rel-18 SL evolution based on the revised WID and the progress of RAN1 until now.

Discussion

Scope of RAN4 work

CA operation

According to the WID objective, supporting CA for SL is on hold till RAN#99:

Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#99)

In LTE V2X, the requirements of intra-band contiguous multi-carrier operation bandwidth class B and class C were defined in addition to the single carrier V2X case. It is noted that it is a full set of Tx and Rx requirements in order to support the multi-carrier operation. Similarly, if the NR sidelink CA operation is to be specified in RAN4, another set of requirements needs to be defined in addition to that of Rel-16/17 V2X/Sidelink. Before that, the input on the sidelink CA physical layer signal/channel design from RAN1 is essential to specify the related requirements in RAN4 RF. We should be clear whether the objective will be completed in Rel-18, considering this part is put on hold until further checking in the next RAN Plenary. Moreover, some other requirements, such as SL-U, con-current transmission of SL-U and licensed Uu, are more likely to be defined, resulting in the too heavy workload of this topic. Thus, the research on the requirements of Sidelink CA operation should be delayed until the completion of the design in RAN1. If there is conclusion in RAN#99 that SL CA is included in Rel-18, in order to enable the mechanism defined by RAN1 and RAN2, necessary feedback could be provided by RAN4 upon the requests from other WGs, but whether to complete the full set of RF requirements in Rel-18 should take other enhanced aspects into consideration. If the workload of RAN4 cannot support to finish all related requirements, including SL-U, co-channel co-existence together with SL CA, RAN4 need to consider the priority for all aspects to be studied in Rel-18, and make a decision whether requirements of some of them should be deferred to next release.

Proposal 1: The research on the requirements of Sidelink CA operation should be delayed until the completion of the design in RAN1. After that, priority on specifying requirements for all affected features in Rel-18 should be checked and decided.

Operating bands

In the WID objective, the unlicensed bands that sidelink focused on are band n46 and n96/n102. Table below lists the frequency range of these unlicensed bands, which includes 5GHz band and 6GHz bands.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n10214 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| NOTE 13: This band is restricted to operation with shared spectrum channel access as defined in 37.213. NOTE 14: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. |  |  |  |

In the objective, Uu operation for mode 1 is limited to licensed spectrum only, which means we need to consider to define the requirements for sidelink operation on unlicensed band rather than that of Uu operation on licensed bands. It is noted that NR-U requirements have separate clauses in the specification as the requirements are obviously distinct from those of licensed bands. On the other hand, requirements for SL defined in Rel-16 and Rel17 are mainly targeted for V2X scenario, but SL for unlicensed bands may not be aimed for V2X, thus a specific suffix for SL-U would be appropriate in TS38.101-1 for specifying the novel requirements. Table 1 Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation(CA) with UL MIMO |
| I | RedCap |
| J | Sidelink on shared spectrum |

According to the WID scope, FR2 spectrum is not considered in Rel-18 RAN4 discussion.

Proposal 2: It is proposed to have a separate suffix in the spec structure to facilitate specifying requirements for “sidelink on shared spectrum”.

Proposal 3: FR2 is not considered by RAN4 in Rel-18 according to the WID.

Con-current operation

Apart from the operation of SL on unlicensed spectrum, the WI objective also mentioned that Uu is limited to licensed spectrum:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

which means for SL mode 1, con-current operation needs to be considered in Rel-18 from RAN4 perspective. The definition of con-current operation is reproduced as below:

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Since SL and Uu are operated at unlicensed and licensed spectrum respectively, only inter-band con-current operation need to be considered for this scenario. As usual, such con-current operation relies on specific band combinations, which should be based on inputs from operators. Until requests received in the group, the requirements can be on hold for the time being.

Proposal 4: The inter-band con-current operation for sidelink and NR Uu should be considered later when specific band combinations are requested.

Co-channel coexistence

For co-channel coexistence in Rel-18, TDM-base and FDM-based resource pool are assumed in RAN1 discussion. In the case of FDM-based semi-static resource pool shared by LTE-V and NR-SL, simultaneous transmissions of LTE and NR for co-channel coexistenceis not an attractive scenario from implementation perspective. If there is no sufficient justification of performance gain for such scenario for a single UE, RAN4 only consider TDM manner for transmission of LTE sidelink and NR sidelink in ITS spectrum. Howver, simultaneous reception for both LTE-SL and NR-SL in the same channel is possible, which should be considered in Rel-18 for RAN4 study. An draft LS in [5] is to inform RAN1 and RAN2 that only TDM transmission and simultaneous reception of LTE-SL and NR-SL are considered in RAN4.

Proposal 5: Send an LS to RAN1 to confirm that for co-channel coexistence in Rel-18, only TDM transmission and simultaneous reception of LTE-SL and NR-SL are considered in RAN4.

Besides co-channel co-existence, for adjacent channel co-existence in unlicensed spectrum, we think that there is no difference between unlicensed spectrum and licensed spectrum from coexistence perspective. It is not necessary to reconsider the evaluation for adjacent channel co-existence for unlicensed spectrum in Rel-18.

Proposal 6: No need to consider evaluation for adjacent channel coexistence for unlicensed spectrum in Rel-18.

SL supporting unlicensed spectrum

System parameters

Waveform

The interlace-based resource allocation, where a 20 MHz is sliced into multiple interlaces, of each contains a number of distributed RBs with equal space, is introduce in NR-U due to the regulation requirement on OCB. To fulfill the requirements when operating on unlicensed spectrum, the interlace-based resource allocation, as illustrated in Figure 1, can be reused in SL-U. There is impact on MPR of the interlace resource allocation.

Observation 1: The resource allocation in SL-U is interlace-based to satisfy the requirement of PSD in unlicensed spectrum.

Proposal 7: The impact on MPR from interlace-based resource allocation should be considered in Rel-18.

Figure 1 Illustration of interlace based sub-channel and resource pool [2]

Channel BW

In the previous discussion, for the sidelink on ITS spectrum and licensend spectrum, the maximum channel bandwidth is 40MHz.

Table 2 LTE V2X Communciation channel bandwidth

| E-UTRA V2X band / V2X channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 |  |  |  | Yes |  | Yes |

Table 3 NR V2X operation channel bandwidths for each operating band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| n14 | 15 | Yes | Yes |  |  |  |
|  | 30 |  | Yes |  |  |  |
|  | 60 |  |  |  |  |  |
| n38 | 15 |  | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |
| n47 | 15 |  | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |
| n79 | 15 |  | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |

Table 4 Channel bandwidths for unlicensed band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n46 | 15 |  | 105 |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n96 | 15 |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n102 | 15 |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

As for the sidelink operation on unlicensed band, it is not necessary to consider such restriction on channel bandwidth. The unlicensed spectrum can be used in the short distance communication with wider channel bandwidth, which could require larger channel bandwidth for certain scenarios.

Proposal 8: The restriction on max channel bandwidth of sidelink operation in ITS and licensed spectrum is not applicable for unlicensed spectrum.

Channel raster and Sync raster

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. For SL operates on licensed band, the RF reference frequency for an RF channel is not changed, which is based on the global frequency channel raster. Thus the channel raster defined for unlicensed spectrum can be reused for the sidelink operation on unlicensed band.

In Rel-16/17, there is no synchronization raster definition for NR V2X for both licensed bands and unlicensed bands. The reason is that the frequency location for S-SSB is (pre-) configured.

Since the SL-U S-SSB design is still under discussion in RAN1, whether to introduce definition of synchronization raster for sidelink operation on licensed spectrum is up to the outcome of RAN1 decision. If the S-SSB is still pre-configured, the conclusion in Rel-16/17 can be reused.

Proposal 9: The channel raster defined for unlicensed spectrum can be reused.

Proposal 10: Whether to define synchronization raster for SL-U is up to RAN1’s conclusion based on S-SSB design in Rel-18.

Wideband operation

In Rel-16, both DL and UL transmission on a wideband carrier (>20MHz) in unlicensed spectrum was discussed. The “out-of-BWP” leakage requirement, in-carrier leakage and blocking requirement were considered. It is recommended that such requirements to be considered in SL-U.

Proposal 11: The requirements on wideband operation defined in Rel-16 can be reused in Rel-18.

Power class

Power class 5 is supported for the NR-U spectrum band of n96 and n102. In Rel-16/17, the allowed NR V2X UE maximum output power is specified with PC3 and PC2. In the Rel-18 SL-U, power class 5 should be considered as the first priority.

Proposal 12: Power class 5 should be considered firstly for the sidelink operation in unlicensed spectrum.

Tx requirements

MPR/A-MPR

Since the interlace-based resource allocation is introduce for Rel-18 SL-U because of the requirement for PSD and OCB in unlicensed spectrum, the MPR should be re-evaluated for the case with the recommended simulation assumptions as shown below.

Table 5 MPR simulation assumption for SL-U

| parameter | Assumption |
| --- | --- |
| center frequency | 5.9GHz |
| Bandwidth | 10/20/30/40/100MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM with unlicensed |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PSD | 10 dBm/MHz |
| Resource allocation | Interlace-based |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For NR-U, the UE operating shall meet additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 of TS 38.101-1 when NS is signaled and when transmission overlaps with any portion of the specified frequency range. In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies. The PSD limitation is also the restriction for SL-U. Therefore, A-MPR for SL-U also needs to be studied and specified.

Proposal 13: It is proposed to agree on the MPR simulation assumptions for SL-U as in Table 5.

Proposal 14: It is proposed to identify the required additional requirements due to maximum mean transmission power density according to regulations for SL-U, and specify the A-MPR requirements for SL-U if needed.

Rx requirements

REFSENS

According to [4], the V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where the target SNR: -0.5 dB. The required SNR for NR-U is different from that of NR-V2X. For REFSENS of SL-U in unlicensed spectrum, SNR should be decided firstly for all affected Rx requirements. In our view, SNR is mainly determined by waveform rather than the spectrum for the operation. Hence, the SNR for V2X should be reused for defining SL-U REFSENS for different CBW.

Proposal 15: It is proposed to reuse V2X SNR to define the REFSNES of SL-U for the supported CBW.

Requirements comparison

From the first release of NR specification, NR V2X and NR-U requirements are introduced in Rel-16 respectively. Some requirements of NR V2X and NR-U are reused from normal NR, but some of them are different due to specific physical layer design as well as the restrictions from regulations. For SL operation in unlicensed spectrum, requirements for both NR V2X and NR-U should be referred. The group can focus on MPR/A-MPR for Tx and REFSENS for Rx firstly, and further consider other requirements after thorough comparison of difference between NR V2X and NR-U. Furthermore, some requirements may also rely on the study progress in RAN1. If dependence on the PHY design is identified during specifying the requirements, such kind of information should be informed to RAN1 timely.

Proposal 16: Difference between NR V2X and NR-U should be compared firstly to make it clear of the direction to specify the requirements of SL-U in the unlicensed bands.

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-18 SL evolution with the following observation and proposals:

Proposal 1: The research on the requirements of Sidelink CA operation should be delayed until the completion of the design in RAN1. After that, priority on specifying requirements for all affected features in Rel-18 should be checked and decided.

Proposal 2: It is proposed to have a separate suffix in the spec structure to facilitate specifying requirements for “sidelink on shared spectrum”.

Proposal 3: FR2 is not considered by RAN4 in Rel-18 according to the WID.

Proposal 4: The inter-band con-current operation for sidelink and NR Uu should be considered later when specific band combinations are requested.

Proposal 5: Send an LS to RAN1 to confirm that for co-channel coexistence in Rel-18, only TDM transmission and simultaneous reception of LTE-SL and NR-SL are considered in RAN4.

Proposal 6: No need to consider evaluation for adjacent channel coexistence for unlicensed spectrum in Rel-18.

Observation 1: The resource allocation in SL-U is interlace-based to satisfy the requirement of PSD in unlicensed spectrum.

Proposal 7: The impact on MPR from interlace-based resource allocation should be considered in Rel-18.

Proposal 8: The restriction on max channel bandwidth of sidelink operation in ITS and licensed spectrum is not applicable for unlicensed spectrum.

Proposal 9: The channel raster defined for unlicensed spectrum can be reused.

Proposal 10: Whether to define synchronization raster for SL-U is up to RAN1’s conclusion based on S-SSB design in Rel-18.

Proposal 11: The requirements on wideband operation defined in Rel-16 can be reused in Rel-18.

Proposal 12: Power class 5 should be considered firstly for the sidelink operation in unlicensed spectrum.

Proposal 13: It is proposed to agree on the MPR simulation assumptions for SL-U as in Table 5.

Proposal 14: It is proposed to identify the required additional requirements due to maximum mean transmission power density according to regulations for SL-U, and specify the A-MPR requirements for SL-U if needed.

Proposal 15: It is proposed to reuse V2X SNR to define the REFSNES of SL-U for the supported CBW.

Proposal 16: Difference between NR V2X and NR-U should be compared firstly to make it clear of the direction to specify the requirements of SL-U in the unlicensed bands.

- References

[1] RP-222806 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #98-e, December. 12-16, 2022

[2] R1-2203907, On physical channel design framework for sidelink on FR1 unlicensed spectrum, Samsung, 3GPP TSG RAN WG1 #109-e, May 9th – 20th, 2022

[3] 3GPP TR 38.885 v16.0.0 Study on NR Vehicle-to-Everything (V2X) (Release 16)

[4] 3GPP TR 38.886 v16.3.0 V2X Services based on NR; User Equipment (UE) radio transmission and reception; (Release 16)

[5] R4-2302047, draft LS on co-channel coexistence, Huawei, Hisilicon, 3GPP TSG RAN WG4 #106, Feb 27th – March 3rd , 2023

- Annex (RAN1 agreements)

At RAN1 meeting #109-e, the following agreements were made regarding sidelink on unlicensed spectrum:

Agreement Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel. - FFS conditions for the actual channel access type(s) used for each SL channel and signal transmitted, and based on COT sharing conditions (if supported) - FFS whether UL CAPC or DL CAPC or both should be used as the baseline, - FFS how the channel access priority classes apply to each SL channel and signal - FFS sidelink priority levels (PQI or L1 priority), channel and signal mapping to the 4 channel access priority classes. The discussion may involve other WGs. Agreement - UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U). - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access and any restrictions (e.g. whether the COT can be shared with a single UE or multiple UEs) - FFS all other details in compliance with the regulatory requirements - CP extension (CPE) is supported for NR sidelink operation in a shared channel. - FFS all remaining details including applicable scenarios, usage, PHY structure, etc. Agreement Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2 Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions - FFS: whether/how to address IBE (In Band Emission) impact Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission (if supported), at least the following candidates can be discussed: - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - FFS: Other resource allocation granularity, e.g., RB-level - 1 sub-channel equals K interlaces if sub-channel is supported - FFS details - Other candidates are not precluded - FFS: mapping of PSCCH to frequency resources - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc. Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure

And the following agreements were made regarding co-channel coexistence for LTE sidelink and NR sidelink:

Agreement For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed. Agreement For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model. - FFS: UE dropping model Agreement For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C). Agreement For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions. Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. Agreement For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported.

At RAN1 meeting #110, the following agreements were made regarding sidelink on unlicensed spectrum:

Agreement The following evaluation scenario can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT in a shared channel. - Scenario 1 (commercial use cases) – recommended: - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates. - Indoor layout - Option 1: a pairs topology for SL-U from R1-2205033 – recommended - a = 20m, b = 60m, c = 20m, d = 80 m - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one is Wi-Fi or NR-U. - For NR-U / Wi-Fi, the same number of UEs / Wi-Fi STA as the total number of SL-U devices are dropped in the area. The NR-U UE / Wi-Fi nodes are dropped uniformly per gNB/AP per 20 MHz. - Companies should report if they used a different number of UEs / Wi-Fi STA as the total number of SL-U devices, as an additional evaluation scenario. - For evaluation of unicast traffic, the topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area. - Companies should report how SL-U UEs are paired - 6 SL-U pairs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - For evaluation of groupcast traffic, SL-U UEs are dropped uniformly at random in the area, SL-UEs form groupcast UE group based on TX-RX UE distancing, the distance is provided by each company. - Companies should report how SL-U UEs form a group - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - For evaluation of broadcast traffic, SL-U UEs are dropped uniformly at random in the area. - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - Option 2: SL UE clusters (R1-2203146) - Indoor layout and UE dropping model with N = 3 or 6 clusters and each with M=5 UEs - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m - No overlapping among the N clusters - For coexistence, there are two operators to model two RATs at a time, where the red one is Wi-Fi AP or NR-U gNB. NR-U UE / Wi-Fi STA are dropped uniformly per gNB/AP. - Simulation bandwidth can be larger than 20MHz (e.g., 80MHz) - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889) - Traffic model - Option 1: R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FFS whether/how the PDB requirement can be captured - Option 2: FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - Option 3: XR cloud gaming model in TR38.838 - FFS whether/how the PDB requirement can be captured - It is up to each company to use either Option 1 or 2 or Option 3 or mixed of them - Interference model: - Layout option 1: Explicit modelling of NR-U / WiFi transmissions (as per TR38.889) - Note, for the interference traffic model: - The same or equivalent traffic model setting as SL-U should be used as much as possible to achieve equal load (e.g., SL-U RAT offered load equal the interfering RAT’s offered load). - The same number of traffic flows should be used between SL-U and the interfering RAT (e.g., 10 UEs with 10 flows, and 5 STAs with 2 flows each, one for DL and one for UL) - Companies should report if they used a different assumption, as an additional evaluation scenario. - Performance metric: UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure. - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation - FFS for groupcast and broadcast - Fair coexistence criterion between SL-U and the interfering RAT (e.g., according to NR-U TR38.889) Agreement - CW adjustment - NR-U DL CW adjustment mechanism is used as the baseline for SL-U when SL-HARQ feedback is enabled in SCI for unicast - FFS any necessary update for SL-U operation - FFS: how to determine CW size when SL-HARQ feedback is disabled in SCI - FFS the case of groupcast option 1 (NACK-only) and groupcast option 2 Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details Agreement - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s) - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

And the following agreements were made regarding co-channel coexistence for LTE sidelink and NR sidelink:

Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

At RAN1 meeting #110bis-e, the following agreements were made regarding sidelink on unlicensed spectrum:

Agreement - Type 1 SL channel access procedure is applicable to the following transmissions by a UE: - PSSCH/PSCCH transmission(s) scheduled or configured by a gNB in SL Mode 1 resource allocation. - PSSCH/PSCCH transmission(s) from the UE in SL Mode 2 resource allocation. - Other SL transmissions including S-SSB and PSFCH transmissions from a UE - FFS: how to set CAPC for S-SSB and PSFCH - Note: Type 1 can be used to initiate a COT - A UE uses a channel access priority class applicable to the sidelink user plane data multiplexed in PSSCH for performing the Type 1 channel access procedures to transmit transmission(s) including PSSCH with user plane data and its associated PSCCH. - Note: how to set CAPC for MAC CE multiplexed in PSSCH is up to RAN2 - A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds the maximum COT duration where the channel access procedures are performed based on a channel access priority class p ppassociated with the UE transmissions, as given in CAPC table for SL. Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation Agreement For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms.. Agreement - RAN1 is to study the definition of a “SL reference duration” following the NR-U principle and RAN1 is to agree on the definition before down-selection to an option for CW adjustment for SL HARQ-ACK feedback enabled/disabled and each cast type - In Type 1 SL channel access procedure, further study the following cases and options. Other options are not precluded. - CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled): - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value. - Option 5: If a collision indicator is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - FFS: the (pre-)configuration ratio values - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class ∈1,2,3,4 CWp=CWmin,p; otherwise CWp is increased. - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: CWp is reset to CWmin,p for every priority class p∈1,2,3,4. - Option B: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p, otherwise CWp is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then CWp=CWmin,p, otherwise CWp is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p ; otherwise CWp is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration). Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH Agreement To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS: - Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH - Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3) Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer Agreement At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement Regarding additional candidate S-SSB occasions: - Their number and time domain locations are (pre-)configured or pre-defined

And the following agreements were made regarding co-channel coexistence for LTE sidelink and NR sidelink:

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement For dynamic resource pool sharing, where the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, continue studying the following alternatives: - Alt 1: The LTE SL module provides the NR SL module with the candidate information (excluding at least the candidate resource sets SA or SB) - The NR SL module identifies a set of resources based on information shared by the LTE SL module. - FFS: how to identify the set of resources - The NR SL module excludes these identified resources from its own candidate resource set when performing the resource (re)selection procedure. - The exclusion process is performed in the PHY layer. - Note: implementation of Alt 1 should not have specification impact to LTE - Alt 2: The LTE SL module provides the NR SL module with the candidate resource sets SA or SB shared by the LTE SL module - The LTE PHY SL module is provided information from the higher layer to generate a candidate resource set SA or SB. The resource set SA or SB is then shared to NR SL module. - The NR SL module performs an intersection operation with the candidate resource set received from the LTE SL module and the candidate resource set generated by the NR SL module. - FFS: how to handle the case where this results in an insufficient set of resources - The intersection operation is performed in the MAC layer. - FFS: How to handle NR V2X parameter settings that are not supported by LTE V2X, e.g., periodicities, sub-channel sizes, etc - Note: implementation of Alt 2 should not have specification impact to LTE - In the next meeting strive to decide between the two alternatives Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed

At RAN1 meeting #111, the following agreements were made regarding sidelink on unlicensed spectrum:

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure Agreement Performance metric, company to report which one of the following options is evaluated in their simulation results. - Option 1: - For GC and BC, a device within the range (a, b) from the TX can be a receiver, and the UPT/latency/PRR can be calculated by average. The packet whose delay exceeding the remaining PDB as transmission failure. - Option 2: - For GC, UPT and latency for a packet is measured from the perspective of the worst-case RX (i.e., the one with the longest transmission time). - For BC, UPT and latency for a packet are measured for each RX separately. - Option 3: - For GC and BC, UPT, latency and PRR are measured from the perspective of each RX UE Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets Agreement SL reference duration is defined as a duration corresponding to a channel occupancy initiated by the UE including transmission of PSSCH(s), starting from the beginning of the channel occupancy initiated by the UE including transmission of PSSCH(s), until either (one option to be selected later): - Option 1a: - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 1b: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 2a: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled if it is transmitted, otherwise until the end of the channel occupancy - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 2b: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled if it is transmitted, otherwise until the time when UE updates the CW - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT.When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE. Agreement - If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p. - If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4. Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets

And the following agreements were made regarding co-channel coexistence for LTE sidelink and NR sidelink:

Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.