3GPP TSG-RAN WG4 Meeting #106                                R4-2302098

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.872 to capture MSD evaluation for CA_n5-n28

Agenda Item:	9.2.3.2

Document for:	Approval

Introduction

In this paper, we provide the following text proposals to capture the MSD evaluation for CA_n5-n28 referring to the WF [1].

- References

[1] R4-2220508, WF on UE architecture and other aspects for CA_n5-n28, vivo

- Text proposal

- <<Start of Change for TR 38.872>>

- 5.2.2.5	REFSENS evaluation

Referring to contribution R4-2202036, the n28 MSD evaluation for CA_n5-n28 is shown as below.

Figure 5.2.2.5-1 the n28 MSD evaluation for CA_n5-n28

Based on the agreed CR R4-2206134, one MSD test configuration (DL n28 5MHz) can be a reference considering the impacts from band n5 UL.

Table 5.2.2.5-1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

- <<Next of Change>>

- 5.2.3.3	REFSENS evaluation

Since there is no two UL IMD issue for CA_n5-n28, no need to specify the MSD due to two UL IMD interference. Currently, the interference from band n28 UL with 25 RB (15kHz SCS) UL configuration has been considered when -78.5dBm REFSENS was specified for band n28 30MHz. Thus, based on the band n28 30MHz 78.5dBm REFSENS, we can further consider the impacts from band n5 UL. Referring to contribution R4-2202036, 2.9 dB MSD can be observed from measurements results. One MSD test configuration (DL n28 30MHz) can be a reference as below considering the impacts from both band n5 UL and band n28 UL.

Table 5.2.3.3-1 MSD due to two UL band cross band interference for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

- <<End of Change>>