3GPP TSG-RAN WG4 Meeting #106                                R4-2302102

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.872 to complete the open issues for CA_n5-n8

Agenda Item:	9.2.2.2

Document for:	Approval

Introduction

In this paper, we provide the following text proposals to capture the remaining parts for CA_n5-n8.

- References

[1] R4-2220507, WF on UE architecture and other aspects for CA_n5-n8, Qualcomm Inc.

- Text proposal

- <<Start of Change for TR 38.872>>

- 5.1.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n8, the TIB,c and RIB,c are given in the tables below considering both 2-antenna and 3–antenna implementations. If 3-antenna implementation is assumed, there isn’t any additional insertion loss observed for CA_n5-n8 compared with single carrier operation. If it’s assumed to derive the the delta Tib and Rib requirements for CA_n5-n8 based on 3–antenna implementation, all the values can be zero. If 2-antenna implementation is assumed to specify these requirements, the existing delta Tib and Rib values for CA_n8-n20 can be reused for CA_n5-n8, i.e. 0.4 Tib for each band and 0 Rib for each band.

Table 5.1.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] for 2-antenna implementation | ΔTIB,c [dB] for 3-antenna implementation |
| --- | --- | --- | --- |
| CA_n5-n8 | n5 | 0.4 | 0 |
|  | n8 | 0.4 | 0 |

Table 5.1.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | 0 |
|  | n8 | 0 |

- <<Next of Change>>

- 5.1.2.5	REFSENS requirements

For full band n5 and n8 RF filters implementation, the scheduling is restricted as DL_n5-n8_UL_n5 and the concurrent operation between DL band n5 and UL band n8 can’t work. There is no MSD for band n8 DL due to band n5 UL interference since RF components (band n5 duplexer) can provide necessary isolation to protect band n8 DL frequency range.

For dedicated RF filters implementation with partial frequency range, the MSD due to cross band isolation between DL band n5 and UL band n8 need to be further studied based on the isolation performance assumption between band n5 DL and band n8 UL.

- <<Next of Change>>

- 5.1.3	Specific for 2 bands UL of CA_n5-n8

- 5.1.3.0 General

Only dedicated RF filter(s) implementation with partial frequency range can support DL_n5-n8_UL_n5-n8 feature. And full band n5 and n8 RF filters implementation can’t support DL_n5-n8_UL_n5-n8 feature. Thus, the following technical analysis is based on dedicated RF filter(s) implementation.

- 5.1.3.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n8.

- 5.1.3.2	UE co-existence studies

Table 5.1.3.2-1 lists Band n5 + Band n8 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.3.2-1: Band n5 and Band n8 UL IMD products with frequency range restriction

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 69 | 91 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 2585 | 2632 | 2665 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1601 | 1877 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3420 | 3536 | 3580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 138 | 3456 | 3500 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2781 | 2436 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1042 | 697 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4440 | 4495 | 4200 | 4255 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4360 | 4415 | 4280 | 4335 |

Based on Table 5.1.3.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

Table 5.1.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.1.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n8 | E-UTRA Band 1, 28, 31, 34, 38, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

- 5.1.3.3	REFSENS requirements

Based on the assumption of restricted frequency range for CA_n5-n8, there is no MSD issue due to intermodulation interference.

- 5.1.4	Feasible RF implementations

Referring to clause 5.1.1 UE RF architecture assumption, it can be observed that two sets of different RF implementations are included currently:

One is full band n5 and n8 RF filters implementation.

Another one is dedicated RF filters implementation with partial frequency range.

These two implementation may have different RF requirements and scheduling restriction. In study phase, we list all the implementations on the table and further trade off each implementation as below.

Table 5.1.4-1: Trade-off between two feasible RF implementations.

| Type of implementation | Pros | Cons |
| --- | --- | --- |
| Full band n5 and n8 RF filters implementation | - Current RF components can be reused easily. - The same RF filters can support both single carrier operation and CA operation. - UE cost may not be improved too much. | - UL_n5-n8 can’t be supported. Only DL_n5-n8_UL_n5 can be supported. Scheduling restriction can be observed. |
| dedicated RF filters implementation with partial frequency range | - DL_n5-n8_UL_n5-n8 can be supported by UE within partial frequency range. Operators’ demands can be met. - There is no scheduling restriction for CA operation, but the specific partial frequency range. | - At least, one n8 filter with UL partial frequency range need to be developed for this specific CA operation. - Dedicated filter and full band filter need to be switched between single carrier operation and CA operation. |

As both implementations are allowed and UE can support them optionally, one solution is needed to further distinguish these two implementations. The following options can be further discussed.

- Option 1: New bands would be required

- Option 2: New bands would not be required. One capability can be introduced to distinguish these two kinds of implementation.

- Option 3: Other

- <<End of Change>>