3GPP TSG-RAN WG4 Meeting # 106	R4-2302177

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0344 | rev | 0 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Updates to precoder configuration for PDSCH, PDCCH and SDR tests in TS 38.101-4 (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon, Qualcomm, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In PDSCH common test parameter Table, PDSCH and PDCCH precoding configuration is generally indicated to randomly selected with each i1, i2 combination. However, there is no i1, i2 defined for 2Tx codebook based on RAN1’s design. Same issues on PDCCH common test parameter Table and SDR test parameter Table. Precoder configuration for PDCCH, PDSCH, PDSCH DMRS is not clearly captured in the specification |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that for 2Tx, the precoder is randomly selected form Table 5.2.2.2.1-1 rather than variable i1, i2 Clarify precoder configuration with number of Tx = 1; Clarify random precoder behavior in time and frequency |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The confusion will still exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3, 5.5, 5.5A, 7.2, 7.3, 7.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

Table 5.2-1: Common test parameters

-----------------Start of Change 1---------------------

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI state |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12].; |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1:Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |

-----------------End of Change 1---------------------

## 5.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 5.3-1 are valid for all PDCCH tests unless otherwise stated.

-----------------Start of Change 2---------------------

Table 5.3-1: Common test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| [… omitting unchanged cells …] |  |  |
| PDCCH Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| [… omitting unchanged cells …] |  |  |

----------------- End of Change 2 ---------------------

## 5.5A	Sustained downlink data rate provided by lower layers

### 5.5A.1	FR1 CA requirements

[…]

----------------- Start of Change 3 ---------------------

Table 5.5A-1: Common test parameters for FDD and TDD component carriers

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL 1 for 30 kHz / 5 MHz 1/AL4 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 1/AL 8 for other combinations |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI State |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For Number of Tx = 2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 For Number of Tx= 4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12].; |
| [… omitting unchanged cells …] |  |  |  |

-----------------End of Change 3---------------------

## 7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

-----------------Start of Change 4---------------------

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | 0 |
|  | Number of PRBs in CORESET |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI state |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection with Wideband size and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |

-----------------End of Change 4---------------------

## 7.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 7.3-1 are valid for all PDCCH tests unless otherwise stated.

-----------------Start of Change 5---------------------

Table 7.3-1: Common test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| [… omitting unchanged cells …] |  |  |
| PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| [… omitting unchanged cells …] |  |  |

-----------------End of Change 6---------------------

## 7.5A	Sustained downlink data rate provided by lower layers

### 7.5A.1	FR2 CA requirements

[…]

-----------------Start of Change 6---------------------

Table 7.5A.1-1: Test parameters for FR2 TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | Table 7.5A.1-2 |
|  | Number of PDCCH candidates and aggregation levels |  | 1/8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1-1 |
|  | TCI State |  | TCI state #1 |
|  | PDCCH &PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 |
| [… omitting unchanged cells …] |  |  |  |