3GPP TSG-RAN4 Meeting #106	R4-2302518

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	9.16.3

Source:	Keysight Technologies

Title:	On Phantom Testing and QZ Sizes

Document for:	Approval

# Introduction

This contribution discusses FR1 MIMO OTA testing when using phantoms.

# Discussion

In [1], various discussions were held on the introduction of phantoms for FR1 MIMO OTA testing, but no concrete conclusion was made as summarized in the WF [2]

Issue 3-1-1: General views on FR1 MIMO OTA for smartphone in browsing mode Agreements: - Further study the necessity and feasibility of the FR1 MIMO OTA test methodology enhancement for Smartphone in browsing mode using hand phantom.

Subsequently, additional discussions were held in RAN4#105 regarding the need for hand phantoms and whether the 20 cm QZ is sufficient to test phantoms. No conclusion was reached whether hand phantoms should be introduced for NR FR1 MIMO OTA testing [3]

Issue 2-1-1: General views - Proposals - Proposal 1: It is necessary to enhance the FR1 MIMO OTA test methodology for smartphone with hand phantom to evaluate and validate the real performance of smartphone in browsing mode. (CAICT, OPPO) - Proposal 2: It is feasible to use hand phantom for FR1 MIMO OTA test enhancement while further detail can be studied during the Rel-18 timeline. (Xiaomi, OPPO) - Proposal 3: Collect more measurement data from different UEs and different DUT positions to study the FR1 MIMO OTA test methodology enhancement for smartphone in browsing mode using hand phantom. (CAICT, OPPO) - Proposal 4: RAN4 should conclude that the Smartphone browsing mode using hand phantom for MIMO OTA testing should not be defined. The group should gather efforts on developing performance requirement for important low-frequency NR bands. (vivo, Samsung) - Proposal 5: The OTA performance can be fully verified with the combination of free space MIMO OTA + phantom TRP TRS. (Samsung, Apple) Tentative Agreement: - Further study the necessity and feasibility of FR1 MIMO OTA test methodology enhancement for smartphone in browsing mode using hand phantom.

Additionally, two options on the QZ requirements for NR FR1 MIMO OTA testing with phantoms were discussed, also without any conclusion [3].

Issue 2-1-2: Quite zone size for FR1 MIMO OTA testing with phantoms - Proposals - Proposal 1: Current anechoic chamber configuration is applicable to MIMO OTA test in browsing mode because the quiet zone can encompass hand phantom and test zone size is not impacted by phantom. (Huawei) - Proposal 2: Increase the test zone for NR FR1 MIMO from 20 cm to 30 cm when considering phantoms. (Keysight) Agreement: - FFS on the feasibility of reusing the existing quite zone size for FR1 MIMO OTA testing with hand phantoms - According to the existing WID, increasing quite zone size is out of scope.

Phantoms have recently been included in the NR FR1 TRP/TRS test methodology [4] and requirements [5] in 3GPP but have been are integral part in CTIA where the phantoms have been developed years ago and been part of the test procedures and requirements for many years [6]; all phantom testing assumed a 30 cm QZ. It should also be pointed out that a 30 cm QZ/test zone was selected LTE MIMO OTA testing [10]; that scope included phantoms as well. Hence, all 3GPP and CTIA OTA testing of smartphones with phantoms has relied on a 30 cm QZ [4][6][10].

Observation 1: Only the 30 cm QZ/test zone has been defined for OTA testing with phantoms for smartphone devices in 3GPP and CTIA.

Smartphones were considered the key device type for NR FR1 MIMO OTA [7] and since the priority was FS testing, only a 20 cm QZ/test zone was considered [8]. This test zone size became an integral part of the NR FR1 2D MPAC architecture with 16 uniformly spaced probes. An overview of the NR FR1 (20 cm test zone with 16 uniform probe config) and LTE (30 cm test zone with 8 uniform probe config) weighted RMS correlation errors was presented in Table 1 of [9]. Clearly, the correlation error in the 20 cm test zone NR FR1 system is significantly worse than that in the 30 cm test zone LTE system (yellow highlighted values). Further extending the test zone without adjusting the number or placement of probes would further degrade the correlation errors.

Table 1: NR FR1 and LTE Weighted RMS Correlation Errors (Option 6) Channel Model; Weighted RMS Correlation Error (Option 6). NR FR1 @3.5GHz and 2.31test zone size; NR FR1 @7.125GHz and 4.8test zone size; LTE @ 1test zone size. 8 Sectorized; 16 Uniform; 8 Sectorized; 16 Uniform; 8 Uniform. SCME UMa; N/A; N/A; N/A; N/A; 0.032. SCME UMi; N/A; N/A; N/A; N/A; 0.047. CDL-A (UMi); 0.02; 0.12; 0.07; 0.29; N/A. CDL-C (UMa); 0.08; 0.08; 0.08; 0.2; N/A.

In [11], it was argued

that test zone size is not altered or associated change is not noticeable by the presence of phantom. This means that there is no need to increase the number of probes in the anechoic chamber when a hand phantom is present. In other words, current requirements on anechoic chambers are equally applicable in browsing mode.

However, this argumentation is not applicable here as the quoted reference document found that a 30 cm test zone LTE MIMO OTA system does not require an increase of uniformly placed probes from 8 to 16, i.e., the baseline system already supported a 30 cm with a relatively small correlation error in the test zone. It can therefore not be argued that work confirms that a 20 cm test zone NR FR1 MIMO OTA system will readily support phantoms without any increase of probes/test zone.

If phantoms are considered for NR FR1 MIMO OTA testing, the WID must be adjusted to increase the quiet zone/test zone size from 20 cm to 30 cm to match the quiet zone for FR1 TRP/TRS testing and to include the phantoms inside the test zone. In the absence of increasing the QZ from 20 cm to 30 cm, phantoms should not be considered for NR FR1 MIMO OTA testing.

Proposal 1: Adjust the WID to augment the existing 20 cm QZ/test zone with a 30 cm QZ if phantoms are considered for NR FR1 MIMO OTA testing.

Proposal 2: In the absence of introducing another QZ/test zone, do not consider phantoms for NR FR1 MIMO OTA testing.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Only the 30 cm QZ/test zone has been defined for OTA testing with phantoms for smartphone devices in 3GPP and CTIA.

Proposal 1: Adjust the WID to augment the existing 20 cm QZ/test zone with a 30 cm QZ if phantoms are considered for NR FR1 MIMO OTA testing

Proposal 2: In the absence of introducing another QZ/test zone, do not consider phantoms for NR FR1 MIMO OTA testing.

# References

- R4-2217501, Email discussion summary for [104-bis-e][327] NR_MIMO_OTA_enh, CAICT, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 2022

- R4-2217461, WF on NR MIMO OTA enhancement, CAICT, 3GPP TSG-RAN WG4 Meeting #104bis-e, October 2022

- R4-2220612, WF on NR MIMO OTA enhancement, CAICT, 3GPP TSG-RAN WG4 Meeting # 105, November 2022

- 3GPP TR 38.834, Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1;Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology

- 3GPP TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

- CTIA Certification, Test Plan for Wireless Device Over-the-Air Performance

- 3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- R4-1904183, Proposals on NR MIMO OTA, CAICT, SAICT, 3GPP TSG-RAN WG4 Meeting #90bis, April 2019

- R4-1912103, Weighted RMS Correlation Errors for NR FR1 2D MPAC, Keysight Technologies, 3GPP TSG-RAN WG4 #92bis, October 2019

- 3GPP TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE)

- R4-2218073, on MIMO OTA tests in browsing mode, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 105, November 2022