3GPP TSG-RAN WG4 Meeting # 106	R4-2302556

# Athens, Greece, Feb 27 – Mar 03, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1440 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1 on cleanups for V2X operating bands and channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LTE_V2X_PC5_combos-Core |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Notes in Table 5.3E.2-1 and Table 5.3E.2-2 are mismatching and should be corrected. Furthermore, some other cleanups need to be done for V2X operating bands and V2X channel bandwidth, e.g., in Table 5.2E.2-1 the unnecessary line in the table should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Cleanups in Table 5.2E.2-1. - Correct notes in Table 5.3E.2-1 and Table 5.3E.2-2. - Unify table formats in Table 5.3E.1-1 as other tables for channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect information will remain in the spec for V2X. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E.1, 5.2E.2, 5.3E.1, 5.3E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.2E	Operating band for V2X

5.2E.1	V2X operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 5.2E.1-1.

Table 5.2E.1-1 V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n142 | 788 MHz | - | 798 MHz | 788 MHz | - | 798 MHz | HD | PC5 |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| n79 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. Note 2: When this band is used for public safety service, the NR band is operated with both in-coverage scenarios and out-of-coverage scenarios. |  |  |  |  |  |  |  |  |

5.2E.2	V2X operating bands for con-current operation

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1 and Table 5.2E.2-2.

Table 5.2E.2-1 Inter-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n1-n47 | n1 | Uu |
|  | n47 | PC5 |
| V2X_n5-n47 | n5 | Uu |
|  | n47 | PC5 |
| V2X_n8-n47 | n8 | Uu |
|  | n47 | PC5 |
| V2X_n39-n47 | n39 | Uu |
|  | n47 | PC5 |
| V2X_n40-n47 | n40 | Uu |
|  | n47 | PC5 |
| V2X_n41-n47 | n41 | Uu |
|  | n47 | PC5 |
| V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |
| V2X_n78-n47 | n78 | Uu |
|  | n47 | PC5 |
| V2X_n79-n47 | n79 | Uu |
|  | n47 | PC5 |

Table 5.2E.2-2 Intra-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n79-n79 | n79 | Uu |
|  | n79 | PC5 |

<< Unchanged sections omitted >>

5.3E	Channel bandwidth for V2X

5.3E.1	General

NR V2X operation channel bandwidths for each operating band are specified in Table 5.3E.1-1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path. The maximum channel bandwidth for SL operation in licensed band is 40MHz.

Table 5.3E.1-1 NR V2X operation channel bandwidths for each operating band

| NR band / SCS / UE Channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS (kHz) | 5 | 10 | 20 | 30 | 40 |
| n14 | 15 | 5 | 10 |  |  |  |
|  | 30 |  | 10 |  |  |  |
|  | 60 |  |  |  |  |  |
| n38 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n47 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n79 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |

5.3E.2	Channel bandwidth for V2X concurrent operation

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band are specified in Table 5.3E.2-1.

Table 5.3E.2-1: Inter-band con-current operation configurations

| NR V2X inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| V2X_n1A-n47A | n1 | Uu | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n5A-n47A | n5 | Uu | 5, 10, 15, 20, 25 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n8A-n47A | n8 | Uu | 5, 10, 15, 20, 35 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n39A-n47A | n39 | Uu | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n40A-n47A | n40 | Uu | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n41A-n47A | n41 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n71A-n47A | n71 | Uu | 5, 10, 15, 20 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n78A-n47A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n79A-n47A | n79 | Uu | 40, 50, 60, 80, 100 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

For NR V2X intra-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band are specified in Table 5.3E.2-2.

Table 5.3E.2-2: Intra-band con-current operation configurations

| NR V2X intra-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| V2X_n79B | n79 | Uu | 40, 50, 60, 80, 100 | 0 |
|  | n79 | PC5 | 10, 20, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

==============================================================

<< End of changes >>