3GPP TSG-RAN WG4 Meeting # 106												R4-2302935

Athens, Greece, 27 February –03 March, 2023

Agenda item:			9.6.7

Source:	Huawei, HiSilicon

Title:	Ad-hoc minutes for FR1 RF enhancement WI demodulation

Document for:	Approval

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Topic #1: Demouldation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300131 | China Telecom | Proposal 1: Cover CA scenario for 8Rx demodulation and CSI requirements, and we are fine to first focus on single CC as first priority. Proposal 2: Cover 16Tx for 8Rx demodulation and CSI requirements. Observation 1: If we use the number of CSI-RS ports as large as more than 8/16 for PDSCH requirements, there will be challenges to the correct PDSCH decoding due to higher data rate on the CSI-RS slots. Proposal 3: For PDSCH demodulation requirements for more than 4Tx, cover PDSCH Mapping Type A and CSI-RS overlapped with PDSCH requirements. Observation 2: xoverhead is 0 for all legacy 2Rx and 4Rx PDSCH demodulation requirements except for the LTE-NR coexistence requirements where UE needs to perform rate matching to the LTE CRS only. Proposal 4: Use xoverhead = 0 for 8Rx requirements as well. |
| R4-2300211 | MediaTek | Proposal 1: Do not define PDCCH requirements for 8Rx UE. Observation 1: For the scenario of rank larger than 4, the DMRS symbols will collide with TRS symbols when TRS located in symbol#{6, 10}. Proposal 2: Configure TRS symbols in symbol#{5, 9} when the number of rank larger is than 4. Proposal 3: Define PDSCH requirements with 2T8R (2layers), 4T8R (4layers) and 8T8R (8layers). Proposal 4: Use MCS16, 2T8R (2layers), TDLC30-100 to define PDSCH requirements for vehicular use case. Proposal 5: Define PDSCH requirements with MIMO correlation ULA low Proposal 6: Define 8Rx SDR requirements under assumption of additional DMRS. |
| R4-2300290 | Apple | Observation#1: PDCCH is not a bottleneck channel in 8Rx when defining requirements for PDSCH Proposal#1: Do not define PDCCH demodulation requirements for 8Rx Proposal#2: Wait for 8Rx RF core requirements to be stablished before any FDD alignment activities Proposal#3: Maintain focus on single-carrier PDSCH. Only work on CA requirements if time permits Observation#2: PDSCH Mapping Type-B is an optional capability Proposal#4: Focus requirements on PDSCH Mapping Type-A only Proposal#5: For test metric, only consider 70% of maximum throughput Proposal#6: Use ULA Low and ULA Medium as antenna correlation scenarios Observation#3: In TDLA30-10 channel and ULA Low antenna correlation and Rank 4, all 64QAM Table MCS values show an acceptable SNR at 70% target throughput Proposal#7: Use MCS19 for Rank 4 over TDLA30-10 ULA Low Observation#4: In TDLA30-10 channel and ULA Medium antenna correlation, only Rank 2 shows acceptable SNR at 70% target throughput, and MCS13 reaches maximum throughput. Proposal#8: Use MCS13 for Rank 2 over TDLA30-10 ULA Medium Proposal#9: RAN4 to study the feasibility of 256QAM with 8 MIMO layers Proposal#10: For 1024QAM, RAN4 to study feasibility, including maximum achievable MIMO layers and MCS Proposal#11: Further study maximum MCS for scaling factor 1 with maximum MIMO layers for each modulation order Proposal#12: For DMRS configuration, use the same configuration as PDSCH demodulation requirements for SDR. Proposal#13: Do not define PMI requirements for 8Rx Proposal#14: Do not define RI requirements for 8Rx |
| R4-2300612 | Nokia | Proposal 1:RAN4 to introduce UE demodulation requirements for both TDD and FDD for 8Rx Proposal 2:RAN4 to introduce 8 Rx UE demodulations requirements for PDCCH. Proposal 3:RAN4 to include both PDSCH type A and type B in 8 Rx UE demodulation requirements Observation 1:Either OTA testability needs to be enhanced to support 8 Rx for UE demodulation requirements, or testing needs to be constrained to conducted and hybrid modes. Proposal 4:RAN4 to test 8RX at least in conducted mode, and RAN4 to discuss the OTA testability for 8 Rx case Proposal 5:Include at least one rank from {5,6,7} with corresponding applicability rules. Proposal 6:RAN4 to keep Rank 2, Rank4 and Rank8 for the 8 Rx UE demodulation requirements. Observation 2:The UE demodulation will be required to handle 2 codewords in case for number of layers higher than four. Proposal 7:RAN4 to define two different MCS for each requirement for rank higher than 4. MCS values and channel model to be defined accordingly. Proposal 8:RAN4 to discuss the spatial channel models to be used for 8 Rx requirements with number of layers >4 to ensure accurate requirements. Proposal 9:Discuss if modulation order 6 is testable in 8 Rx conducted Proposal 10:RAN4 to specify 8 Rx requirements considering 2 Tx antennas for Rank 2. 4 Tx for Rank 4, and 8 Tx antennas for Rank 6 and Rank 8. Proposal 11:Use as evaluation metrics the SNR at which the PDSCH achieves 70% and 30% of throughput to test 8 Rx PDSCH performance. Proposal 12:Define 8 layers requirements using a modulation order lower than 6. Proposal 13:Only define 1024QAM requirements with number of layers <=4. Proposal 14:define the same DMRS configuration used for PDSCH requirement for the SDR requirements. Proposal 15:RAN4 to extend the PMI and Rank reporting requirements for 8 Rx. |
| R4-2300613 | Nokia | Provide simulation results |
| R4-2300830 | Samsung | Proposal 1: Antenna configuration for rank2, 4 and 8 Rank2: 2T8R Rank4: 4T8R Rank8: 8T8R Proposal 2: Use ULA Low and ULA medium A as test correlation matrix. Proposal 3: MCS used for the initial simulation assumptions: Rank2: MCS19 Rank4: MCS13 and MCS17 Rank8: MCS4, MCS13 and MCS17 Proposal 4: For rank 8, the same MCS level for two codewords should be defined. Proposal 5: Support option 1 (Yes, define 8 Rx requirements with two layers, low MCS, channel model with substantial Doppler spread for vehicular use case.) Proposal 6: Use option 1 (Option1: 70% of maximum TP) as 8Rx PDSCH demodulation test metric. Proposal 7: Use N1=4 and N2=1 for 8 ports case initial simulation results alignment. Proposal 8: For UE capable of 64QAM and 256QAM, RAN4 to study the feasibility of 64QAM and 256QAM with 8 MIMO layers. Proposal 9: For UE capable of 1024QAM, option 1 (RAN4 to study the feasibility of 1024QAM with 4 MIMO layers). |
| R4-2300831 | Samsung | Provide simulation results |
| R4-2301050 | ZTE | Proposal 1: To define ULA low correlation for 8 Rx. Proposal 2: To define MCS 13 for 8 Rx PDSCH . Proposal 3: To use the same MCS for two codewords. Proposal 4: To consider TDLC-30-100 low channel model for vehicle use case. |
| R4-2301894 | Ericsson | Provide simulation results |
| R4-2301893 | Ericsson | Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR1 with: TDD mode as a priority Number of receive antennas: 8Rx Modulation order: Up to 1024QAM Rank: Up to 8 DL MIMO layers. Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with: FR1 TDD n77/ n78: fc = 3.5 GHz FR1 TDD: SCS = 30 kHz, CBW = 40 MHz and 106 RBs. Channel models: TDLA and TDLC (30 ns delay spread) Max Doppler frequencies: up to 100 Hz (vehicular). Proposal 3: Align the RF parameters to the RF enhancements in the core part, and consider as a starting point Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM Proposal 4: Define the 8 Rx UE demodulation requirements for PDSCH Type A only. Proposal 5: Define the 8 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only. Proposal 6: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup. Proposal 7: Referring to the Table in Proposal 6, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow FR1 TDD, 40 MHz/30 kHz, Rank > 4, Metric: 70% Peak Throughput FR1 TDD, 40 MHz/30 kHz, Rank = 4, Metric: 70% Peak Throughput FR1 TDD, 40 MHz/30 kHz, Rank = 2, Metric: 70% Peak Throughput Proposal 8: Define PDCCH demodulation requirements for 8 Rx UE in FR1 with the following test setup. FR1 TDD, 40 MHz/30 kHz Using the common test parameters Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 8 PDSCH MIMO layers using 8 Rx are considered. Proposal 9: Define SDR requirements applicable to FR1 CC, considering Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’ Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2. Proposal 10: Do not define new PMI reporting requirements for 8Rx UE in FR1. Proposal 11: Do not define RI reporting for 8 Rx UE. |
| R4-2302349 | Qualcomm Incorporated | Proposal 1: RAN4 to use MCS13 for 8 layers, 8Rx simulation assumptions Proposal 2: Don’t have a PDSCH grant in special slot for 8 layers, 8Rx Proposal 3: RAN4 to use MCS 17 for 4 layers, 8Rx simulation assumptions Proposal 4: 2 Layers, 8Rx demod requirements are not essential Proposal 5: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 4 layers, 8Rx Proposal 6: Symbol 4, 8 shall be locations of CSI-RS for tracking in 8 layers, 8Rx Proposal 7: Choose row 6 or 7 of Table 7.4.1.5.3-1 of 38.211 specification [4] for 8 layers, 8Rx Proposal 8: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 8 layers, 8Rx Proposal 9: Further discuss 1024 QAM and corresponding TxEVM requirements Proposal 10: Do not define demod requirements for PDCCH, and PBCH |
| R4-2302169 | Huawei, HiSilicon | Proposal 1: Take TDD as 1st priority for the demodulation requirements discussion. Proposal 2: Use 3% Tx EVM value for 256QAM Proposal 3: RAN4 still focuses on key issues on single CC requirements. CA requirements can be introduced easily by applying per CC requirements if needed. Proposal 4: Not introduce 8RX PDCCH requirements Proposal 5: Use contents listed in Table 2-1 as specification structure for 8RX UE and CSI requirements Proposal 6: Not introduce performance requirements for PDSCH mapping type B for 8Rx Proposal 7: Configure equals to 0 Proposal 8: Only consider 70% of max TP requirements Proposal 9: Only consider TDLA30-10 for PDSCH requirements definition. Proposal 10: For cases with rank>4, only consider same MCS for two codewords. Proposal 11: Consider following two cases for 8RX PDSCH requirements: - Case 1: 8T8R, Rank8, TDLA30-10 Low, MCS16 (MCS Table 1, 16QAM, 616/1024) - Case 2: 2T8R, Rank2, TDLA30-10 medium B MCS19 (MCS Table 1, 64QAM, 517/1024) Proposal 12: Consider following configuration: - Define SDR requirements for 64QAM with rank 8 up to MCS 24 - Define SDR requirements for 256QAM with rank 8 up to MCS 23 - Define SDR requirements for 1024QAM with rank 4 up to MCS 23 Proposal 14: Define following applicability rules For PDSCH requirements: - Case1&Case2: For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.2.3.2 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3: For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.2.2.2 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier tests specified in 5.2.3.2 with 4Rx except cases listed in Table 5.2.3.1.1-7 (Enhanced Receiver Type 1), 5.2.3.2.16 (MMSE-IRC with inter cell interference), 5.2.3.2.17(MMSE-IRC with intra cell inter user interference), 5.2.3.2.18(CRS-IM with scenario1) and 5.2.3.2.19 (CRS-IM with scenario2) are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 1.5 dB less than the number specified for 4Rx tests. For PDCCH requirements: - Case1&Case2:For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3:For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.3.2.2 except 5.3.2.2.4 (RedCap) with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. Same requirements specified with 4Rx should be applied. For SDR, CSI requirements - For 8RX capable UEs, only apply 8RX tests and no need to apply 2RX and 4RX test. |
| R4-2301362 | DOCOMO | Proposal 1: Define UE demodulation requirements for 8-by-8 MIMO with 1 or 2 MIMO layers Proposal 2: Define UE demodulation requirements for 8-by-8 MIMO with 8 MIMO layers. |

Open issues summary

Sub-topic 1-1 General

Issue 1-1-1: Whether to consider CA requirements

Agreements in RAN4#105 R4-2220613: - Focus on single CC for PDSCH and SDR requirements as 1st priority FFS on the CA requirements

- Proposals

  - Option 1: Cover both CA and single CC scenarios and focus on single CC as first priority (China Telecom, Huawei)

  - Option 2: Maintain focus on single-carrier PDSCH. Only work on CA requirements if time permits (Apple)

- Recommended WF

Agreement:

  - Focus on single CC performance requirements definition now

  - Discuss CA scenario after RF session agrees to define any CA band combination(s) for 8Rx

Issue 1-1-2: Duplex mode

- Proposals

  - Option 1: Focus on TDD and consider FDD if there’s progress in the RF session (Apple, Huawei, Ericsson)

  - Option 2: Cover both FDD and TDD (Nokia)

- Recommended WF

  - TBA

Discussion:

Samsung: Option 1 is agreed in last meeting.

Nokia: Focus on TDD and then define requirements for FDD when RF session has conclusion for FDD

Apple: If RF session does not cover them.

CTC: support both FDD and TDD from coverage POV, but Nokia’s proposal is fine.

Agreement:

Firstly Focus on TDD and then define requirements for FDD when RF session has conclusion for FDD

Issue 1-1-3: EVM

- Proposals

  - Option 1: Consider following configurations: (Ericsson, Huawei)

  - Tx EVM = 6% for up to 64QAM

  - Tx EVM = 3% for 256QAM

  - Tx EVM = 2.5% for 1024QAM

- Recommended WF

  - Option 1 is agreeable

Agreement:

Option 1 is agreed

Issue 1-1-4: Whether to define PDCCH requirements

- Proposals

  - Option 1: Yes (Nokia, Ericsson, CTC)

  - Option 2: No (MTK, Apple, Qualcomm, Huawei, Samsung, ZTE)

- Recommended WF

  - TBA

Discussion:

Ericsson: We can conducted simulation, if SNR is low with TDLA, we check the cases with TDLC/CDL channel. Regular PDCCH cases are defined, we need to check it for 8Rx by following the channel for PDSCH.

RF is discussing the aggregation level for PDCCH, can keep it FFS and check.

Apple: Prefer to not define requirement for PDCCH. No point to define requirement for it. Even with other channel model with higher SNR, always we can do it for higher SNR value. With PDSCH requirements, the PDCCH performance can be guaranteed.

Nokia: Shall define PDCCH requirements for 8Rx, to find reasonable SNR value, such as change the channel model and AL.

QC: AL 8 is under discussion in RF, -6dB for 4Rx, PDCCH is not the bottle neck, no need to define requirements. Meaningful PDCCH throughput can be achieved by higher SNR.

Samsung: Not define requirements, the PDCCH always low rank, if 8Rx used, the SNR always lower.

Huawei: Not define requirements. UE can choose to not use 8 antenna receivers for PDCCH, maybe 2 or 4 antennas for PDCCH, considering the robust of PDCCH. RF for REFSEN, with AL8 and 1 layer considered, low SNR for it. Demod with high rank will have higher SNR.

Nokia: 4Rx PDCCH is defined even it is not the bottleneck. No technical reason not to define requirements.

ZTE：Agree with Samsung and Huawei. No need to define requirements.

MTK: Would like not to define requirements. 1 layer is not related to MIMO correlation.

Samsung: lower than -6dB SNR can be tested or not?

CTC: Cover PDCCH requirements. Ok to do evaluation with other channel model.

Ericsson: The main reason is the low SNR value. We can find way to get higher SNR. No technical reason.

Nokia: TP is lower than 4Rx?

Agreement:

Keep it FFS for this meeting.

Issue 1-1-5: PDCCH simulation assumptions (If agreed)

- Proposals

  - Option 1: (Ericsson)

    - FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 4: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

  - Other options.

- Recommended WF

  - This issue pends on discussion of Issue 1-1-4

Issue 1-1-6: OTA testability

- Proposals

  - Option 1:  RAN4 to test 8Rx at least in conducted mode, discuss the OTA testability for 8Rx (Nokia)

- Either OTA testability needs to be enhanced to support 8 Rx for UE demodulation requirements, or testing needs to be constrained to conducted and hybrid modes.

- Recommended WF

  - Moderator: 8Rx is applicable for FR1, only define conducted performance requirements?

Agreements:

Only define conducted performance requirements for 8Rx

Issue 1-1-7: Applicability rules

- Proposals

  - Option 1: Consider following test applicability rules

- Case1: with both 2Rx and 4Rx supported RF bands - Case2: without 2Rx supported RF bands but with 4Rx supported RF bands - Case3: with 2Rx supported RF bands but without 4Rx supported RF bands - Case4: without both 2Rx and 4Rx supported RF bands, i.e. with only 8Rx supported RF bands Test applicability rules Table Antenna port# in the supported RF bands; 2Rx test; 4Rx test; 8Rx test. Case1; ×; √; √. Case2; ×; √; √. Case3; √; ×; √. Case4; ×; √; √. For PDSCH requirements: - Case1&Case2: For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.2.3.2 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3: For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.2.2.2 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier tests specified in 5.2.3.2 with 4Rx except cases listed in Table 5.2.3.1.1-7 (Enhanced Receiver Type 1), 5.2.3.2.16 (MMSE-IRC with inter cell interference), 5.2.3.2.17(MMSE-IRC with intra cell inter user interference), 5.2.3.2.18(CRS-IM with scenario1) and 5.2.3.2.19 (CRS-IM with scenario2) are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 1.5 dB less than the number specified for 4Rx tests. For PDCCH requirements: - Case1&Case2:For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3:For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.3.2.2 except 5.3.2.2.4 (RedCap) with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. Same requirements specified with 4Rx should be applied. For SDR, CSI requirements - For 8RX capable UEs, only apply 8RX tests and no need to apply 2RX and 4RX test.

- Recommended WF

  - Discuss whether option 1 is acceptable

Sub-topic 1-2 PDSCH requirements

Issue 1-2-1: Antenna configuration

Agreements in RAN4#105 R4-2220613: MIMO layers - Focus on Rank 2, 4, 8 as first priority for the feasibility study and initial simulation alignment purpose. - Other options not precluded - FFS PDSCH requirements definition coverage on number of MIMO layers Antenna configuration - Option 1 - Rank 1,2: 2T8R - Rank 3,4: 4T8R - Rank 5,6,7,8: 8T8R - Other options not precluded

- Proposals

  - Option 1: 16Tx (China Telecom)

  - Option 2: 2Tx: Rank 1, 2; 4Tx: Rank 3,4; 8Tx: Rank 6 and 8 (Nokia)

  - Option 3: 2Tx: Rank 2; 4Tx: Rank 4; 8Tx: Rank 8 (Samsung, MTK, Qualcomm)

  - Option 4: 8T8R (NTT DOCOMO)

  - Option 5: 2Tx: Rank2; 4Tx: Rank4; 8Tx: Rank6 and Rank8 (Ericsson)

  - Option 6: 2Tx: Rank2; 8Tx: Rank8 (Huawei)

- Recommended WF

  - Based on majorities’ view:

- Rank 1 and Rank 2: 2T8R

- Rank 3 and Rank 4: 4T8R

- Rank 6 and Rank 8: 8T8R

- FFS 16Tx

Discussion:

CTC: TDD with 16TX which is popular. Check other companies’ view

Samsung: What’s the meaning of introduction of 16Tx? It is the number of antenna port or physical antenna number?

Agreement:

- Rank 1 and Rank 2: 2T8R

- Rank 3 and Rank 4: 4T8R

- Rank 6 and Rank 8: 8T8R

Issue 1-2-2: MIMO layers

- Proposals

  - Option 1: Rank 2,4,8 (MTK)

  - Option 2: Rank 2 and 4 (Apple)

  - Option 3: Rank 2,4,8 and at least one rank from {5,6,7} (Nokia)

  - Option 4: Rank 1 or 2 and Rank 8 (NTT DOCOMO)

  - Option 5: Rank 2,4,6 and 8 (Ericsson)

  - Option 6: Rank 4 and 8 (Qualcomm)

  - Option 7: Rank 2 and 8(Huawei)

- Recommended WF

  - TBA

Discussion:

  - Option 1: 2, 4 (Apple)

  - Option 2: 2, 8 (Huawei, NTT DOCOMO, Qualcomm, MTK, Apple)

  - Option 3: 4, 8 (Qualcomm, MTK, Samsung)

  - Option 4: 2, 4, 8 (MTK, Samsung, CTC)

  - Option 5: 2, 4, 6, 8 (Nokia, Ericsson, CTC)

Qualcomm: LTE defined extended rule for 2Rx/4Rx cases for 8Rx, Rank 2 or 4 can be tested by existing requirements.

Ericsson: Lower rank. Rank 2, 4, 8 is agreed in last meeting. Recommend Rank 2/4/8 as tentative agreement.

Nokia：Rank 2, 4 and 8.  35dB SNR for Rank 8, how about the testing issue?

CTC: 4Rx includes Rank 1/2/3/4 cases. It is new UE type for 8Rx.

Samsung: Include Rank 4 that is typical. Rank 4 for CQI is static channel.

Huawei: Rank 2 is the lowest UE capability for support different rank. Only consider Rank 2 and 8. Rank 4 has been considered in CQI test. Focus on Higher layers, the inter-layer interference is more important. Low correlation is considered. Medium correlation is more reasonable for Rank 2.

MTK: Considering MIMO correlation, prefer Rank 2 and 8.

Agreement:

Define requirements for Rank 8 with Low correlation.

FFS for Rank 2, 4 and 6.

Issue 1-2-3: Antenna correlation

| Correlation Model | α | β |
| --- | --- | --- |
| Low correlation | 0 | 0 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |
| High Correlation | 0.9 | 0.9 |

- Proposals

  - Option 1: ULA Low (MTK, ZTE, Ericsson, Qualcomm?)

  - Option 2: ULA Low and ULA Medium (Apple)

  - Option 3: ULA Low and ULA Medium A (Samsung)

  - Option 4: ULA Low and ULA Medium B (Huawei)

- Recommended WF

  - ULA Low is for Rank 8

  - Further discuss ULA Medium, ULA Medium A and ULA Medium B correlation.

Discussion:

Rank 8：ULA Low

Huawei/QC: Rank 4/8 ULA Low only, Rank 2 with ULA Medium B

Samsung: Rank 8 ULA Low and ULA Medium B. Rank 2/4 ULA Low and ULA Medium A

Apple: The definition of ULA Medium A and B should be clarified.

Ericsson: Option 1, only ULA Low

Agreement:

Rank 8: ULA Low, FFS ULA Medium B

Rank 2: ULA Medium A

Rank 4: ULA Low, FFS ULA Medium A

Rank 6: ULA Low

Issue 1-2-4: Propagation conditions

- Proposals

  - Option 1: TDLC300-100 for Rank2 and TDLA30-10 for Rank 4 and 8. (MTK, Ericsson, ZTE, Huawei, CTC)

  - Option 2: Only TDLA30-10 (Huawei, Apple, QC)

  - Option 3: TDLA and TDLC with 30ns RMS (Ericsson)

  - Option 4: Discuss the spatial channel models to be used for 8 Rx requirements with number of layers >4 to ensure accurate requirements (Nokia)

  - Option 5: TDLC300-100 and TDLA30-10 for Rank2 and TDLA30-10 for Rank 4 and 8. (Samsung)

- Recommended WF

  - TBA

Discussion:

Nokia: Prefer to consider Beamforming model on top of TDL channel

Samsung: Need clarification for consider beamforming model.

Agreement:

For evaluation purpose:

TDLC300-100 for Rank2 and TDLA30-10 for Rank 4 and 8.

FFS on consideration of beamforming model on top of TDL channel model

Issue 1-2-5: One MCS configuration

- Proposals

  - Option 1:  MCS19 for Rank4 and MCS13 for Rank2.(Apple)

  - Option 2:  Rank 8 to have requirements with 64QAM or lower. (Nokia)

  - Option 3:  Rank2: MCS19, Rank4: MCS13 and MCS17, Rank8: MCS4, MCS13 and MCS17 (Samsung)

  - Option 4: Only MCS13 (ZTE)

  - Option 5: Rank 2: MCS2, MCS7, MCS13 and MCS26; Rank 4: MCS4, MCS13 and MCS26; Rank 6: MCS13 and MCS20; Rank 8: MCS4, MCS13 and MCS17 (Ericsson)

  - Option 6: Rank4: MCS17; Rank8: MCS13 (Qualcomm)

  - Option 7: Rank2: MCS19; Rank8: MCS16 (Huawei)

- Recommended WF

  - Discuss this issue after issues 1-2-1 ~ 1-2-4 are finalized.

Issue 1-2-6: Two MCS configuration for Rank>4

- Proposals

  - Option 1: Configure two MCSs for Rank>4 and discuss the spatial channel model to be used to guarantee different precoding gain for each layer (Nokia)

  - Option 2: Only consider single MCS (Huawei, Samsung, ZTE, Qualcomm)

- Recommended WF

  - Option 2 is agreeable?

Discussion:

Qualcomm: challenge for different MCS testing

Agreement: FFS

Issue 1-2-7: Overhead

Agreements in RAN4#105 R4-2220613: - Option 1: 6 - Other options not precluded

- Proposals

  - Option 1: 0 (China Telecom, Huawei, Ericsson)

  - Option 2: TBA

- Recommended WF

  - Confirm Option1?

Agreement:

Option 1, i.e. Overhead = 0 is agreed.

Issue 1-2-8: Mapping Type

Agreements in RAN4#105 R4-2220613: Applicable test scenarios - Focus on PDSCH mapping type A as first priority - FFS PDSCH mapping type B

- Proposals

  - Option 1: Only Type A (Apple, Huawei, Ericsson)

  - Option 2: Both Type A and Type B (Nokia)

- Recommended WF

  - TBA

Agreement:

Include Type A, FFS type B

Issue 1-2-9: TRS configuration

Agreements in RAN4#105 R4-2220613: - For rank>4: DMRS Type 1 with 2+2 with symbol# 2,3,10,11 - For rank<=4: DMRS Type 1 with 1+1 with symbol# 2,10

- Proposals

  - Option 1: Configure TRS symbols in symbol #{5, 9} for rank larger than 4 (MTK)

- For the scenario of rank larger than 4, the DMRS with symbol#2,3,10,11 will collide with TRS symbols when TRS is located in symbol#{6, 10}.

  - Option 2: Configure TRS symbols in symbol #{4, 8} for 8 layers (Qualcomm)

- Recommended WF

  - Either Option 1 or Option 2 is agreeable?

Agreement:

Option 1 is agreed: Configure TRS symbols in symbol#{5, 9}

Issue 1-2-10: NZP-CSI-RS configuration

- Proposals

  - Option 1: Choose row 6 or 7 of Table 7.4.1.5.3-1 of 38.211 specification [4] for 8 layers, 8Rx.(Qualcomm)

  - Other options.

- Recommended WF

  - TBA

Agreement:

NZP CSI-RS configuration of row 6 is agreed for 8 layers.

Issue 1-2-11: Whether consider scenarios for CSI overlapping with PDSCH for CSI-RS ports larger than 4Tx

- Proposals

  - Option 1: Yes (China Telecom)

- If we use the number of CSI-RS ports as large as more than 8/16 for PDSCH requirements, there will be challenges to the correct PDSCH decoding due to higher data rate on the CSI-RS slots

  - Option 2: No

- Recommended WF

  - FFS consider scenario with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx

Issue 1-2-12: Special slot

- Proposals

  - Option 1:  Don’t schedule PDSCH in special slot for 8 layers cases (Qualcomm)

- Recommended WF

  - Option1 is agreeable?

Agreement:

Option 1 is agreed: Don’t schedule PDSCH in special slot for 8 layers cases.

Issue 1-2-13: N1 and N2 configuration for 8Tx case

- Proposals

  - Option 1:  Use (N1,N2)=(4,1) (Samsung)

  - Other options

- Recommended WF

  - TBA.

Discussion:

Samsung/MTK: For 8 CSI-RS ports, there are two (N1, N2) configurations, it is still meaningful for ULA antenna configuration. For Type I single panel random PMI selection, what is the codebook for 8 ports?

Apple: XP is used for PMI cases with (N1, N2) configuration. XP is used for 4Rx PDSCH requirements for LTE; NR use ULA for PDSCH requirements.

Samsung: Without (N1, N2) configuration, how to select the random precoding for 8Tx?

Agreement:

Further discuss configuration of (N1,N2)=(4,1) for 8Tx case for Type I random precoding selection.

Issue 1-2-14: Test metric

Agreements in RAN4#105 R4-2220613: - Option1: 70% of maximum TP - Option 2: 70% and 30% of maximum TP - Option 3: Other options are not precluded

- Proposals

  - Option 1: Only 70% of maximum TP (Apple, Samsung, Ericsson, Huawei, CTC, Nokia)

  - Option 2: 70% and 30% of maximum TP (China Telecom, Nokia)

- Recommended WF

Agreement:

Only 70% of maximum TP

Issue 1-2-15: Dedicated test cases for vehicular devices

Agreements in RAN4#105 R4-2220613: Whether to define dedicated test cases for vehicle devices - Option 1: Yes, define 8 Rx requirements with two layers, low MCS, channel model with substantial Doppler spread for vehicular use case. - Option 2: No

- Proposals

  - Option 1: Yes

  - Option 1a: Use MCS16, 2T8R (2layers), TDLC300-100. (MTK)

  - Option 1b: Two layers, low MCS, channel model with substantial Doppler spread (Samsung)

  - Option 1c: TDLC300-100 Low channel model. (ZTE,)

  - Option 1d: Use TDLC300-100 for Rank 2 with MCS7 Table 2 (16QAM, CR = 0.48) (Ericsson)

  - Option 1e: Use TDLB100-400 (Nokia)

  - Option 2: No (Huawei)

- Recommended WF

  - TBA

Discussion:

Nokia: TDLB100-400 for vehicular device, bring up this issue for higher Doppler shift.

Samsung: How to conduct the testing for vehicular device without UE capability.

QC: There is existing PDSCH test cases for TDLB100-400 for Rank 1.

Agreement:

Taken into account the agreed channel model, FFS introduction of TDLB100-400 channel model for vehicular device

Sub-topic 1-3 SDR requirements

Issue 1-3-1: Whether to configure additional DMRS symbol

- Proposals

  - Option1: Yes (MTK, Apple, Nokia, Qualcomm)

  - Other options

- Recommended WF

Agreement:

Option1 is agreed.

Issue 1-3-2: Modulation order for MIMO 8 layers

- Proposals

  - Option 1: Define 8 layers requirements using a modulation order lower than 6 (Nokia)

  - Option 2: Define 8 layers requirements using modulation order up to 6 (Ericsson, Nokia)

- Recommended WF

  - TBA

Discussion:

Nokia: Same view with Option 2.

Moderator: Remove Option 1 and add Nokia to support Option 2.

Issue 1-3-3: Feasibility of 64QAM and MIMO layers

- Proposals

  - Option 1: Further study the feasibility of 64QAM with 8 MIMO layers. (Samsung)

  - Option 2: 8 MIMO layers, maximum MCS 27 (Ericsson)

  - Option 3: 8 MIMO layers, maximum MCS 24 (Huawei)

  - Option 4: It is feasible to define 8 MIMO layer for 64QAM (Nokia, Ericsson, Samsung)

- Recommended WF

  - TBA

Discussion:

Moderator: Add Option 4 as per discussion for Issue 1-3-2.

Ericsson: Support Option 4, can remove Option 2.

Samsung: Support Option 4.

Agreement:

It is feasible to define SDR test for 8 MIMO layer for 64QAM, FFS MCS.

Issue 1-3-4: Feasibility of 256QAM and MIMO layers

- Proposals

  - Option 1: Further study the feasibility of 256QAM with 8 MIMO layers including maximum achievable MCS for scaling factor 1.(Apple, Samsung)

  - Option 2: For 4 MIMO layers, max MCS 26 (scaling factor =1) (Legacy configuration defined in existing spec). For 8 MIMO layers: max MCS26 (scaling factor =1). (Qualcomm)

  - Option 3: Consider 8 MIMO layers, max MCS23 (Huawei)

- Recommended WF

Discussion:

Huawei: 4 MIMO layers use the legacy test cases; 8 MIMO layer: new test cases

Ericsson: 6 layers for 256QAM, with 8x8 antenna configuration.

Agreement：

  - Evaluate SDR test cases for 2, 4 and 8 MIMO layers for 8Rx; FFS 6 MIMO layers

    - 2 and 4 MIMO layers use the existing MCS value defined in Table 5.5A-5 of TS 38.101-4

    - 8 MIMO layers for the max achievable MCS

    - use 8x8 for 6 MIMO layers if agreed, avoid precoder that results in zeroes on any Rx branches

Issue 1-3-5: Feasibility of 1024QAM and MIMO layers

- Proposals

  - Option 1: Study feasibility of the maximum achievable MIMO layers and MCS. (Apple)

  - Option 2:  MIMO layers <= 4 (Nokia)

  - Option 3: Further study the feasibility of 1024QAM with 4 MIMO layers. (Samsung)

  - Option 4:  2 MIMO layers with up to MCS24 (Ericsson)

  - Option 5: Further discuss 1024 QAM and corresponding TxEVM requirements. (Qualcomm)

  - Option 6: 4 MIMO layers with up to MCS23 (Huawei)

- Recommended WF

  - More simulation is needed

Issue 1-3-6: MCS look-up Table

- Proposals

  - Option 1 (MTK)

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | upper bound MCS (without additional DMRS) | upper bound MCS (with additional DMRS) | practical MCS | requirement MCS |
| --- | --- | --- | --- | --- | --- | --- |
| 8 | 8 | 1 | 27 | 27 | TBD | TBD |
| 8 | 8 | 0.8 | 23 | 27 |  | TBD |
| 8 | 8 | 0.75 | 22 | 27 |  | TBD |
| 8 | 8 | 0.4 | 12 | 16 |  | TBD |
| 8 | 6 | 1 | 28 | 28 | TBD | TBD |
| 8 | 6 | 0.8 | 25 | 28 |  | TBD |
| 8 | 6 | 0.75 | 24 | 28 |  | TBD |
| 8 | 6 | 0.4 | 15 | 18 |  | TBD |
| 8 | 4 | 1 | 16 | 16 | TBD | TBD |
| 8 | 4 | 0.8 | 16 | 16 |  | TBD |
| 8 | 4 | 0.75 | 16 | 16 |  | TBD |
| 8 | 4 | 0.4 | 11 | 13 |  | TBD |
| 8 | 2 | 1 | 9 | 9 | TBD | TBD |
| 8 | 2 | 0.8 | 9 | 9 |  | TBD |
| 8 | 2 | 0.75 | 9 | 9 |  | TBD |
| 8 | 2 | 0.4 | 5 | 6 |  | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |  |  |  |

  - Option 2 (Ericsson)

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 27 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Recommended WF

  - This issue can be postponed until Issue 1-3-3 to 1-3-6 are closed

Topic #2: CSI requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300131 | China Telecom | Proposal 5: Support to use the 70% max TP, also fine to additionally consider 30% max TP requirement to also verify the HARQ re-transmission performance. Proposal 6: Align with the existing 2Rx and 4Rx to cover both PMI and RI tests. For PMI requirements, we support to cover 16 Tx which is more typical for high frequencies. |
| R4-2300290 | Apple | Proposal#13: Do not define PMI requirements for 8Rx Proposal#14: Do not define RI requirements for 8Rx |
| R4-2300612 | Nokia | Proposal 15:RAN4 to extend the PMI and Rank reporting requirements for 8 Rx. |
| R4-2300830 | Samsung | Proposal 10: Only introduce CQI requirements for 8Rx CSI requirements, do not introduce PMI and RI requirements for 8Rx CSI requirements. |
| R4-2301050 | ZTE | Proposal 5. Follow LTE conclusions, not define PMI requirements and RI requirements for NR 8 Rx |
| R4-2301893 | Ericsson | Proposal 10: Do not define new PMI reporting requirements for 8Rx UE in FR1. Proposal 11: Do not define RI reporting for 8 Rx UE. |
| R4-2302349 | Qualcomm | Proposal 11: SNR points be (2,3) and (8,9) of Table 6.2.3.1.1.1-1 of [2] for 4L, 8Rx in static channel Proposal 12: Follow LTE approach for Rank Indicator and do not specify new 8Rx requirements. Proposal 13: Follow LTE approach for PMI reporting and do not specify new 8Rx requirements |
| R4-2302170 | Huawei, HiSilicon | Observation 1: Restriction on i2 is only applied to “reportQuantity” setting to “CRI/Ri/i1/CQI”, which mixes the purpose of CQI testing and PMI testing if “reportQuantity” is set to “cri-RI-PMI-CQI” Proposal 1: Consider following simulation assumptions: - Antenna configuration: XP 4T8R - (N1,N2)=(2,1); (O1,O2)=(4,1) - ReportQuantity: CRI/Ri/i1/CQI - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001 Proposal 2: Use simulation assumptions listed in Table 2-1 for CQI testing. Proposal 3: Set lower SNR to [4,5] dB for 16QAM and higher SNR to [10,11] dB for 64QAM for 8RX CQI testing. Proposal 4: Don’t introduce PMI requirements Proposal 5: Don’t define RI requirements |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 CQI requirements

Agreements in RAN4#105 R4-2220613: Whether define CQI requirements for 8RX - Introduce CQI requirements with rank 4 under static channel Test setup for CQI requirements for static conditions - 40 MHz/30 kHz, 106 RBs. - CQI index for up to 64QAM. - Rank 4.

Issue 2-1-1: Simulation assumptions

- Proposals

  - Option 1: (Huawei) Consider the following test parameters

- Antenna configuration: XP 4T8R

- (N1,N2)=(2,1); (O1,O2)=(4,1)

- ReportQuantity: CRI/Ri/i1/CQI

- Two-one-TypeI-SinglePanel-Restriction:  0b00000001

- TypeI-SinglePanel-codebookSubsetRestriction-i2:  0x0000000000000001

  - Other options.

- Recommended WF

  - TBA

Issue 2-1-1a: Summary of simulation assumptions

- Proposals

  - Option 1: (Huawei) Use the following simulation assumptions for 8Rx CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  |

  - Option 2:(Ericsson)

| - Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

- Recommended WF

  - TBA

Issue 2-1-2: SNR

- Proposals

  - Option 1: [2, 3] dB and [8, 9]dB (Qualcomm)

  - Option 2: [4, 5] dB and [10, 11]dB (Huawei)

  - Option 3: [1, 2] dB and [7, 8] dB (Ericsson)

- Recommended WF

  - TBA

Sub-topic 2-2 PMI requirements

Agreements in RAN4#105 R4-2220613: Whether define PMI requirements for 8RX - FFS whether to define PMI requirements Test setup for PMI requirements if agreed - Option 1: - 40 MHz/30 kHz - Propagation environment: TDLA30-5 - Single PMI- with 8 Tx Type I with single panel codebook and antenna configuration: High XP 8 x 8 with (N1, N2) = (4, 1) - Multiple PMI with 16 Tx Type II codebook and antenna configuration: Medium XP 16x8 with (N1, N2) = (4, 2) - Other options are not precluded.

Issue 2-2-1: Whether to define PMI requirements

- Proposals

  - Option 1: Yes (China Telecom, Nokia)

  - Option 2: No (Apple, Samsung, ZTE, Ericsson, Qualcomm, Huawei)

- Recommended WF

  - TBA

Issue 2-2-2: Test configuration if agreed

- Proposals

  - Option 1: 16Tx (China Telecom)

  - Other options

- Recommended WF

  - TBA

Sub-topic 2-3 RI requirements

Agreements in RAN4#105 R4-2220613: • FFS whether to define RI requirements

Issue 2-3-1: Whether to define RI requirements

- Proposals

  - Option 1: Yes (China Telecom, Nokia)

  - Option 2: No (Apple, Samsung, ZTE, Ericsson, Qualcomm, Huawei)

- Recommended WF

  - TBA

Topic #3: Specification structure

Issue 3-1: Spec structure

- Proposals

  - Option 1: (Ericsson)

UE demodulation requirements:

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General | Updates on FR1 8Rx |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.2 | 2RX requirements |  |
| 5.2.3 | 4RX requirements |  |
| 5.2.4 | 8RX requirements | New section |
| 5.2.4.1 | FDD | Void |
| 5.2.4.2 | TDD | New section |
| 5.2.4.2.1 | Minimum requirements for PDSCH Mapping Type A | New section |
| 5.2A | PDSCH demodulation requirements for CA |  |
| 5.2A.4 | 8RX requirements | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.2 | 2RX requirements |  |
| 5.3.3 | 4 RX requirements |  |
| 5.3.4 | 8RX requirements | New section |
| 5.3.4.1 | FDD | Void |
| 5.3.4.2 | TDD | New section |
| 5.5 | Sustained downlink data rate provided by lower layers | Updates on FR1 8 Rx |
| 5.5.1 | FR1 single carrier requirements | Updates on FR1 8 Rx |
| 5.5A | Sustained downlink data rate provided by lower layers |  |
| 5.5A.1 | FR1 CA requirements | Updates on FR1 8 Rx |

CSI requirements:

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General | Updates on FR1 8 Rx |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.4 | 8RX requirements | New section |
| 6.2.4.1 | FDD | Void |
| 6.2.4.2 | TDD | New section |
| 6.2A | Reporting of Channel Quality Indicator (CQI) for CA |  |
| 6.2A.5 | 8RX requirements | New section |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 6.3.4 | 8RX requirements | New section |
| 6.3.4.1 | FDD | Void |
| 6.3.4.2 | TDD | New section |

  - Option 2: (Huawei)

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General |  |
| 5.1.1.12 | Applicability of performance requirements for 8Rx capable UEs |  |
| 5.1.1.12.1 | Applicability rule and antenna connection for single carrier PDSCH tests |  |
| 5.1.1.12.2 | Applicability rule and antenna connection for single carrier PDCCH tests |  |
| 5.1.1.12.3 | Applicability rule and antenna connection for CA/DC tests |  |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.4 | 8RX requirements |  |
| 5.2.4.1 | FDD | (If necessary) |
| 5.2.4.1.1 | Minimum requirements for PDSCH Mapping Type A | (If necessary) |
| 5.2.4.1.2 | Minimum requirements for PDSCH Mapping Type B and UE processing capability 2 | (If necessary) |
| 5.2.4.2 | TDD |  |
| 5.2.4.2.1 | Minimum requirements for PDSCH Mapping Type A |  |
| 5.2.4.2.2 | Minimum requirements for PDSCH Mapping Type B and UE processing capability 2 |  |
| 5.2A | PDSCH demodulation requirements for CA | (If necessary) |
| 5.2A.4 | 8RX requirements | (If necessary) |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.4 | 8RX requirements |  |
| 5.3.4.1 | FDD | (If necessary) |
| 5.3.4.1.1 | 1 Tx Antenna performances | (If necessary) |
| 5.3.4.1.2 | 2 Tx Antenna performances | (If necessary) |
| 5.3.4.2 | TDD |  |
| 5.3.4.2.1 | 1 Tx Antenna performances |  |
| 5.3.4.2.2 | 2 Tx Antenna performances |  |
| 5.5 | Sustained downlink data rate provided by lower layers |  |
| 5.5.1 | FR1 single carrier requirements |  |
| 5.5A | Sustained downlink data rate provided by lower layers |  |
| 5.5A.1 | FR1 CA requirements | (If necessary) |
| 6.1 | General |  |
| 6.1.1.7 | Applicability of performance requirements for 8Rx capable UEs |  |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.4 | 8RX requirements |  |
| 6.2.4.1 | FDD | (If necessary) |
| 6.2.4.1.1 | CQI reporting definition under AWGN conditions | (If necessary) |
| 6.2.4.2 | TDD |  |
| 6.2.4.2.1 | CQI reporting definition under AWGN conditions |  |
| 6.2A | Reporting of Channel Quality Indicator (CQI) for CA | (If necessary) |
| 6.2A.5 | 8RX requirements | (If necessary) |
| 6.2A.5.1 | CQI reporting definition under AWGN conditions | (If necessary) |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) | (If necessary) |
| 6.3.4 | 8RX requirements | (If necessary) |
| 6.3.4.1 | FDD | (If necessary) |
| 6.3.4.2 | TDD | (If necessary) |
| 6.4 | Reporting of Precoding Matrix Indicator (RI) | (If necessary) |
| 6.3.4 | 8RX requirements | (If necessary) |
| 6.3.4.1 | FDD | (If necessary) |
| 6.3.4.2 | TDD | (If necessary) |
| A.3 | DL reference measurement channels |  |
| A.3.2 | Reference measurement channels for PDSCH performance requirements |  |
| A.3.2.1 | FDD | (If necessary) |
| A.3.2.2.1 | Reference measurement channels for SCS 15 kHz FR1 | (If necessary) |
| A.3.2.2 | TDD |  |
| A.3.2.2.2 | Reference measurement channels for SCS 30 kHz FR1 | Add FRC for Rank8 cases |
| A.4 | CSI reference measurement channels | Add FRC for Rank4 cases |
| B.1 | Static propagation condition |  |
| B.1.3 | UE Receiver with 8Rx |  |
| B.2 | Multi-path fading propagation conditions |  |
| B.2.3 | MIMO channel correlation matrices |  |
| B.2.3.1 | MIMO Correlation Matrices using Uniform Linear Array (ULA) |  |
| B.2.3.2 | MIMO Correlation Matrices using Cross Polarized Antennas (X-pol) |  |

- Recommended WF

  - This can be discussed after the test scenarios are finalized, i.e. low priority for this meeting.