3GPP TSG- RAN WG4 Meeting #106	R4-2302963

Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | 0001 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.115-1: Correction of some errors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Perf |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The clause structures for 6.7 and 6.8 are not aligned with other clauses. Some editorial errors exist in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The followings are corrected, - Modify the clause structures for 6.7 and 6.8. - Correct some editorial errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec includes the errors which may bring misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | It’s revised from R4-2300564 |  |  |  |  |  |  |  |  |

<Start of the changes>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWNominal	Nominal channel bandwidth

BWPassband	Passband bandwidth

f	Separation between the passband edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper passband edge

Foffset,low	Frequency offset from FC,low to the lower passband edge

f_offset	Separation between the passband edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the operating band

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

Pmax,p,AC	Maximum passband output power measured per antenna connector

Prated,in	Rated pass band input power to the repeater for the test per antenna connector

Prated,p,AC	Rated passband output power per antenna connector

Prated,t,AC	Rated total output power declared per antenna connector

Wgap	Inter passband Bandwidth gap size

<Next change>

4.1.2.2	Conducted characteristics measurements

Table 4.1.2.2-1: Maximum Test System uncertainty for conducted characteristics tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Repeater output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |  |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 4.2 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |  |
| 6.5.2 ACLR/ CACLR | BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.5.3 Operating band unwanted emission | 1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ 4.2 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.5.4 Spurious emissions | In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB 4 GHz < f  19 GHz ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.6 EVM | 1% signal analyser 2% stimulus signal |  |
| 6.7 Input intermodulation | ±1,2 dB | Formula: RSS CW1 level error, 2 x CW2 level error, and measurement error (using all errors = ±0,5 dB) |
| 6.8 Output intermodulation | For operating band unwanted emission: 2,1 dB The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. For spurious emission: In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB; 4GHz < f 19 GHz ±4,0 dB. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required in 6.5.4 and 6.5.5. | Formula: RSS 2x Interference signal level error and operating band unwanted emission measurement level error. (1 dB interference signal level error is assumed). |
| 6.9 ACRR | 0,7 dB |  |
| 6.10.1 Transmit OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.10.2 Transmitter transient period | N/A |  |

<End of the changes>