3GPP TSG-RAN WG4 Meeting # 106                                                             R4-2303177

# Athens, GR, February 27th – March 03rd, 2023

Title:	Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility

Response to:	R1-2210727

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN WG 4

To:	RAN WG 1

Cc:	RAN WG 2

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 thanks RAN1 for the LS sent in R1-2210727. In the last meeting, RAN4 has sent the Reply LS to RAN1, some issues related to intra-frequency L1 non-serving measurement were still FFS.

In this meeting, RAN4 further discuss the remaining issues. RAN4 would like to further provide the following feedback for RAN1’s Question 1:

Question 1 (to RAN4): RAN1 respectfully asks RAN4 if the restriction on e.g., SFN offset alignment, BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP, and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

RAN4 feedback:

Whether to consider RTD of serving cell and neighbour cell larger than one CP for intra-frequency L1-RSRP measurement?

- For SSB based intra-frequency L1 measurement, RAN4 agreed to support the scenario that RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than CP length of the corresponding SCS with additional UE capability.

- Note: the need for UE capability can be further discussed subject to the outcome of the discussion on measurement framework.

Whether SFN offset alignment can be relaxed?

- The SFN offset (sfn-SSB-Offset) alignment can be relaxed if UE performs L3 measurement before L1 measurement.

Whether SSB for intra-frequency L1 measurement should be covered by serving cell active BWP?

- At least in some scenarios, the SSB for intra-frequency L1 measurement may not be covered by serving cell active BWP. The scenarios and whether to define the L1-RSRP measurement requirements for those scenarios are under discussion in RAN4.

2 Actions

To RAN WG1 group

ACTION: 	RAN4 sincerely asks RAN1 to take into account the above feedback into consideration.

3 Dates of Next TSG RAN WG4 Meetings

TSG-RAN WG4 Meeting #106bis-e	17 April – 26 April 2023	      Online

TSG-RAN WG4 Meeting #107	22 May – 26 May 2023		 Incheon , KR