3GPP TSG-RAN WG4 Meeting # 106											          R4-2303206

Athens, Greece, February 27 – March 3, 2022

Agenda item:			5.4

Source:	Huawei, HiSilicon, ZTE

Title:	WF on R17 NR MG enhancements RRM maintenance

Document for:	Approval

- Topic #1: NCSG

Issue 1-1: Overlap between SMTC and NCSG for measurement without MG

- Agreement

  - keep the current assumption of no simultaneous measurement within NCSG (i.e. measurement without MG performed in NCSG simultaneously with other measurements to be performed with NCSG) for Rel-17

  - FFS whether simultaneous measurement within NCSG (i.e. measurement without MG performed in NCSG simultaneously with other measurements to be performed with NCSG) is supported for Rel-18.

Issue 1-2: Requirements for inter-frequency measurement without MG

- Agreement

  - For inter-frequency without MG (cl. 9.3.9), define separate NW sync assumption and number of samples for

    - Scenario 1, where inter-frequency SSB is within UE active BWP, and

    - Scenario 2, where UE indicates ‘nogap-noncsg’ for the inter-frequency measurement.

# Topic #2: Con-MG

Issue 2-1: MO associated with a MG but measured without MG

- Wayforward

  - For an MO that can be measured outside MG and associated with an MG, FFS

    - Option 1: the MO is always measured within the associated MG

    - Option 2: the MO is measured within the associated MG only when it needs to be measured with MG

Issue 2-2: MO fully overlapping with the union of the MG1 and MG2

- Wayforward

  - the MO should be measured within the associated MG in the following cases,

    - When a MO is fully overlapping with the associated MG, or

    - FFS: When a MO is partially overlapping with the associated MG and fully overlapping with the union of the ConMGs.

  - Whether to clarify this in spec is FFS

Issue 2-3: E-UTRAN measurement with con-MGs

- Wayforward

  - FFS whether all E-UTRAN measurement objects are expected to be associated with a single concurrent gap pattern or not when both E-UTRAN measurement objects and other type of measurements are configured

    - Option 1: Yes.

    - Option 2: No.

    - Option 3: Up to UE capability (e.g. concurrentMeasGapEUTRA-r17)

      - For UE supporting the capability, different LTE MOs can be associated with multiple MGs.

      - For UE not supporting the capability, all LTE MOs can be associated with only a single MG.